// =============================================================

// dsu_stall_tb.sv

// Tapeout-grade SV verification environment for GARUDA dsu_stall.v

// (2-cycle read-after-read structural hazard detector for the DSU)

//

// UNLIKE the other GARUDA unit TBs in this set (barrel_shifter,

// dsu_decoder, csa_3to2), dsu_stall is SEQUENTIAL: it has real

// registered state (prev_was_2cycle, prev_acc_sel). Verifying it

// correctly requires cycle-accurate stimulus SEQUENCES and a

// STATEFUL reference model that mirrors the register-update rules

// independently -- not independent single-cycle transactions.

//

// Two architectural findings from tracing dsu_stall.v against

// mac_unit.v's actual pipeline timing are called out explicitly in

// directed test cases 6 and 7 below (see comments there) rather than

// silently assumed either way -- they may be real hazard-detection

// gaps, or there may be upstream context this unit-level TB can't see.

//

// Requires a simulator with full IEEE1800 support (covergroup,

// assert property). NOT Icarus Verilog.

// =============================================================

`timescale 1ns / 1ps

// -------------------------------------------------------------

// 0. DEFINES (inlined from dsu_defs.vh)

// -------------------------------------------------------------

`ifndef DSU_DEFS_VH

`define DSU_DEFS_VH

`define ACC_FX  2'b00

`define ACC_FY  2'b01

`define ACC_MAG 2'b10

`define MAC_CTRL_W      7

`define MAC_CTRL_EN     6

`define MAC_CTRL_ADDSUB 5

`define MAC_CTRL_CLEAR  4

`define MAC_CTRL_LOAD   3

`define MAC_CTRL_ABS    2

`define MAC_CTRL_DOT    1

`define MAC_CTRL_FLUSH  0

`define FUNCT5_MAC_SEL  5'd0

`define FUNCT5_MACSUB   5'd1

`define FUNCT5_MACABS   5'd2

`define FUNCT5_MACDOT   5'd3

`define FUNCT5_MACLOAD  5'd4

`define FUNCT5_MACCLEAR 5'd5

`define FUNCT5_MACSAT   5'd6

`define FUNCT5_MACRD_LO 5'd7

`define FUNCT5_MACRD_HI 5'd8

`define FUNCT5_MACSHIFT 5'd9

`endif

// -------------------------------------------------------------

// 1. DUT: dsu_stall.v (pasted verbatim, self-contained file)

// -------------------------------------------------------------

module dsu_stall (

input wire clk,

input wire rst_n,

input wire flush,

input wire [4:0] funct5,

input wire [2:0] funct3,

input wire       is_custom0,

input wire [1:0] acc_sel,

output wire dsu_busy

);

wire cur_is_rtype = is_custom0 & (funct3 == 3'b000);

wire cur_is_itype = is_custom0 & (funct3 == 3'b001);

wire cur_is_2cycle = cur_is_rtype & ((funct5 == `FUNCT5_MACRD_LO) | (funct5 == `FUNCT5_MACRD_HI) | (funct5 == `FUNCT5_MACSAT));

wire cur_reads_acc = (cur_is_rtype & ((funct5 == `FUNCT5_MACRD_LO) | (funct5 == `FUNCT5_MACRD_HI) | (funct5 == `FUNCT5_MACSAT))) | cur_is_itype;

reg       prev_was_2cycle;

reg [1:0] prev_acc_sel;

wire stall_now;

always @(posedge clk or negedge rst_n) begin

if(!rst_n) begin

prev_was_2cycle <= 1'b0;

prev_acc_sel <= 2'b00;

end

else if (flush) begin

prev_was_2cycle <= 1'b0;

prev_acc_sel <= 2'b00;

end

else if (~stall_now) begin

prev_was_2cycle <= cur_is_2cycle;

prev_acc_sel <= acc_sel;

end

end

assign stall_now = prev_was_2cycle & cur_reads_acc & (acc_sel == prev_acc_sel);

assign dsu_busy = stall_now;

endmodule

// -------------------------------------------------------------

// 2. INTERFACE (+ bound SVA assertions)

// -------------------------------------------------------------

interface dsu_stall_if (input bit clk);

logic       rst_n;

logic       flush;

logic [4:0] funct5;

logic [2:0] funct3;

logic       is_custom0;

logic [1:0] acc_sel;

logic       dsu_busy;

// A1: X-propagation -- known inputs must never produce an unknown

//     dsu_busy (also implicitly checks prev_was_2cycle/prev_acc_sel

//     never go unknown after a real reset has occurred).

property p_no_x_propagation;

@(posedge clk) disable iff (!rst_n)

(!$isunknown({funct5, funct3, is_custom0, acc_sel, flush})) |-> (!$isunknown(dsu_busy));

endproperty

a_no_x_propagation: assert property (p_no_x_propagation)

else $error("[SVA-FAIL] X detected on dsu_busy with fully-known inputs");

// A2: reset must clear dsu_busy -- since dsu_busy = stall_now and

//     stall_now requires prev_was_2cycle=1, and reset clears that,

//     dsu_busy must read 0 on the cycle immediately following reset

//     release (this cycle's prev_was_2cycle is guaranteed 0).

property p_reset_clears_busy;

@(posedge clk) $rose(rst_n) |-> !dsu_busy;

endproperty

a_reset_clears_busy: assert property (p_reset_clears_busy)

else $error("[SVA-FAIL] dsu_busy asserted on the cycle immediately following reset release");

// A3: flush must clear dsu_busy on the cycle immediately after it

//     was asserted (mirrors A2's reasoning: flush forces

//     prev_was_2cycle<=0 for the next cycle).

property p_flush_clears_busy_next_cycle;

@(posedge clk) disable iff (!rst_n)

$past(flush) |-> !dsu_busy;

endproperty

a_flush_clears_busy_next_cycle: assert property (p_flush_clears_busy_next_cycle)

else $error("[SVA-FAIL] dsu_busy asserted the cycle after a flush");

// A4: dsu_busy can only ever be asserted when the current

//     instruction is one that reads the accumulator. If cur_reads_acc

//     would be false, dsu_busy must be false regardless of history.

property p_busy_implies_reads_acc;

@(posedge clk) disable iff (!rst_n)

dsu_busy |-> (is_custom0 & ((funct3 == 3'b000 & (funct5 == `FUNCT5_MACRD_LO | funct5 == `FUNCT5_MACRD_HI | funct5 == `FUNCT5_MACSAT)) | funct3 == 3'b001));

endproperty

a_busy_implies_reads_acc: assert property (p_busy_implies_reads_acc)

else $error("[SVA-FAIL] dsu_busy asserted for an instruction that does not read the accumulator");

endinterface

// -------------------------------------------------------------

// 3. PER-CYCLE STIMULUS ITEM -- rand + constraint => real CRV

// -------------------------------------------------------------

typedef enum {

OP_MAC_SEL, OP_MACSUB, OP_MACABS, OP_MACDOT, OP_MACLOAD, OP_MACCLEAR,

OP_MACSAT, OP_RD_LO, OP_RD_HI, OP_MACSHIFT, OP_NON_CUSTOM0, OP_BAD_FUNCT3

} stall_op_e;

class stall_cycle;

rand stall_op_e op;

rand bit [1:0]  acc_sel;

rand bit        flush;

rand bit        rst_n;

// Raw-field override, used only by the exhaustive sweep (section 10

// of the generator) to reach (funct5,funct3,is_custom0) combinations

// that don't correspond to any single named op in stall_op_e -- the

// op enum is deliberately a curated, meaningful instruction set for

// directed/CRV use; the exhaustive sweep needs the full raw space.

bit       use_raw;

bit [4:0] raw_funct5;

bit [2:0] raw_funct3;

bit       raw_is_custom0;

constraint c_op_dist {

op dist {

OP_MAC_SEL :/ 8, OP_MACSUB :/ 8, OP_MACABS :/ 8, OP_MACDOT :/ 8,

OP_MACLOAD :/ 8, OP_MACCLEAR :/ 8, OP_MACSAT :/ 10, OP_RD_LO :/ 12,

OP_RD_HI :/ 12, OP_MACSHIFT :/ 10, OP_NON_CUSTOM0 :/ 4, OP_BAD_FUNCT3 :/ 4

};

}

constraint c_rst_n_dist  { rst_n  dist { 1'b1 :/ 97, 1'b0 :/ 3  }; }

constraint c_flush_dist  { flush  dist { 1'b0 :/ 85, 1'b1 :/ 15 }; }

function void fields(output bit [4:0] funct5, output bit [2:0] funct3, output bit is_custom0);

if (use_raw) begin

funct5 = raw_funct5; funct3 = raw_funct3; is_custom0 = raw_is_custom0;

return;

end

is_custom0 = 1'b1;

case (op)

OP_MAC_SEL:   begin funct3 = 3'b000; funct5 = `FUNCT5_MAC_SEL;  end

OP_MACSUB:    begin funct3 = 3'b000; funct5 = `FUNCT5_MACSUB;   end

OP_MACABS:    begin funct3 = 3'b000; funct5 = `FUNCT5_MACABS;   end

OP_MACDOT:    begin funct3 = 3'b000; funct5 = `FUNCT5_MACDOT;   end

OP_MACLOAD:   begin funct3 = 3'b000; funct5 = `FUNCT5_MACLOAD;  end

OP_MACCLEAR:  begin funct3 = 3'b000; funct5 = `FUNCT5_MACCLEAR; end

OP_MACSAT:    begin funct3 = 3'b000; funct5 = `FUNCT5_MACSAT;   end

OP_RD_LO:     begin funct3 = 3'b000; funct5 = `FUNCT5_MACRD_LO; end

OP_RD_HI:     begin funct3 = 3'b000; funct5 = `FUNCT5_MACRD_HI; end

OP_MACSHIFT:  begin funct3 = 3'b001; funct5 = 5'd0;             end

OP_BAD_FUNCT3:begin funct3 = 3'b101; funct5 = 5'd0;             end

OP_NON_CUSTOM0: begin is_custom0 = 1'b0; funct3 = 3'b000; funct5 = 5'd0; end

default: begin funct3 = 3'b000; funct5 = 5'd0; end

endcase

endfunction

endclass

// -------------------------------------------------------------

// 4. GENERATOR -- directed sequences (each entry is a full cycle-by-

//    cycle sequence, since this DUT's correctness is defined by

//    sequences, not single cycles) + CRV sequences.

// -------------------------------------------------------------

class stall_generator;

stall_cycle seq_q[$][$];   // queue of sequences, each a queue of cycles

string      seq_name[$];

int num_random_seqs;

int random_seq_len;

function new(int num_random_seqs = 15, int random_seq_len = 30);

this.num_random_seqs = num_random_seqs;

this.random_seq_len  = random_seq_len;

endfunction

function stall_cycle mk(stall_op_e op, bit [1:0] acc_sel = 2'b00, bit flush = 0, bit rst_n = 1);

stall_cycle c = new();

c.op = op; c.acc_sel = acc_sel; c.flush = flush; c.rst_n = rst_n;

return c;

endfunction

// Raw-field variant for the exhaustive sweep -- see stall_cycle's

// use_raw comment.

function stall_cycle mk_raw(bit [4:0] funct5, bit [2:0] funct3, bit is_custom0, bit [1:0] acc_sel);

stall_cycle c = new();

c.use_raw = 1'b1;

c.raw_funct5 = funct5; c.raw_funct3 = funct3; c.raw_is_custom0 = is_custom0;

c.acc_sel = acc_sel; c.flush = 1'b0; c.rst_n = 1'b1;

return c;

endfunction

function void add_seq(string name, stall_cycle q[$]);

seq_name.push_back(name);

seq_q.push_back(q);

endfunction

function void build();

// 1) Basic hazard trigger: MACRD_LO(FX) -> MACRD_LO(FX) => stall on cycle 2

begin

stall_cycle q[$];

q.push_back(mk(OP_RD_LO, `ACC_FX));

q.push_back(mk(OP_RD_LO, `ACC_FX));

q.push_back(mk(OP_MAC_SEL, `ACC_FX));  // bubble-ish next instr, hazard should have cleared

add_seq("basic_hazard_same_acc", q);

end

// 2) No hazard: MACRD_LO(FX) -> MACRD_LO(FY) (different accumulator)

begin

stall_cycle q[$];

q.push_back(mk(OP_RD_LO, `ACC_FX));

q.push_back(mk(OP_RD_LO, `ACC_FY));

add_seq("no_hazard_diff_acc", q);

end

// 3) All three read-family producers x all three consumer types, same acc

begin

stall_op_e producers[3] = '{OP_RD_LO, OP_RD_HI, OP_MACSAT};

stall_op_e consumers[4] = '{OP_RD_LO, OP_RD_HI, OP_MACSAT, OP_MACSHIFT};

foreach (producers[i]) begin

foreach (consumers[j]) begin

stall_cycle q[$];

q.push_back(mk(producers[i], `ACC_MAG));

q.push_back(mk(consumers[j], `ACC_MAG));

add_seq($sformatf("producer_%0d_consumer_%0d_same_acc", i, j), q);

end

end

end

// 4) Reset mid-sequence: establish a hazard-pending state, then

//    reset, then confirm the pending hazard is gone

begin

stall_cycle q[$];

q.push_back(mk(OP_RD_LO, `ACC_FX));

q.push_back(mk(OP_RD_LO, `ACC_FX, 1'b0, 1'b0));  // rst_n=0 this cycle

q.push_back(mk(OP_RD_LO, `ACC_FX));              // would have hazarded pre-reset

add_seq("reset_mid_hazard", q);

end

// 5) Flush mid-hazard: establish hazard-pending state, flush,

//    then confirm the pending hazard is gone the cycle after

begin

stall_cycle q[$];

q.push_back(mk(OP_RD_LO, `ACC_FX));

q.push_back(mk(OP_RD_LO, `ACC_FX, 1'b1, 1'b1));  // flush=1 this cycle

q.push_back(mk(OP_RD_LO, `ACC_FX));              // should NOT stall (state was cleared)

add_seq("flush_mid_hazard", q);

end

// 5b) Flush asserted on the SAME cycle as the hazard condition --

//     per the RTL, stall_now is purely combinational from

//     PRE-edge state, so it still fires THIS cycle; flush only

//     prevents the state from propagating to the NEXT cycle.

begin

stall_cycle q[$];

q.push_back(mk(OP_RD_LO, `ACC_FX));

q.push_back(mk(OP_RD_LO, `ACC_FX, 1'b1, 1'b1));  // hazard AND flush together

add_seq("flush_same_cycle_as_hazard", q);

end

// 6) *** FINDING 1 *** -- compute op immediately followed by a

//     read op on the SAME accumulator. Per dsu_stall.v's literal

//     spec, cur_is_2cycle never fires for compute ops, so NO

//     stall is inserted here -- yet mac_unit.v's actual 2-stage

//     carry-save pipeline means the compute op's effect on `acc`

//     is not yet visible. This sequence exists to make that gap

//     directly observable in the log/coverage, not to assert a

//     verdict either way -- flag for architecture review.

begin

stall_op_e computes[6] = '{OP_MAC_SEL, OP_MACSUB, OP_MACABS, OP_MACDOT, OP_MACLOAD, OP_MACCLEAR};

foreach (computes[i]) begin

stall_cycle q[$];

q.push_back(mk(computes[i], `ACC_FY));

q.push_back(mk(OP_RD_LO, `ACC_FY));

add_seq($sformatf("FINDING1_compute_%0d_then_read_same_acc", i), q);

end

end

// 7) *** FINDING 2 *** -- held-stall livelock demonstration.

//     Trigger a hazard, then hold the SAME triggering instruction

//     constant for several more cycles (as a simple "freeze and

//     retry" pipeline would). Per the RTL, prev_was_2cycle/

//     prev_acc_sel freeze while stalled, so if the held inputs

//     never change, dsu_busy is expected to remain asserted for

//     the ENTIRE held duration -- this sequence measures exactly

//     that, for the log/report, rather than asserting a verdict.

begin

stall_cycle q[$];

q.push_back(mk(OP_RD_LO, `ACC_MAG));

repeat (6) q.push_back(mk(OP_RD_LO, `ACC_MAG));  // hold identical instruction

q.push_back(mk(OP_MAC_SEL, `ACC_MAG));            // finally changes

add_seq("FINDING2_held_stall_livelock_probe", q);

end

// 8) acc_sel boundary sweep (00/01/10/11) as both producer and

//    consumer accumulator selector, matched and mismatched

begin

bit [1:0] sels[4] = '{2'b00, 2'b01, 2'b10, 2'b11};

foreach (sels[i]) foreach (sels[j]) begin

stall_cycle q[$];

q.push_back(mk(OP_RD_HI, sels[i]));

q.push_back(mk(OP_RD_HI, sels[j]));

add_seq($sformatf("accsel_prod%0d_cons%0d", i, j), q);

end

end

// 9) Non-Custom-0 and bad-funct3 instructions never read the

//    accumulator and never set prev_was_2cycle

begin

stall_cycle q[$];

q.push_back(mk(OP_RD_LO, `ACC_FX));

q.push_back(mk(OP_NON_CUSTOM0, `ACC_FX));

q.push_back(mk(OP_RD_LO, `ACC_FX));  // hazard should be gone (non-custom0 cleared prev state)

add_seq("non_custom0_breaks_chain", q);

end

begin

stall_cycle q[$];

q.push_back(mk(OP_RD_LO, `ACC_FX));

q.push_back(mk(OP_BAD_FUNCT3, `ACC_FX));

q.push_back(mk(OP_RD_LO, `ACC_FX));

add_seq("bad_funct3_breaks_chain", q);

end

// 10) EXHAUSTIVE: sweep every (funct5, funct3, is_custom0) combination

//     -- 32*8*2 = 512 combos -- against two base states:

//       (a) no hazard pending -> busy must be 0 for ALL 512 combos

//           regardless of what reads_acc evaluates to (proves the

//           prev_was_2cycle gate genuinely dominates)

//       (b) hazard pending with a MATCHING acc_sel -> busy must

//           exactly equal reads_acc(funct5,funct3,is_custom0) for

//           each combo, which exhaustively validates the read-family

//           decode logic (the equality/OR chain most likely to hide

//           an off-by-one or a missed encoding) against every

//           possible input, not a sample of it.

begin

for (int f5i = 0; f5i < 32; f5i++) begin

for (int f3i = 0; f3i < 8; f3i++) begin

for (int ic0i = 0; ic0i < 2; ic0i++) begin

stall_cycle setup_a[$], setup_b[$];

// (a) no-hazard-pending base state, then probe

setup_a.push_back(mk(OP_MAC_SEL, `ACC_FX));  // establishes prev_was_2cycle=0

setup_a.push_back(mk_raw(f5i[4:0], f3i[2:0], ic0i[0], `ACC_FX));

add_seq($sformatf("EXH_nohazard_f5%0d_f3%0d_ic0%0d", f5i, f3i, ic0i), setup_a);

// (b) hazard-pending (matching acc_sel), then probe

setup_b.push_back(mk(OP_RD_LO, `ACC_FY));    // establishes prev_was_2cycle=1, prev_acc_sel=FY

setup_b.push_back(mk_raw(f5i[4:0], f3i[2:0], ic0i[0], `ACC_FY));

add_seq($sformatf("EXH_hazardpend_f5%0d_f3%0d_ic0%0d", f5i, f3i, ic0i), setup_b);

end

end

end

end

// 11) CONSTRAINED-RANDOM multi-cycle sequences

for (int s = 0; s < num_random_seqs; s++) begin

stall_cycle q[$];

for (int c = 0; c < random_seq_len; c++) begin

stall_cycle t = new();

assert (t.randomize()) else $error("[GEN] randomize() failed");

q.push_back(t);

end

add_seq($sformatf("crv_seq_%0d", s), q);

end

endfunction

endclass

// -------------------------------------------------------------

// 5. REFERENCE MODEL -- stateful, cycle-accurate mirror of the

//    RTL's register-update rules (independently structured: uses a

//    case statement on the op's funct5 rather than the RTL's chained

//    equality/OR expression, so it cannot silently repeat the same

//    conceptual bug).

// -------------------------------------------------------------

class stall_ref_model;

bit       prev_was_2cycle;

bit [1:0] prev_acc_sel;

function new();

prev_was_2cycle = 1'b0;

prev_acc_sel    = 2'b00;

endfunction

// Returns predicted dsu_busy for this cycle's inputs (combinational,

// using state as of BEFORE this cycle's edge), then advances the

// model's state per the RTL's exact register-update priority

// (reset > flush > hold-while-stalled > normal update).

function bit step(bit [4:0] funct5, bit [2:0] funct3, bit is_custom0,

bit [1:0] acc_sel, bit flush, bit rst_n);

bit is_rtype, is_itype, is_2cycle, reads_acc, busy;

is_rtype = is_custom0 && (funct3 == 3'b000);

is_itype = is_custom0 && (funct3 == 3'b001);

case (1'b1)

(is_rtype && (funct5 == `FUNCT5_MACRD_LO || funct5 == `FUNCT5_MACRD_HI || funct5 == `FUNCT5_MACSAT)):

is_2cycle = 1'b1;

default: is_2cycle = 1'b0;

endcase

reads_acc = is_2cycle || is_itype;

busy = prev_was_2cycle && reads_acc && (acc_sel == prev_acc_sel);

if (!rst_n) begin

prev_was_2cycle = 1'b0;

prev_acc_sel    = 2'b00;

end else if (flush) begin

prev_was_2cycle = 1'b0;

prev_acc_sel    = 2'b00;

end else if (!busy) begin

prev_was_2cycle = is_2cycle;

prev_acc_sel    = acc_sel;

end

// else: stalled -- state holds unchanged

return busy;

endfunction

endclass

// -------------------------------------------------------------

// 6. DRIVER

// -------------------------------------------------------------

class stall_driver;

virtual dsu_stall_if vif;

function new(virtual dsu_stall_if vif); this.vif = vif; endfunction

task apply_cycle(stall_cycle c);

bit [4:0] f5; bit [2:0] f3; bit ic0;

c.fields(f5, f3, ic0);

@(negedge vif.clk);

vif.funct5     <= f5;

vif.funct3     <= f3;

vif.is_custom0 <= ic0;

vif.acc_sel    <= c.acc_sel;

vif.flush      <= c.flush;

vif.rst_n      <= c.rst_n;

endtask

endclass

// -------------------------------------------------------------

// 7. MONITOR

// -------------------------------------------------------------

class stall_result;

string seq_name; int cycle_idx;

stall_cycle c;

bit dsu_busy;

endclass

class stall_monitor;

virtual dsu_stall_if vif;

covergroup cg_stall;

cp_busy: coverpoint vif.dsu_busy;

cp_flush: coverpoint vif.flush;

cp_rst_n: coverpoint vif.rst_n;

cp_op_reads_acc: coverpoint (vif.is_custom0 && (vif.funct3 == 3'b001 ||

(vif.funct3 == 3'b000 && (vif.funct5 == `FUNCT5_MACRD_LO ||

vif.funct5 == `FUNCT5_MACRD_HI || vif.funct5 == `FUNCT5_MACSAT))));

cross cp_busy, cp_flush;

cross cp_busy, cp_rst_n;

endgroup

function new(virtual dsu_stall_if vif);

this.vif = vif;

cg_stall = new();

endfunction

task sample_one(output bit busy);

#1; // allow combinational logic to settle after apply_cycle()'s NBA update

busy = vif.dsu_busy;

cg_stall.sample();

endtask

endclass

// -------------------------------------------------------------

// 8. SCOREBOARD

// -------------------------------------------------------------

class stall_scoreboard;

int pass_cnt = 0;

int fail_cnt = 0;

int max_consecutive_busy = 0;

task check(string seq_name, int cycle_idx, stall_cycle c, bit [4:0] f5, bit [2:0] f3, bit ic0,

bit actual_busy, ref stall_ref_model model, ref int consec_busy);

bit expected_busy;

expected_busy = model.step(f5, f3, ic0, c.acc_sel, c.flush, c.rst_n);

if (actual_busy) begin

consec_busy++;

if (consec_busy > max_consecutive_busy) max_consecutive_busy = consec_busy;

end else begin

consec_busy = 0;

end

if (actual_busy === expected_busy) begin

pass_cnt++;

$display("[PASS] seq=%-38s cyc=%0d op=%-14s acc_sel=%0d flush=%0b rst_n=%0b -> busy=%0b",

seq_name, cycle_idx, c.op.name(), c.acc_sel, c.flush, c.rst_n, actual_busy);

end else begin

fail_cnt++;

$display("[FAIL] seq=%-38s cyc=%0d op=%-14s acc_sel=%0d flush=%0b rst_n=%0b -> got=%0b exp=%0b",

seq_name, cycle_idx, c.op.name(), c.acc_sel, c.flush, c.rst_n, actual_busy, expected_busy);

end

endtask

endclass

// -------------------------------------------------------------

// 9. ENVIRONMENT

// -------------------------------------------------------------

class stall_env;

virtual dsu_stall_if vif;

stall_generator  gen;

stall_driver     drv;

stall_monitor    mon;

stall_scoreboard sb;

function new(virtual dsu_stall_if vif, int num_random_seqs = 15, int random_seq_len = 30);

this.vif = vif;

gen = new(num_random_seqs, random_seq_len);

drv = new(vif);

mon = new(vif);

sb  = new();

endfunction

task run();

stall_ref_model model;

int consec_busy;

int total_cycles = 0;

gen.build();

foreach (gen.seq_q[s]) begin

model = new();  // fresh reference-model state per sequence

// (each named sequence starts from an

// implicit "unknown history" -- directed

// sequences that need a specific starting

// state build it explicitly as their own

// first cycle(s), same as real usage after

// a cold reset)

consec_busy = 0;

foreach (gen.seq_q[s][i]) begin

stall_cycle c = gen.seq_q[s][i];

bit [4:0] f5; bit [2:0] f3; bit ic0;

bit actual_busy;

c.fields(f5, f3, ic0);

drv.apply_cycle(c);

mon.sample_one(actual_busy);

sb.check(gen.seq_name[s], i, c, f5, f3, ic0, actual_busy, model, consec_busy);

total_cycles++;

end

end

$display("\n================ DSU_STALL UNIT TB SUMMARY ================");

$display(" SEQUENCES=%0d  TOTAL_CYCLES=%0d  PASS=%0d  FAIL=%0d", gen.seq_q.size(), total_cycles, sb.pass_cnt, sb.fail_cnt);

$display(" MAX OBSERVED CONSECUTIVE dsu_busy CYCLES (any single sequence) = %0d", sb.max_consecutive_busy);

$display(" FUNCTIONAL COVERAGE = %0.2f %%", mon.cg_stall.get_coverage());

if (sb.fail_cnt == 0)

$display(" RESULT: ALL CHECKS PASSED (dsu_stall matches its own literal specification)");

else

$display(" RESULT: %0d CHECK(S) FAILED", sb.fail_cnt);

$display(" NOTE: see FINDING1_* and FINDING2_* sequences above for the two architectural");

$display("       questions raised during this exercise (missing compute->read hazard,");

$display("       and held-stall livelock) -- both PASS against dsu_stall's own spec,");

$display("       which is exactly what makes them worth a human architecture review.");

$display("=============================================================\n");

endtask

endclass

// -------------------------------------------------------------

// 10. TEST

// -------------------------------------------------------------

class stall_test;

stall_env env;

function new(virtual dsu_stall_if vif, int num_random_seqs = 15, int random_seq_len = 30);

env = new(vif, num_random_seqs, random_seq_len);

endfunction

task run(); env.run(); endtask

endclass

// -------------------------------------------------------------

// 11. TB TOP

// -------------------------------------------------------------

module tb_top;

bit clk = 0;

always #5 clk = ~clk;

dsu_stall_if vif(clk);

dsu_stall dut (

.clk        (clk),

.rst_n      (vif.rst_n),

.flush      (vif.flush),

.funct5     (vif.funct5),

.funct3     (vif.funct3),

.is_custom0 (vif.is_custom0),

.acc_sel    (vif.acc_sel),

.dsu_busy   (vif.dsu_busy)

);

stall_test test;

initial begin

vif.rst_n = 1'b0;

vif.flush = 1'b0;

vif.funct5 = 5'd0; vif.funct3 = 3'd0; vif.is_custom0 = 1'b0; vif.acc_sel = 2'b00;

repeat (3) @(posedge clk);

test = new(vif, 15, 30);

test.run();

$finish;

end

endmodule