GARUDA SoC — DSU (Custom-0 MAC cluster)

# Unit-Level Verification Report

Module Under Test: dsu_decoder.v

| Document | Unit Verification Report — dsu_decoder |
| --- | --- |
| Project | GARUDA RV32IM SoC — Team AeroSoC |
| Subsystem | DSU (Custom-0 MAC cluster) — instruction decode |
| Verification Phase | Phase 1 — Unit-Level Verification (per 1Tops RISC-V SoC Verification Strategy) |
| Simulator | Cadence Xcelium (xrun / xmsim) |
| Methodology | Directed tests + Constrained-Random Verification (CRV) + SVA + Functional Coverage |
| Overall Status | PASS — all functional checks pass; coverage 98.78%, near-closed |

# 1. Module Overview

dsu_decoder is a purely combinational instruction decoder for the GARUDA DSU (Custom-0 MAC cluster). It recognizes Custom-0 (opcode 0001011) instructions in two formats: R-type (funct3=000) for compute/load/clear/sat/readback operations, and I-type (funct3=001) for the MACSHIFT operation. It produces a packed control bus for the MAC cluster plus auxiliary enables for the Saturation unit, Barrel Shifter, and Read-back Mux.

- 9 R-type operations, selected by funct5: MAC_SEL, MACSUB, MACABS, MACDOT, MACLOAD, MACCLEAR, MACSAT, MACRD_LO, MACRD_HI (funct5 = 0-8).

- 1 I-type operation: MACSHIFT, reached only via funct3=001 (not via any funct5 comparison).

- illegal_instr flags a recognized-but-malformed Custom-0 instruction (bad funct3, bad funct5 in R-type, or acc_sel=11) while dsu_en=1.

- Several outputs (funct5, funct3, is_custom0, rd_addr, shift_amt, shift_dir, shift_acc_sel) are unconditional raw decodes of instr -- always live regardless of instruction legality or dsu_en, a deliberate decoder characteristic rather than a defect.

# 2. Signal Description

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| instr | 32 bit | Input | Full instruction word. For DSU (Custom-0) instructions: R-type layout for compute/load/clear/sat/readback ops (funct3=000), I-type layout for MACSHIFT (funct3=001). |
| dsu_en | 1 bit | Input | Global enable for the DSU. When 0, all functional enables are gated off, but the unconditional decode taps (funct5/funct3/is_custom0/rd_addr/shift fields) remain live. |
| flush | 1 bit | Input | Pipeline flush. Passes through unconditionally to control[0] (mac_flush) regardless of instruction validity or dsu_en. |
| control | 7 bit | Output | Packed MAC control bus: {mac_en, mac_addsub, mac_clear, mac_load, mac_abs, mac_dot, mac_flush}. |
| acc_sel | 2 bit | Output | Selects which accumulator (FX/FY/MAG) an op targets. Sourced from acc_sel_r (R-type) or imm_i[7:6] (I-type). |
| sat_op / shift_op / rd_lo_op / rd_hi_op | 1 bit ea. | Output | Per-operation enables for the Saturation unit, Barrel Shifter, and Read-back Mux. |
| shift_amt / shift_dir / shift_acc_sel | 6/1/2 bit | Output | Raw, unconditional slices of instr[31:20] -- always live regardless of instruction type, only meaningful when shift_op=1. |
| result_src | 2 bit | Output | Selects regfile writeback source: 00=none, 01=rd_lo, 10=rd_hi, 11=barrel shifter result. |
| writes_regfile | 1 bit | Output | Asserted for exactly one of {rd_lo_op, rd_hi_op, shift_op} when enabled and legal. |
| rd_addr | 5 bit | Output | Unconditional raw copy of instr[11:7]. Decoder does not special-case x0. |
| illegal_instr | 1 bit | Output | Asserted for a recognized-but-malformed Custom-0 instruction: bad funct3, bad funct5 (R-type), or acc_sel=11 -- while dsu_en=1. |
| funct5 / funct3 / is_custom0 | 5/3/1 bit | Output | Unconditional raw decode taps of instr, live regardless of legality or dsu_en. |

# 3. Verification Environment

A self-checking SystemVerilog class-based environment was built: transaction → generator → driver → monitor → scoreboard → env → test, using the same sequential drive-sample-check architecture validated during barrel_shifter verification (no fork/join, avoiding the driver/monitor race identified and fixed in that earlier exercise).

- Scoreboard reference model re-derives the decode logic independently (via explicit signal-level reasoning) rather than mirroring the RTL expression, so it cannot silently repeat an RTL bug.

- 12 concurrent SVA properties bound directly to the interface, checking spec invariants independently of the scoreboard.

- Stimulus = 78 directed test cases (16 categories, including two coverage-closure sweeps) + 200 constrained-random transactions, distribution-weighted across all 10 legal ops and 6 distinct illegal categories.

# 4. Directed Test Cases

| Category | Coverage Intent | Count / Result |
| --- | --- | --- |
| 1 | Each of the 10 legal ops (9 R-type MAC ops + MACSHIFT), dsu_en=1 | 10 cases / PASS |
| 2 | Each of the 10 legal ops again with dsu_en=0 -- verifies full functional gating while unconditional taps (funct5/funct3/is_custom0/rd_addr) stay live | 10 cases / PASS |
| 3 | Illegal: bad funct3 (not 000/001), including the boundary value directly above 001 | 2 cases / PASS |
| 4 | Illegal: bad funct5 in R-type, mid-range value | 1 case / PASS |
| 5 | CRITICAL CORNER -- funct5=9 on an R-type instruction. 9 is FUNCT5_MACSHIFT's own encoding, but MACSHIFT is reachable only via funct3=001 (I-type), never via a funct5 compare in the R-type branch -- must be flagged illegal, not decoded as an aliased shift. | 1 case / PASS |
| 6 | Illegal: bad funct5 at max representable value (31) | 1 case / PASS |
| 7 | Boundary sanity: funct5=8 (MACRD_HI, last legal value) must NOT be illegal | 1 case / PASS |
| 8 | Illegal: acc_sel=11 (R-type) / Boundary sanity: acc_sel=10 (last legal value) | 2 cases / PASS |
| 9 | Illegal: shift_acc_sel=11 (I-type) | 1 case / PASS |
| 10 | Non-Custom-0 opcode entirely | 1 case / PASS |
| 11 | Flush passthrough: alone (everything else off), alongside an illegal instruction, and alongside a legal enabled compute op simultaneously | 3 cases / PASS |
| 12 | rd_addr boundary: x0 and x31 | 2 cases / PASS |
| 13 | shift_amt boundary: 0 and 63 (max) | 2 cases / PASS |
| 14 | CRITICAL CORNER -- shift_amt/shift_dir/shift_acc_sel unconditional decode, verified even when the instruction is an R-type (not MACSHIFT) | 1 case / PASS |
| 15 | Coverage closure sweep: funct5 (12 representative values) x dsu_en, with a non-Custom-0 opcode -- closes cp_funct5 x cp_dsu_en x cp_is_custom0 cross | 24 cases / PASS |
| 16 | Coverage closure sweep: funct3 (all 8 values) x dsu_en, with a non-Custom-0 opcode -- closes cp_funct3 x cp_is_custom0 cross | 16 cases / PASS |

# 5. Constrained-Random Verification (CRV)

200 constrained-random transactions were generated via a dist-weighted rand+constraint model: legal ops weighted 10 each, illegal categories weighted 6 each, non-Custom-0 weighted 4. An early regression at this stimulus level reached 89% functional coverage; the gap was root-caused to the cp_funct5/cp_funct3 x cp_is_custom0 cross never varying funct5/funct3 for non-Custom-0 opcodes (those fields don't affect legality there, so the original stimulus held them fixed). Two deterministic sweep cases (category 15 and 16 above) were added to close that gap, raising coverage to 98.78% in this run.

# 6. Functional & Assertion Coverage

## 6.1 Functional Coverage Model

- cp_funct3: rtype{000} / itype{001} / 6 individual illegal_funct3 bins

- cp_funct5: 9 named legal-op bins + alias9{9} + mid_illegal[10:30] + max_illegal{31}

- cp_is_custom0, cp_dsu_en, cp_flush, cp_illegal: single-bit coverpoints

- cp_acc_sel: fx{00} / fy{01} / mag{10} / bad{11}

- cp_shift_amt: zero{0} / mid[1:62] / max{63}, cp_result_src: auto-bins over the 2-bit value

- cross cp_funct5 × cp_dsu_en × cp_is_custom0; cross cp_funct3 × cp_is_custom0; cross cp_acc_sel × cp_illegal

## 6.2 SVA Assertions

| Assertion | Checks | Status |
| --- | --- | --- |
| a_non_custom0_never_illegal | A non-Custom-0 opcode can never be flagged illegal_instr. | PASS |
| a_disabled_never_illegal | illegal_instr can never fire while dsu_en=0. | PASS |
| a_flush_passthrough | control[0] always equals flush, unconditionally. | PASS |
| a_funct5_unconditional | funct5 always equals instr[31:27]. | PASS |
| a_funct3_unconditional | funct3 always equals instr[14:12]. | PASS |
| a_is_custom0_unconditional | is_custom0 always equals (opcode==0x0B). | PASS |
| a_rd_addr_unconditional | rd_addr always equals instr[11:7]. | PASS |
| a_writeback_source_onehot0 | writes_regfile implies at most one of {rd_lo_op, rd_hi_op, shift_op}. | PASS |
| a_mac_modifiers_onehot0 | mac_en implies at most one of the 5 MAC modifier bits (addsub/clear/load/abs/dot). | PASS |
| a_illegal_gates_everything | illegal_instr forces every functional enable (mac_en, sat_op, shift_op, rd_lo_op, rd_hi_op, writes_regfile) off. | PASS |
| a_bad_accsel_is_illegal | acc_sel=11 while is_custom0 & dsu_en always flags illegal_instr. | PASS |
| a_shift_fields_unconditional | shift_amt/shift_dir/shift_acc_sel always equal their raw instr[31:20] slices. | PASS |

# 7. Results Summary

| Metric | Count | % of Total |
| --- | --- | --- |
| Total transactions applied (78 directed + 200 CRV) | 278 | 100% |
| PASS (scoreboard, vs. independent reference model) | 278 | 100% |
| FAIL (scoreboard, vs. independent reference model) | 0 | 0% |
| SVA property failures (12 properties) | 0 | - |
| Functional coverage (final) | - | 98.78% |

All 278 transactions passed against the independent scoreboard reference model, and all 12 SVA properties held for the full run with zero fires. No RTL defects were found in dsu_decoder.v during this exercise.

# 8. Open Item — Residual Coverage Gap

Functional coverage closed at 98.78%, up from an initial 89% after adding the two deterministic sweep cases in §5. The remaining ~1.2% is consistent with a small number of low-probability cross bins that constrained-random stimulus is not guaranteed to hit within a 200-transaction budget -- most likely within the cp_acc_sel × cp_illegal cross (e.g. acc_sel=11 coinciding with dsu_en=0, which is legal-but-unlikely under the current dist weighting) or a sparsely-weighted corner of the cp_funct5 × cp_dsu_en × cp_is_custom0 cross not included in the 12-value representative sweep.

Recommendation: pull the per-coverpoint/per-cross breakdown from the .ucd database (not just the aggregate percentage) to identify the exact unhit bin(s), then add one or two additional targeted directed cases to close them -- following the same method used to raise coverage from 89% to 98.78% in this exercise. Given all functional and assertion checks already pass at 100%, this is a coverage-completeness item, not a functional defect.

# 9. Signoff Status

- 100% pass on directed + constrained-random regression: MET (278/278 pass, 100%).

- All SVA properties pass: MET (12/12, zero fires across 278 transactions).

- Functional coverage goals met: PARTIAL (98.78% -- see §8 for the residual gap and recommended closure step).

- Bug burn-down: 0 RTL defects found in dsu_decoder.v.

Recommendation: dsu_decoder.v meets all functional and assertion signoff criteria. Pull the detailed coverage breakdown, close the remaining ~1.2% with 1-2 targeted directed cases, and re-run to confirm 100% before final Phase 1 signoff; the module is otherwise ready to proceed toward Phase 2 (Core Sanity TB) integration.