GARUDA SoC — DSU (Custom-0 MAC cluster)

# Unit-Level Verification Report

Module Under Test: dsu_stall.v

| Document | Unit Verification Report — dsu_stall |
| --- | --- |
| Project | GARUDA RV32IM SoC — Team AeroSoC |
| Subsystem | DSU (Custom-0 MAC cluster) — structural (read-after-read) hazard detector |
| Verification Phase | Phase 1 — Unit-Level Verification (per 1Tops RISC-V SoC Verification Strategy) |
| Simulator | Cadence Xcelium (xrun / xmsim) |
| Methodology | Exhaustive decode sweep + Directed sequences + CRV + SVA + Functional Coverage (stateful, cycle-accurate) |
| Overall Status | PASS — 2593/2593 cycles, 100% functional coverage, 2 architectural findings for review |

# 1. Module Overview

dsu_stall is the DSU's structural (read-after-read) hazard detector. Unlike every other module verified in this set, it is sequential: it maintains registered state (prev_was_2cycle, prev_acc_sel) tracking whether the previous non-stalled cycle's instruction was one of the accumulator-reading "2-cycle" ops (MACRD_LO, MACRD_HI, MACSAT). If the current cycle's instruction also reads the accumulator (via the same three ops, or MACSHIFT) AND targets the same acc_sel, dsu_busy is asserted.

- Reset is asynchronous (posedge clk or negedge rst_n) -- state clears the instant rst_n drops, independent of the clock.

- dsu_busy = stall_now is purely combinational from PRE-edge state -- a flush asserted the same cycle as a hazard does not retroactively cancel that cycle's stall; it only clears state for the following cycle.

- prev_was_2cycle/prev_acc_sel are held (not updated) while dsu_busy is asserted -- the module remembers the pending hazard until it resolves.

# 2. Signal Description

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| clk | 1 bit | Input | Free-running pipeline clock. The only sequential module in this verification set (barrel_shifter, dsu_decoder, csa_3to2 are all combinational). |
| rst_n | 1 bit | Input | Asynchronous active-low reset (posedge clk or negedge rst_n). Clears prev_was_2cycle/prev_acc_sel the instant it drops, not at the next clock edge. |
| flush | 1 bit | Input | Pipeline flush. Clears prev_was_2cycle/prev_acc_sel on the NEXT clock edge -- does not retroactively cancel a stall already combinationally asserted this cycle. |
| funct5 / funct3 / is_custom0 / acc_sel | 5/3/1/2 bit | Input | Decoded fields of the instruction currently at this pipeline stage, sourced from dsu_decoder. |
| dsu_busy | 1 bit | Output | Structural hazard stall: asserted when the current instruction reads the same accumulator (acc_sel) as a 2-cycle op (MACRD_LO/MACRD_HI/MACSAT) issued the previous (non-stalled) cycle. |

# 3. Verification Environment

Because dsu_stall has real registered state, correctness is defined by cycle-accurate SEQUENCES, not independent single-cycle transactions. The environment drives one instruction per clock and checks dsu_busy every cycle against a STATEFUL reference model that independently mirrors the RTL's register-update priority (reset > flush > hold-while-stalled > normal update), implemented via a case statement on funct5 rather than the RTL's chained equality/OR expression.

- Every named sequence is isolated by a real async-reset pulse applied to the DUT before it runs, keeping the reference model's state in lockstep with the actual DUT (see §7, TB-001).

- 4 concurrent SVA properties bound directly to the interface, checking spec invariants independently of the scoreboard.

- Exhaustive phase: all 512 (funct5, funct3, is_custom0) combinations swept against both a hazard-pending and a no-hazard-pending base state (1024 total exhaustive sequences), exhaustively validating the read-family decode logic rather than sampling it.

# 4. Directed, Exhaustive & CRV Coverage

| Category | Coverage Intent | Count / Result |
| --- | --- | --- |
| 1 | Basic hazard trigger: two 2-cycle ops on the same accumulator back-to-back, hazard clears once a non-reading op follows | 1 sequence / PASS |
| 2 | No-hazard: two 2-cycle ops on different accumulators | 1 sequence / PASS |
| 3 | Full producer x consumer matrix (MACRD_LO/MACRD_HI/MACSAT as producer, x MACRD_LO/MACRD_HI/MACSAT/MACSHIFT as consumer), same accumulator | 12 sequences / PASS |
| 4 | Reset mid-hazard: establish a pending hazard, assert async rst_n, confirm the pending state is truly cleared | 1 sequence / PASS |
| 5 | Flush mid-hazard, and flush asserted on the SAME cycle as the hazard (verifies flush only affects the NEXT cycle's state, not this cycle's combinational stall) | 2 sequences / PASS |
| 6 | FINDING 1: compute op (MAC_SEL/MACSUB/MACABS/MACDOT/MACLOAD/MACCLEAR) immediately followed by a read on the same accumulator -- see §8 | 6 sequences / PASS (vs. spec) |
| 7 | FINDING 2: held-stall probe -- same triggering instruction repeated for 6 consecutive cycles -- see §8 | 1 sequence / PASS (vs. spec) |
| 8 | acc_sel boundary matrix: all 4x4 combinations of producer/consumer accumulator selector, matched and mismatched | 16 sequences / PASS |
| 9 | Non-Custom-0 opcode and bad-funct3 instructions correctly break a pending hazard chain (don't read the accumulator) | 2 sequences / PASS |
| 10 | EXHAUSTIVE: every (funct5, funct3, is_custom0) combination -- 32x8x2=512 -- swept against both a no-hazard-pending and a hazard-pending base state | 1024 sequences / PASS |
| 11 | Constrained-random multi-cycle sequences (dist-weighted op mix, ~3% async reset injection, 15% flush injection) | 15 sequences x 30 cycles / PASS |

# 5. SVA Assertions

| Assertion | Checks | Status |
| --- | --- | --- |
| a_no_x_propagation | Known inputs (post-reset) never produce an unknown dsu_busy. | PASS |
| a_reset_clears_busy | dsu_busy reads 0 on the cycle immediately following reset release. | PASS |
| a_flush_clears_busy_next_cycle | dsu_busy reads 0 on the cycle immediately following a flush. | PASS |
| a_busy_implies_reads_acc | dsu_busy can only be asserted for an instruction that actually reads the accumulator. | PASS |

# 6. Functional Coverage Model

- cp_busy, cp_flush, cp_rst_n: single-bit coverpoints on dsu_busy, flush, rst_n

- cp_op_reads_acc: whether the current instruction reads the accumulator at all

- cross cp_busy × cp_flush: all 4 combinations reachable and hit, including (busy=1, flush=1) via the same-cycle flush+hazard sequence (§4, category 5)

- cross cp_busy × cp_rst_n: 1 of 4 combinations -- (busy=1, rst_n=0) -- excluded via a documented ignore_bins. This is not a functional gap: this environment's reset is asynchronous and sampled after the driver has already applied it, so by sample time a rst_n=0 cycle's registers have already cleared, making busy=1 unobservable at that sample point by construction. Forcing it would mean sampling inside the async-clear propagation delta rather than testing a real pipeline state.

# 7. Testbench Defects Found & Fixed

An earlier regression against this same DUT and stimulus set reported 20 failures out of 2593 checks. Root-causing traced every one of them to two testbench defects, not RTL defects -- both are documented here as part of this exercise's verification record, since this is exactly the kind of methodology maturation a real signoff process should show.

| Defect | Root Cause & Fix | Status |
| --- | --- | --- |
| TB-001: sequence state leakage | Reference model reset fresh at the start of every named sequence, but dsu_stall is a single continuous simulation whose internal registers genuinely carry state across sequence boundaries. Fixed by inserting a real async-reset pulse on the DUT between every sequence, keeping model and DUT state in lockstep. | RESOLVED |
| TB-002: async-reset ordering | Reference model computed dsu_busy from pre-reset state, then cleared state afterward. dsu_stall's reset is asynchronous and the driver applies it before the monitor's sampling point, so the real register has already cleared by sample time. Fixed by reordering step() to apply an asserted reset before computing busy. | RESOLVED |

With both fixes applied, the identical stimulus set produced 2593/2593 PASS -- confirming the DUT was correct throughout; only the checking methodology needed correction.

# 8. Architectural Findings for Review

## 8.1 Finding 1 — Possible missing compute→read hazard

cur_is_2cycle (the condition that arms a future stall) only fires for MACRD_LO/MACRD_HI/MACSAT. It never fires for the compute ops (MAC_SEL, MACSUB, MACABS, MACDOT, MACLOAD, MACCLEAR). Tracing mac_unit.v's actual pipeline separately (outside this unit-level exercise): a compute op's product does not fold into the accumulator until at least one further enabled cycle, due to its 2-stage carry-save reduction. A MACRD_LO/MACRD_HI/MACSAT issued immediately after a compute op on the same accumulator may therefore read a stale value with no stall inserted. The 6 directed sequences in category 6 (§4) demonstrate this sequence exactly and confirm dsu_stall does not flag it -- correct behavior relative to dsu_stall's own written specification, but a question for whoever owns the cross-module timing contract between this unit and mac_unit.

## 8.2 Finding 2 — Held-stall duration is unbounded by construction

prev_was_2cycle/prev_acc_sel freeze while dsu_busy is asserted. The FINDING2 sequence held an identical triggering instruction constant for 6 consecutive cycles; dsu_busy remained asserted for all 6 and only cleared once the instruction actually changed (measured directly: MAX OBSERVED CONSECUTIVE dsu_busy CYCLES = 6, matching exactly). This confirms the mechanism: dsu_busy will not self-clear while the same instruction keeps being re-presented. Whether this is ever reachable in practice depends entirely on whether the pipeline upstream of this unit guarantees the stalled instruction is eventually replaced (e.g. by injecting a bubble) -- a question for the pipeline-freeze/replay logic owner, not something resolvable from this unit-level testbench alone.

# 9. Results Summary

| Metric | Count | % of Total |
| --- | --- | --- |
| Total sequences (11 directed categories + 1024 exhaustive + 15 CRV) | 1081 | 100% |
| Total clock cycles checked | 2593 | - |
| PASS (scoreboard, vs. independent stateful reference model) | 2593 | 100% |
| FAIL | 0 | 0% |
| SVA property failures (4 properties) | 0 | - |
| Functional coverage (final, with 1 documented ignore_bins) | - | 100.00% |
| Max observed consecutive dsu_busy cycles (any sequence) | 6 | see §8 Finding 2 |

# 10. Signoff Status

- 100% pass on directed + exhaustive + CRV regression: MET (2593/2593, 100%).

- All SVA properties pass: MET (4/4, zero fires across 2593 cycles).

- Functional coverage goals met: MET (100.00%, with 1 documented and justified ignore_bins).

- Bug burn-down: 0 RTL defects found in dsu_stall.v; 2 testbench defects found and fixed (§7).

- Open items: 2 architectural findings (§8) requiring human review outside the scope of this unit-level exercise -- neither is a dsu_stall defect against its own specification.

Recommendation: dsu_stall.v meets all Phase 1 unit-level signoff criteria against its own written specification. Before this module is exercised in Phase 2 (Core Sanity TB), route Findings 1 and 2 to the architecture/integration owner: confirm whether mac_unit.v's compute-op latency requires a stall condition this unit does not currently implement, and confirm the pipeline's stall-retry contract guarantees the held instruction is eventually replaced.