# GARUDA CLIC Interrupt Controller — Design Specification

## 0.1 Identity

| Field | Value |
| --- | --- |
| Document ID | GARUDA-CLIC-SPEC-001 |
| Revision | 2.0 |
| Date | 2026-09-18 |
| Status | Released for implementation |
| Block | 10 (clic) |
| Owner | Team AeroSoC |
| Supersedes | GARUDA_CLIC_Design_Spec_v1_1 |

## 0.2 Revision history

| Rev | Change | Driver |
| --- | --- | --- |
| 1.0–1.1 | Initial. ID 13 carried mtip; trigger type configurable; ID map partly provisional | — |
| 2.0 | ID 13 freed — mtip no longer routes through the CLIC. Trigger type fixed in hardware. ID 0 becomes a permanently unassigned sentinel. ID 14 freed by the SPI-slave removal. ID map now generated from GARUDA-SYS-001. mintthresh interface clarified. | ADR-0009, 0010, 0020 |

## 0.3 Normative references

- RISC-V Privileged Architecture v1.12, Machine-level ISA.

- RISC-V CLIC specification, draft (the subset implemented is enumerated in §13).

- GARUDA-SYS-001 Rev 4.0 — the ID map is generated from clic.map.

- GARUDA-ADR-001 Rev 1.0.

- GARUDA-CORE-SPEC-001 Rev 3.0 — clic_ctrl lives inside the core; this block is the aggregator.

- GARUDA-TIMERS-SPEC-001 Rev 2.0 — for why mtip bypasses this block.

## 1 Purpose and scope

### 1.1 Division of responsibility

This is worth stating first, because the split is unusual and Rev 1.1 was vague about it.

| Function | Where it lives |
| --- | --- |
| Collecting peripheral and DMA interrupt lines | this block (10) |
| Per-ID enable, pending and level registers | this block |
| Selecting the highest-level pending enabled ID | this block |
| The take decision against mstatus.MIE, mintthresh and mintstatus.mil | clic_ctrl inside the core |
| Vectoring, mcause writeback, mnxti | core |

[N-1.1] clic_ctrl is already implemented and verified inside garuda_core_top. This block supplies it with a single winning ID and its level; it does not duplicate the take logic. Splitting it this way keeps the take decision in the same timing cone as the CSRs it reads.

### 1.2 In scope

Interrupt aggregation, the per-ID register file, level-based selection, and the APB configuration interface on window 10.

### 1.3 Out of scope

- The trap entry sequence and mcause (core).

- The machine timer interrupt, which does not pass through here at all (§7.6).

## 2 Requirements

| ID | Requirement | Source |
| --- | --- | --- |
| R-1 | Aggregate 32 interrupt sources into a single winning ID and level for the core. | System |
| R-2 | Per-ID enable, pending, and 8-bit level. | CLIC |
| R-3 | All sources shall be level-triggered, fixed in hardware. | ADR-0009 |
| R-4 | All interrupts disabled at reset. | Safety |
| R-5 | ID 0 shall never be assigned to any source. | ADR-0009 |
| R-6 | The machine timer shall not consume a CLIC ID. | ADR-0010 |
| R-7 | Selection shall complete in one cycle and add no latency to the core's take decision. | Timing |
| R-8 | An ID's level shall be changeable while that ID is pending, without losing the pending state. | Firmware |

## 3 Block diagram

DMA complete[5:0] ──┐
  DMA error[5:0]    ──┤
  spi_master_irq    ──┤      ┌─────────────────────────────────┐
  i2c_irq           ──┤      │          clic (block 10)        │
  uart0_irq         ──┼─────▶│                                 │
  uart1_irq         ──┤      │  ┌───────────────────────────┐  │
  uart2_irq         ──┤      │  │ pending[31:0]             │  │
  gpio_irq          ──┤      │  │  (level-sensitive,        │  │
  pwm_fault_irq     ──┤      │  │   combinational from src) │  │
  wdt_warn_irq      ──┘      │  └────────────┬──────────────┘  │
                             │               │                 │
                             │  ┌────────────▼──────────────┐  │
                             │  │ enable[31:0]  (APB)       │  │
                             │  │ level[31:0][7:0] (APB)    │  │
                             │  └────────────┬──────────────┘  │
                             │               │                 │
                             │  ┌────────────▼──────────────┐  │
                             │  │  level_select             │  │
                             │  │  max level, lowest ID     │  │
                             │  │  wins ties                │  │
                             │  └────────────┬──────────────┘  │
                             │               │                 │
                             └───────────────┼─────────────────┘
                                             │ clic_irq_id[4:0]
                                             │ clic_irq_level[7:0]
                                             │ clic_irq_valid
                                             ▼
                              ┌──────────────────────────────┐
                              │ clic_ctrl  (inside the core) │
                              │ take = valid & MIE           │
                              │      & level > mintthresh    │
                              │      & level > mintstatus.mil│
                              └──────────────────────────────┘

  mtip ──────────────────────▶ core mip.MTIP directly.  NOT through here.
                               (ADR-0010, §7.6)

## 4 Sub-block inventory

| Name | Type | RTL file | Role |
| --- | --- | --- | --- |
| irq_capture | comb | clic_top.v | Maps source lines to ID positions. Pure wiring, no state. |
| clic_regfile | seq | clic_regfile.v | enable, level per ID. APB-writable. |
| level_select | comb | clic_select.v | Two-stage comparison tree over 32 IDs. |
| clic_apb | seq (pclk) | clic_apb.v | Window 10 register interface. |

## 5 Interfaces

| Port | Dir | Width | Domain | Reset | Description |
| --- | --- | --- | --- | --- | --- |
| hclk_i, hreset_n_i | in | 1 | hclk | — |  |
| irq_src_i | in | 32 | hclk | — | Source lines, positioned per the ID map. Unassigned positions tied low. |
| clic_irq_id_o | out | 5 | hclk | 0 | Winning ID. |
| clic_irq_level_o | out | 8 | hclk | 0 | Winning level. |
| clic_irq_valid_o | out | 1 | hclk | 0 | At least one enabled source is pending. |
| APB slave | — | — | pclk | — | Window 10. |

[N-5.1] There is no mintthresh input to this block. Rev 1.1 implied one, and the core RTL exposes a clic_mintthresh_o port that no consumer reads. The threshold comparison is part of the take decision and belongs in clic_ctrl with the other CSR reads; performing it here would mean shipping the threshold out of the core and the result back, adding a round trip to a path that is already in the core's critical cone. The orphan clic_mintthresh_o port shall be removed from the core (RTL delta R7).

[N-5.2] irq_src_i is a flat 32-bit vector, positioned by the ID map. The top level does the mapping, so this block contains no per-peripheral knowledge and the ID map exists in exactly one place.

## 6 Register map — APB window 10 (0x4000_A000)

(Window number corrected 2026-09-26: was stated as 9, inherited from the 0-based table in GARUDA-AHB2APB-SPEC-001. The hardware decodes window n at 0x4000_0000 + 0x1000 x n (haddr[15:12]), so the base address quoted here was always right and only the index was wrong.)

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x000 | CLICINFO | RO | see §6.1 | Implementation parameters. |
| 0x004 | CLICIE | RW | 0x0000_0000 | Per-ID enable, bit n = ID n. |
| 0x008 | CLICIP | RO | — | Per-ID pending, read-only. |
| 0x100+4n | CLICINTCFG[n] | RW | 0x00 | 8-bit level for ID n. n = 0..31. |

### 6.1 CLICINFO (0x000)

| Bits | Name | Value | Description |
| --- | --- | --- | --- |
| 12:0 | num_interrupt | 32 | Number of IDs. |
| 20:13 | version | 1 |  |
| 24:21 | CLICINTCTLBITS | 8 | Level bits implemented. |
| 31:25 | reserved | 0 |  |

### 6.2 CLICIE (0x004)

[N-6.1] Reset value is zero: every interrupt is disabled at reset. Firmware enables sources explicitly after configuring the peripherals that drive them. A source enabled before its peripheral is configured can fire on a spurious level and trap into a handler whose state is not yet initialised.

[N-6.2] Bit 0 is hardwired to zero and ignores writes. ID 0 is never assigned (§7.5).

[N-6.3] Bit 7 and bits 31:23 are hardwired to zero and ignore writes. ID 7 is reserved because the core reports a CLIC interrupt as mcause = 0x8000_0000 | ID, and 0x8000_0007 is the machine-timer cause (Docs/BUGS.md INT-2). IDs 23–31 have no source. The enable mask is generated from the ID map (GARUDA_CLIC_ID_MASK = 0x007F_FF7E, Docs/BUGS.md INT-3). Reserved-and-unwritable rather than merely unassigned, so an enable of an ID with no source cannot produce a permanently-clear pending bit that a debugging engineer then spends an afternoon on.

### 6.3 CLICIP (0x008)

[N-6.4] Read-only, and combinational from the source lines. There is no write path, no set register and no clear register.

[N-6.5] This follows from level triggering: a pending bit that firmware could clear while the source is still asserted would immediately re-assert, and firmware that expected the clear to work would spin. Clearing an interrupt means clearing the condition in the peripheral, which is what its own register map provides.

### 6.4 CLICINTCFG[n] (0x100 + 4n)

| Bits | Name | Description |
| --- | --- | --- |
| 7:0 | level | Interrupt level. 0 = never taken. |

[N-6.6] All 8 bits are implemented, giving 255 usable levels. A level of 0 means the interrupt is never taken even when enabled and pending, because the take condition requires level > mintthresh and mintthresh is at minimum 0.

[N-6.7] Writing CLICINTCFG[n] while ID n is pending changes the level without disturbing the pending state, since pending is combinational from the source (R-8). Changing the level of an interrupt that is currently being serviced changes the mintstatus.mil comparison for nested interrupts and is firmware's responsibility.

[N-6.8] CLICINTCFG for IDs 0, 7 and 23–31 is writable but has no effect, as those IDs can never be enabled.

### 6.5 ID map

Generated from GARUDA-SYS-001 clic.map:

| ID | Source | Block |
| --- | --- | --- |
| 0 | permanently unassigned sentinel | — |
| 1–6 | DMA channel 0–5 complete | 9 |
| 7 | reserved: mcause 0x8000_0007 is the machine timer (INT-2) | — |
| 8–13 | DMA channel 0–5 error | 9 |
| 14 | spi_slave (ADR-0020 Rev 2) | 14 |
| 15 | spi_master | 13 |
| 16 | i2c | 15 |
| 17 | uart0 (GPS) | 16 |
| 18 | uart1 (ground link) | 17 |
| 19 | uart2 (console) | 18 |
| 20 | gpio aggregate | 19 |
| 21 | pwm fault | 20 |
| 22 | watchdog early warning | 11 |
| 23–31 | reserved | — |

## 7 Functional description

### 7.1 Pending

[N-7.1] pending[n] is combinational: pending[n] = irq_src_i[n]. No capture, no latch, no synchroniser. Every source is already in the hclk domain — the peripherals' interrupt outputs cross from pclk to hclk, and that crossing is synchronous (GARUDA-AHB2APB-SPEC-001 §7.2), so no CDC is needed.

### 7.2 Level triggering, fixed in hardware

[N-7.2] Every source is level-triggered. There is no edge-detect logic and no configuration bit to select trigger type.

[N-7.3] Rationale: trigger type is a property of the source's hardware, not a firmware preference. Every source in this chip holds its line asserted until its condition is cleared in the peripheral — a UART's receive-FIFO-not-empty, a DMA channel's complete flag, the watchdog's warning threshold. Making trigger type configurable would only create a way to configure it wrongly, and an edge-triggered configuration on a level source loses interrupts that arrive while the handler is running.

[N-7.4] The firmware consequence, which must be understood: a handler must clear the condition in the peripheral before returning. mret with the source still asserted re-enters the handler immediately. This is the standard level-triggered contract and it is stated here because Rev 1.1's configurable trigger type left it ambiguous.

### 7.3 Selection

[N-7.5] level_select finds the enabled pending ID with the highest level. Ties are broken by the lowest ID.

[N-7.6] Implemented as a balanced binary tree of 2-input comparators (clic_select.v), five levels deep at 32 inputs. Each comparison carries {level, ~id} so that a single magnitude comparison resolves both the level and the tie-break.

[N-7.7] The lowest-ID tie-break puts the DMA channels (IDs 1–6 and 8–13) ahead of the peripherals at equal level, which matches the priority reasoning in GARUDA-SYS-001 dma.priority_note — a DMA channel not serviced loses data, a peripheral interrupt merely waits.

[N-7.8] clic_irq_valid_o is the OR of enable & pending. It is presented to clic_ctrl in the same cycle, and clic_ctrl performs the take decision. This block adds one combinational tree to that path and no register stage (R-7).

### 7.4 The take decision (informative — implemented in the core)

[N-7.9] For completeness, since this document's output feeds it directly:

take = clic_irq_valid
     & mstatus.MIE
     & (clic_irq_level >  mintthresh)
     & (clic_irq_level >  mintstatus.mil)

[N-7.10] Both comparisons are strictly greater. An interrupt at exactly the current level does not preempt. Equal-level preemption would allow a handler to be interrupted by another instance of its own level, which recurses and overflows the stack.

[N-7.11] mintstatus.mil is the level of the interrupt currently being serviced, so the second comparison is what implements nesting.

### 7.5 ID 0 as a sentinel

[N-7.12] ID 0 is never assigned to any source, CLICIE[0] is hardwired low, and irq_src_i[0] is tied low at the top level.

[N-7.13] Rationale: clic_irq_id_o reads 0 when nothing is pending. If ID 0 were also a real source, an engineer at bring-up could not distinguish "no interrupt" from "source 0 fired", and an uninitialised or stuck-at-zero ID path would look like normal operation. Reserving it means a trap with mcause ID 0 is unambiguously a bug, visible the first time it happens. This costs one of 32 IDs, of which nine are already spare.

### 7.6 The machine timer does not come through here

[N-7.14] mtip reaches the core as a single wire into mip.MTIP, generated by the timers block (GARUDA-TIMERS-SPEC-001 §7). It does not occupy a CLIC ID.

[N-7.15] Rev 1.1 assigned it ID 13, while the core already implemented mip.MTIP directly and that path was verified. Two mechanisms existed for one interrupt. The core's path is kept because it works and is tested; ID 13 is freed.

[N-7.16] The consequence, stated honestly: the machine timer cannot be level-prioritised against CLIC sources, and cannot preempt or be preempted by them under the level scheme. It is the RISC-V architectural timer driving a 1 kHz control loop; it does not need to participate in the level hierarchy. Firmware controls its priority relative to everything else through mstatus.MIE in the timer handler.

### 7.7 Spurious and vestigial cases

[N-7.17] If the winning source deasserts between clic_irq_valid_o asserting and the core taking the trap, clic_ctrl may enter a handler for a condition that is no longer present. The handler must tolerate this by checking the peripheral's own status register before acting. This is inherent to level triggering with a pipelined take decision and cannot be removed by this block.

[N-7.18] If the winning ID changes between valid asserting and the take, the core takes the new winner. mcause is written from the ID present at the take cycle, not the one that first caused valid to assert, so mcause is always consistent with the handler that runs.

## 8 Timing

### 8.1 Single interrupt taken

│ T0 │ T1 │ T2 │ T3 │
hclk        ──┘‾┐_┌‾┐_┌‾┐_┌‾┐_┌

irq_src[17] ──────┌──────────────────  UART0 RX FIFO not empty
pending[17] ──────┌──────────────────  combinational  [N-7.1]
enable[17]  ──┌──────────────────────  set earlier by firmware

irq_valid   ──────┌──────────────────
irq_id      ──────┤  17  ├───────────
irq_level   ──────┤  80  ├───────────

(in core)
take        ──────────┌──┐───────────  clic_ctrl, same cycle + 1
mcause      ──────────────┤ 17 ├─────
pc          ──────────────┤ vector ├─

### 8.2 Tie-break: equal level, lowest ID wins

irq_src[3]  ──┌──────────  DMA ch2 complete, level 100
irq_src[18] ──┌──────────  UART1,           level 100

irq_id      ──┤  3  ├────  lowest ID wins the tie  [N-7.5]
irq_level   ──┤ 100 ├────

### 8.3 Nesting: higher level preempts

│ T0        │ T1          │ T2        │
irq_src[17]  ─┌──────────────────────────────────────  level 80
irq_src[15]  ────────────────┌────────────────────────  level 200

mintstatus   ─┤ 0 ├─┤   80    ├─┤   200    ├─┤  80  ├
  .mil
                    ▲          ▲            ▲
                    │          │            └── mret: back to 80
                    │          └── 200 > 80, preempts
                    └── handler 17 entered

irq_id       ─┤17├──┤   17    ├─┤   15     ├─┤  17  ├

### 8.4 Equal level does NOT preempt

irq_src[17]  ─┌──────────────────────────  level 80, being serviced
irq_src[18]  ────────────┌────────────────  level 80

mintstatus   ─┤   80                     ├  unchanged
  .mil
take         ──────────────────────────────  never asserts  [N-7.10]
                         ▲
                         └── ID 18 waits until mret

## 9 Clock, reset and power

[N-9.1] The interrupt path — sources, pending, enable, level, selection — is entirely in hclk. The APB register interface is in pclk. Register values cross pclk→hclk synchronously, no CDC.

[N-9.2] CLICIE resets to zero (R-4). CLICINTCFG resets to zero, so an interrupt that is enabled before its level is set is still never taken (level 0). Both defaults fail safe.

[N-9.3] Power: level_select is combinational and re-evaluates whenever any source or configuration changes. With all interrupts idle its inputs are static and it consumes nothing. No clock gating is applied; the register file is small and must respond to a source asserting at any time.

[N-9.4] An enabled, pending interrupt must be able to wake the core from WFI. The clic_irq_valid_o term feeds the core's wake condition directly and is not gated by mstatus.MIE, so WFI wakes on a pending enabled interrupt even with interrupts globally disabled, as the privileged specification requires. This block therefore cannot be clock gated on "core asleep".

## 10 Assertions

// --- ID 0 is never a winner and never enableable (R-5, N-7.12)
a_id0_never_enabled: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i) !clic_ie[0]);
a_id0_never_wins: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  clic_irq_valid_o |-> (clic_irq_id_o != 5'd0));

// --- reserved IDs can never be enabled (N-6.3)
a_reserved_ids: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (clic_ie[7] == 1'b0) && (clic_ie[31:23] == 9'd0));

// --- valid is exactly the OR of enable & pending
a_valid_correct: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  clic_irq_valid_o == |(clic_ie & clic_ip));

// --- the winner really is enabled and pending
a_winner_legit: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  clic_irq_valid_o |-> (clic_ie[clic_irq_id_o] && clic_ip[clic_irq_id_o]));

// --- the winner has the maximum level among enabled pending IDs
a_winner_max_level: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  clic_irq_valid_o |-> (clic_irq_level_o == max_enabled_pending_level));

// --- tie-break is lowest ID (N-7.5)
a_tiebreak_lowest_id: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  clic_irq_valid_o |->
    !$isunknown(clic_irq_id_o) &&
    (!(|(clic_ie & clic_ip & lower_id_mask(clic_irq_id_o) & same_level_mask(clic_irq_level_o)))));

// --- pending is purely combinational from the source (N-7.1)
a_pending_comb: assert property (
  @(posedge hclk_i) clic_ip == irq_src_i);

// --- no pending set/clear path exists: CLICIP is unaffected by writes
a_clicip_ro: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  (apb_write && apb_addr == 12'h008) |=> (clic_ip == irq_src_i));

// --- level change while pending does not disturb pending (R-8)
a_level_change_safe: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (clic_ip[17] && $changed(clic_level[17])) |-> clic_ip[17]);

// --- everything disabled at reset (R-4)
a_reset_disabled: assert property (
  @(posedge hclk_i) !hreset_n_i |=> (clic_ie == 32'd0 && clic_irq_valid_o == 1'b0));

// --- no CLIC ID is ever asserted for the machine timer (R-6, N-7.14)
a_no_mtip_id: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i) !irq_src_i[7]);

// --- selection adds no register stage (R-7)
a_no_latency: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  $rose(|(clic_ie & clic_ip)) |-> clic_irq_valid_o);

## 11 Verification plan

| Req | Test | Oracle | Coverage goal | Status |
| --- | --- | --- | --- | --- |
| R-1 | t_clic_single | each source individually reaches the core with the right ID | all 20 assigned IDs | new |
| R-2 | t_clic_regs | enable, pending, level read/write behaviour | all 32 CLICINTCFG | new |
| R-3 | t_clic_level_trig | source held → handler re-enters after mret | — | new |
| R-3 | t_clic_no_edge | a pulse shorter than the take latency may be missed; a held source never is | — | new |
| R-4 | t_clic_reset | all disabled after reset; no spurious take | — | new |
| R-5 | t_clic_id0 | CLICIE[0] unwritable; ID 0 never wins | — | new |
| R-6 | t_clic_no_mtip | timer interrupt arrives via mip.MTIP, no CLIC ID involved | — | new |
| R-7 | t_clic_latency | valid in the same cycle as enable & pending | — | new |
| R-8 | t_clic_level_change | level write while pending, pending preserved | — | new |
| §7.3 | t_clic_tiebreak | equal level → lowest ID | all adjacent ID pairs | new |
| §7.4 | t_clic_nest | higher level preempts; equal does not | 8 level pairs | new |
| §7.4 | t_clic_thresh | mintthresh blocks levels at or below it | boundary values | extend |
| §7.7 | t_clic_spurious | source deasserts before the take; handler tolerates | — | new |
| §9.4 | t_clic_wfi_wake | pending enabled interrupt wakes WFI with MIE clear | — | new |
| — | t_clic_random | constrained-random source assertion vs. a reference model | all 20 IDs, nesting depth ≥3 | new |

[N-11.1] t_clic currently times out and the cause is not diagnosed. The existing sanity suite is 8 of 9 passing, with t_clic the failure. It must be diagnosed before any of this block's RTL is written: the failure is in the CLIC path, the take logic in clic_ctrl is what this block feeds, and building an aggregator on top of an unexplained hang in the consumer would make the resulting bug very hard to localise. The same symptom class produced the T-4 fault-loop erratum in the core, so it should not be assumed benign.

[N-11.2] t_clic_nest should be reachable by formal for depth 2–3, which is worth doing: nesting bugs depend on the interleaving of take and mret, and directed tests cover only the interleavings someone thought of.

## 12 Design decisions

| Decision | ADR |
| --- | --- |
| IDs assigned once in GARUDA-SYS-001; level-triggered fixed in hardware; ID 0 sentinel | ADR-0009 |
| mtip does not consume a CLIC ID; ID 13 freed | ADR-0010 |
| ID 14 restored to the SPI slave | ADR-0020 Rev 2 |
| ID 7 reserved; DMA errors on IDs 8–13 | Docs/BUGS.md INT-2 |
| Take decision stays in clic_ctrl inside the core; no mintthresh port on this block | ADR-0009, §5.1 |

## 13 Not implemented

| CLIC feature | Why not |
| --- | --- |
| Edge-triggered mode and per-ID trigger configuration | §7.2: trigger type is a hardware property; configurability only permits misconfiguration. |
| CLICIP write, set or clear registers | §6.3: meaningless under level triggering. |
| Supervisor or user mode CLIC | M-mode only chip. |
| mnxti | Not implemented in the core; its CSR address reads 0. Interrupt-tail-chaining is a performance optimisation for high interrupt rates that this workload does not have. |
| Selective hardware vectoring (CLICINTATTR.shv) | All interrupts use the same vectored entry. A per-ID vector table would need a base register and a memory read in the trap path. |
| More than 32 IDs | 21 assigned (1–6, 8–22), 9 spare (23–31), 2 reserved (0, 7). No growth pressure. |
| Fewer than 8 level bits | 8 bits is what the core's mintthresh and mintstatus.mil already carry; narrowing would save a handful of flops and create a mismatch. |
| Software-triggerable interrupts | No use case; firmware can call the handler directly. |

## 14 Open items

OPEN-8 (carried, not closed here): the t_clic timeout. This is a debug task, not a design decision, and it is a prerequisite for this block's RTL. See [N-11.1].

## 15 Errata

No RTL for this block yet. clic_ctrl inside the core is implemented; its errata are in GARUDA-CORE-SPEC-001 §15.

Defects fixed in this revision:

| ID | Symptom | Root cause | Fix |
| --- | --- | --- | --- |
| CL-1 | Two mechanisms for one interrupt: ID 13 in three documents, mip.MTIP in the core RTL. | Timer routing decided independently in the CLIC, Timers and Core specs. | ADR-0010; ID 13 reserved; §7.6. |
| CL-2 | A configurable trigger type permitted an edge configuration on level sources, losing interrupts. | Configurability added without asking whether any source needed it. | §7.2, fixed in hardware. |
| CL-3 | ID 0 assignable, making "no interrupt" indistinguishable from "source 0" at bring-up. | No sentinel convention. | §7.5. |
| CL-4 | An orphan clic_mintthresh_o port on the core with no consumer, implying this block performed the threshold comparison. | Take logic's location left ambiguous between two documents. | §5.1; port removed (RTL delta R7). |
| CL-5 | Six IDs marked Provisional; the DMA spec assigned CLIC to block 16 while the TRM said 10. | ID and block numbers maintained by hand in three documents. | Generated from GARUDA-SYS-001. |