# GARUDA Clock and Reset Subsystem — Design Specification

## 0.1 Identity

| Field | Value |
| --- | --- |
| Document ID | GARUDA-CLKRST-SPEC-001 |
| Revision | 2.0 |
| Date | 2026-09-18 |
| Status | Released for implementation |
| Blocks | 21 (clk_div), 22 (reset_ctrl) |
| Owner | Team AeroSoC |
| Supersedes | GARUDA_ClkRst_Design_Spec_v1_1 |

## 0.2 Revision history

| Rev | Change | Driver |
| --- | --- | --- |
| 1.0 | Initial | — |
| 1.1 | Frequency raised to 250 MHz | — |
| 2.0 | Reset combining rewritten: stretch counter, source-outside-own-domain rule, DM carved out of ndm_rst_n. pclk moved into this document as a real 125 MHz toggle-flop divide. Divider placement fixed at the pad. Analog POR dependency removed. | ADR-0001, 0002, 0003, 0018, 0019 |

## 0.3 Normative references

- GARUDA-SYS-001 Rev 4.0 — System Definition. All numeric values in this document are generated from it.

- GARUDA-ADR-001 Rev 1.0 — Architecture Decision Record.

- GARUDA-DEBUG-SPEC-001 Rev 2.0 — for ndmreset and hartreset semantics.

- GARUDA-TIMERS-SPEC-001 Rev 2.0 — for the watchdog reset request.

## 1 Purpose and scope

### 1.1 In scope

clk_div (block 21) takes the external 500 MHz reference and produces the chip's two functional clocks: hclk at 250 MHz and pclk at 125 MHz.

reset_ctrl (block 22) collects the three reset requests, stretches them, releases them synchronously, and records why the last reset happened.

### 1.2 Out of scope

- The board-level supply supervisor that drives ext_rst_n (§2 [R-4] states the requirement it must meet).

- Clock tree synthesis and balancing (physical design).

- The per-block clock gates for power management — those live in the blocks they gate. This document defines only the root clocks they gate from.

### 1.3 Why this block is not trivial

Reset looks like the simplest logic in a chip and is one of the most common sources of silicon bugs, because a reset that is too short, or that resets whatever produced it, fails in a way no functional simulation notices. §7.2 and §7.3 exist specifically to close those two failures.

## 2 Requirements

| ID | Requirement | Source |
| --- | --- | --- |
| R-1 | Divide the external 500 MHz reference to 250 MHz hclk. | ADR-0001 |
| R-2 | Provide 125 MHz pclk for the APB peripherals, 50% duty. | ADR-0002 |
| R-3 | The divide ratio shall be a parameter, so a timing miss is recoverable without redesign. | ADR-0001 |
| R-4 | Reset shall not depend on any analog cell inside the chip. ext_rst_n is the only power-on reset source, driven by a board supervisor that holds it low until the core supply is stable. | ADR-0019 |
| R-5 | Every reset assertion shall last at least 1024 reference cycles at the point of use. | ADR-0003 |
| R-6 | No block shall be able to reset the logic that generates its own reset request. | ADR-0003 |
| R-7 | Reset release shall be synchronous to the clock of the domain being released. | ADR-0003 |
| R-8 | The cause of the last reset shall be readable by firmware after that reset. | ADR-0003 |
| R-9 | A debug-initiated reset shall not reset the Debug Module or the JTAG TAP. | RISC-V Debug 0.13 §3.2 |
| R-10 | 500 MHz shall exist on no net other than the pad-to-divider connection. | ADR-0018 |

## 3 Block diagram

┌──────────── chip boundary ────────────┐
                       │                                        │
  refclk pad ──────────┼──▶ ┌─────────┐                         │
  (500 MHz)            │    │ div2    │  250 MHz                │
     [abuts pad, R-10] │    │ toggle  ├──────────┬──────────────┼──▶ hclk
                       │    └────┬────┘          │              │
                       │         │               ▼              │
                       │         │          ┌─────────┐         │
                       │         │          │ div2    │ 125 MHz │
                       │         │          │ toggle  ├─────────┼──▶ pclk
                       │         │          └─────────┘         │
                       │         │               │              │
                       │         │               └──────────────┼──▶ pclk_phase
                       │         │                              │
  ext_rst_n pad ───────┼───┐     │                              │
                       │   ▼     ▼                              │
                       │ ┌───────────────────────────────┐      │
  wdt_rst_req ─────────┼▶│         reset_ctrl            │      │
  ndm_rst_req ─────────┼▶│  ┌──────────┐  ┌───────────┐  ├──────┼──▶ hreset_n
  (from DM)            │ │  │ stretch  │  │ reason    │  │      │
                       │ │  │ counter  │  │ register  │  ├──────┼──▶ preset_n
                       │ │  │ (aon_clk)│  │ (ext only)│  │      │
                       │ │  └──────────┘  └───────────┘  ├──────┼──▶ core_rst_n
  hartreset_req ───────┼▶│                               │      │
  (from DM)            │ └───────────────────────────────┘      │
                       │                                        │
                       └────────────────────────────────────────┘

Note what the diagram does not show: any path from wdt_rst_req back into the watchdog's own request flop, or from ndm_rst_req into the Debug Module. Their absence is the substance of §7.3.

## 4 Sub-block inventory

| Name | Type | RTL file | Role |
| --- | --- | --- | --- |
| t1_q | seq (500 MHz) | clk_div.v | Divide-by-2 toggle flop on refclk. The only 500 MHz flop in the chip. Its output is aon_clk (D-14) and the DIV=2 source. |
| t2_q, t3_q, t4_q | seq (ripple) | clk_div.v | Further divide-by-2 stages for DIV=4/8/16. |
| sel_req_q, sel_act_q, hclk_mux | seq + comb | clk_div.v | Glitch-free ratio select ([N-7.5]). |
| pclk_q | seq | clk_div.v | Divide-by-2 toggle flop, hclk → pclk. |
| pclk_phase_o | comb | clk_div.v | ~pclk_q: marks the hclk edge that coincides with a pclk edge. |
| rst_sync_in | seq | reset_ctrl.v | Synchronises ext_rst_n deassertion to aon_clk. |
| rst_stretch | seq (aon_clk) | reset_ctrl.v | 1024-cycle down counter (D-14). |
| rst_reason | seq (ext-reset domain only) | reset_ctrl.v | Sticky W1C cause bits. |
| rst_release_hclk | seq | reset_ctrl.v | 2-flop synchronous deassert onto hclk. |
| rst_release_pclk | seq | reset_ctrl.v | 2-flop synchronous deassert onto pclk. |
| apb_regs | seq (pclk) | reset_ctrl_apb.v | APB register interface, window 9. |

## 5 Interfaces

### 5.1 clk_div (block 21)

| Port | Dir | Width | Domain | Reset value | Description |
| --- | --- | --- | --- | --- | --- |
| refclk_i | in | 1 | — | — | 500 MHz external reference. |
| raw_rst_n_i | in | 1 | async | — | Unstretched, unsynchronised ext_rst_n. See §7.4 for why the divider uses the raw signal. |
| div_sel_i | in | 2 | hclk | 2'b00 | 00=DIV2, 01=DIV4, 10=DIV8, 11=DIV16. |
| hclk_o | out | 1 | — | — | 250 MHz at DIV2. |
| pclk_o | out | 1 | — | — | 125 MHz, always hclk/2. |
| pclk_phase_o | out | 1 | hclk | 1'b0 | 1 on the hclk cycle whose rising edge coincides with a pclk rising edge. |
| aon_clk_o | out | 1 | — | — | refclk / 2, always on, independent of DIVSEL (D-14). |
| div_act_o | out | 2 | — | 2'b00 | Ratio in effect (CLKSTAT.DIVACT). |
| div_busy_o | out | 1 | — | — | A ratio change is pending (CLKSTAT.DIVBUSY). |

### 5.2 reset_ctrl (block 22)

| Port | Dir | Width | Domain | Reset value | Description |
| --- | --- | --- | --- | --- | --- |
| aon_clk_i | in | 1 | — | — | Stretch counter, request capture and RSTREASON clock (D-14). See §7.2. |
| hclk_i | in | 1 | — | — | For the hclk release synchroniser. |
| pclk_i | in | 1 | — | — | For the pclk release synchroniser. |
| ext_rst_n_i | in | 1 | async | — | Active-low external reset from the board supervisor. |
| wdt_rst_req_i | in | 1 | hclk | — | Watchdog timeout, one-cycle pulse. |
| ndm_rst_req_i | in | 1 | tck→hclk | — | Debug non-debug-module reset, already synchronised by the DM. |
| hartreset_req_i | in | 1 | tck→hclk | — | Debug core-only reset. |
| hreset_n_o | out | 1 | hclk | 0 | Reset for the AHB fabric, memories, DMA, CLIC, timers. |
| preset_n_o | out | 1 | pclk | 0 | Reset for the APB bridge and peripherals. |
| core_rst_n_o | out | 1 | hclk | 0 | Reset for core + DSU. Asserts with hreset_n_o, and additionally for hartreset. |
| APB slave | — | — | pclk | — | Window 9. See §6. |

preset_n_o and hreset_n_o deassert on different clocks and therefore at different times. The bridge is the only block spanning both, and GARUDA-AHB2APB-SPEC-001 §9 states its requirement: the bridge must hold hreadyout low until both are deasserted.

## 6 Register map — reset_ctrl, APB window 9

(Window number corrected 2026-09-26: was stated as 8, inherited from the 0-based table in GARUDA-AHB2APB-SPEC-001. The hardware decodes window n at 0x4000_0000 + 0x1000 x n (haddr[15:12]), so the base address quoted here was always right and only the index was wrong.)

The register description Design_Docs/regs/crg.rdl is written from this table and generates the UVM register model tb/clk_div/uvm/crg_reg_pkg.sv (tools/gen/gen_ral.py).

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x00 | RSTREASON | R/W1C | see below | Cause of the most recent reset. |
| 0x04 | RSTCTL | RW | 0x0000_0000 | Software reset request and divider select. |
| 0x08 | CLKSTAT | RO | — | Divider status. |

### 6.1 RSTREASON (0x00)

| Bit | Name | Set by |
| --- | --- | --- |
| 0 | EXT | ext_rst_n assertion |
| 1 | WDT | Watchdog timeout |
| 2 | NDM | Debug ndmreset |
| 3 | SW | RSTCTL.SWRST write |
| 4 | BOOTFAIL | Bootloader on CRC mismatch (§8 of the memory spec) |
| 31:5 | reserved | reads 0 |

[N-6.1] RSTREASON bits are reset only by ext_rst_n. They survive watchdog, debug and software resets, because a reason register cleared by the reset it records is useless.

[N-6.2] Exactly one of bits 0–3 is set by any given reset. Bit 4 is set by software and is orthogonal.

[N-6.3] Writing 1 to a bit clears it. Writing 0 has no effect.

### 6.2 RSTCTL (0x04)

| Bit | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0 | SWRST | W | 0 | Writing 1 requests a full reset. Self-clearing. |
| 9:8 | DIVSEL | RW | 2'b00 | Divider select, passed to clk_div. |
| 31:10, 7:1 | reserved | — | 0 | Writes ignored. |

[N-6.4] A DIVSEL write takes effect only at the next pclk_phase boundary, and the divider transitions glitch-free (§7.5). Changing DIVSEL changes hclk and therefore all peripheral baud rates and both timer rates. Firmware is responsible for reconfiguring them.

[N-6.5] DIVSEL is not reset by SWRST, so a software reset does not silently restore 250 MHz after firmware has stepped down to a lower frequency.

### 6.3 CLKSTAT (0x08)

| Bit | Name | Description |
| --- | --- | --- |
| 1:0 | DIVACT | Divide ratio currently in effect. |
| 2 | DIVBUSY | 1 while a ratio change is pending. |
| 31:3 | reserved | reads 0 |

## 7 Functional description

### 7.1 Clock generation

[N-7.1] hclk at DIV=2 is generated by a single toggle flop clocked by refclk. Its output is also aon_clk (D-14):

always @(posedge refclk_i or negedge raw_rst_n_i)
  if (!raw_rst_n_i) t1_q <= 1'b0;
  else              t1_q <= ~t1_q;

t2_q..t4_q are a ripple chain of further toggle flops on t1_q. hclk is a mux of t1_q..t4_q (DIV=2/4/8/16).

[N-7.2] A toggle flop is used rather than a clock gate. A gate that passes every other reference pulse produces a 2 ns high time in an 8 ns period — 25% duty — which halves the usable setup window for every negative-edge-sensitive element and distorts hold margins. A toggle flop gives exactly 50%.

[N-7.3] pclk is a second toggle flop clocked by hclk, giving 125 MHz at 50% duty.

[N-7.4] pclk_phase_o is asserted for the one hclk cycle preceding each pclk rising edge. The bridge uses it to know which hclk edge is safe for transfer between the two clocks. This single bit is why no clock-domain-crossing logic is needed anywhere in the chip between hclk and pclk:

Every pclk rising edge coincides with an hclk rising edge. The set of pclk edges is a strict subset of the set of hclk edges. Both sides of the boundary therefore always sample on a shared edge, with a fixed and known phase relationship. This is a synchronous multi-frequency design, not an asynchronous crossing. Static timing analysis handles it with one create_clock on refclk and create_generated_clock statements for the derived clocks; no synchronisers, no metastability, no CDC waivers.

[N-7.5] Ratio changes are glitch-free: on a DIVSEL write, DIVBUSY sets, the divider output is held at its current level until the counter reaches a state where the new ratio's edge aligns, then the new ratio takes effect. No output pulse is shorter than the shorter of the two ratios' half-periods.

[N-7.6] The three clocks and their relationship (values from GARUDA-SYS-001):

| Clock | Source | Ratio | Frequency at DIV=2 |
| --- | --- | --- | --- |
| refclk | external pad | — | 500 MHz |
| aon_clk | refclk / 2 | fixed 2 | 250 MHz (all DIV values) |
| hclk | refclk / DIV | 2 (parameter) | 250 MHz |
| pclk | hclk / 2 | fixed 2 | 125 MHz |

### 7.2 Reset stretching — the mechanism that makes internal resets work

[N-7.7] All reset requests pass through a 1024-cycle down counter clocked by aon_clk (D-14). 1024 aon_clk cycles are 2048 refclk cycles. A request loads the counter; the reset output stays asserted until the counter reaches zero.

[N-7.8] The stretch counter is clocked by aon_clk, not hclk. This is load-bearing. hclk is not a dependable clock during reset. A stretch counter on hclk would not advance, and reset would never release. aon_clk is the pad-adjacent ÷2 flop: only the raw pin resets it, it runs through every internal reset, and DIVSEL does not change it. It is aon_clk and not refclk because R-10 lets the 500 MHz net reach one flop only (D-14).

[N-7.9] Without the stretch, an internally generated reset is self-cancelling. The watchdog asserts wdt_rst_req; that resets the watchdog; the watchdog stops asserting; the reset deasserts — one flop delay later. A pulse that short does not reliably reset a flop, let alone a chip, and it is invisible in RTL simulation where reset is modelled as instantaneous. The stretch counter makes the request's duration independent of the requester's state.

[N-7.10] A new request arriving while the counter is running reloads it. Resets extend; they do not queue.

### 7.3 Reset domains — no source inside its own domain

[N-7.11] The reset domain of each source:

| Request | Resets | Does not reset | Why |
| --- | --- | --- | --- |
| ext_rst_n | everything | RSTREASON | The reason register must survive to be read. |
| wdt_rst_req | everything except below | the watchdog's request flop, RSTREASON | Otherwise the request cancels itself (§7.9). |
| ndm_rst_req | everything except below | the Debug Module, the JTAG TAP, RSTREASON | RISC-V Debug 0.13 §3.2. Otherwise a debug-initiated reset kills the debug session that initiated it, and dmcontrol.ndmreset clears itself before the debugger can observe the effect. |
| hartreset_req | core + DSU only | fabric, memories, peripherals, DM | Lets the debugger hold the core while the bus stays alive for System Bus Access. |

[N-7.12] The watchdog's request flop is in the ext_rst_n domain. GARUDA-TIMERS-SPEC-001 §9 states this on the watchdog side; both documents must agree, and GARUDA-SYS-001 reset.sources[wdt_rst_n].resets is the authority.

[N-7.13] hartreset does not pass through the stretch counter. It is a level from the Debug Module, held as long as the debugger holds it, and is released synchronously to hclk.

### 7.4 Assertion and release

[N-7.14] Assertion is asynchronous. ext_rst_n reaches every flop's asynchronous reset input without waiting for a clock, because at power-on there is no clock to wait for.

[N-7.15] Release is synchronous, through a two-flop synchroniser in the target domain: one for hclk, one for pclk. This prevents flops in the same domain from leaving reset on different cycles, which is how a reset-release race produces a state machine in an illegal state.

[N-7.16] clk_div's divider flops use raw_rst_n — ext_rst_n directly, unstretched and unsynchronised. They cannot use the stretched output because that output depends on the divider itself, which must be in a known state before hclk exists at all. This is the one deliberate exception to [N-7.15]. The divider is seven flops on four clocks, not one. The exception is safe because each of those flops, and each flop of reset_ctrl that the same pin releases, either is a free-running divider (every state is legal) or has its reset value at its input for the first two cycles after release (Docs/BUGS.md CRG-3).

[N-7.17] Release order is aon_clk domain, then hclk, then pclk. pclk is last because it is the slowest, so a peripheral can never see its bus interface active before its own reset has released.

### 7.5 Reset sequence — timeline

| Phase | Duration | State |
| --- | --- | --- |
| 1 | supply ramp | ext_rst_n low (board supervisor). Nothing is clocked. |
| 2 | ≤10 ref cycles | ext_rst_n released; refclk present; divider leaves raw reset; hclk/pclk begin. |
| 3 | 1024 aon_clk cycles (2048 ref cycles, ≈4.1 µs) | Stretch counter runs. All functional resets held. |
| 4 | 2 hclk cycles | hreset_n and core_rst_n release synchronously. |
| 5 | 2 pclk cycles | preset_n releases. |
| 6 | — | Core fetches from 0x1000_0000. |

[N-7.18] Total reset time from ext_rst_n release to first instruction fetch is approximately 4.1 µs at DIV=2. Nothing in the system depends on this being short.

### 7.6 Floorplan constraint — the 500 MHz net

[N-7.19] The divider flop t1_q (u_clk_div) shall be placed adjacent to the refclk pad. The refclk net shall connect the pad's input buffer to that flop's clock input and to nothing else.

[N-7.20] Rationale, and why this is in a design document rather than left to physical design: 500 MHz is a 2 ns period, and this is a 1.45 mm die with a bond-wire package. A 500 MHz net routed across the die would need its own insertion-delay budget, would couple into neighbours, and would make the reference clock's duty distortion a chip-wide concern. Confining it to a short pad-adjacent connection means 500 MHz exists on roughly 50 µm of metal and every other net in the chip is 250 MHz or slower.

[N-7.21] t1_q is constrained at 500 MHz (2 ns period). Its timing path is a single flop with its output inverted back to its own input — no combinational logic in between — which is the simplest path that exists, and it will close. But it must be told to the timing tool explicitly, or it will be checked against the wrong clock. The constraints are flow/constraints/garuda.sdc (28 nm target, draft) and flow/constraints/asap7/ (exploratory, validated on ASAP7, Docs/DECISIONS.md D-27): refclk on the pin, and aon_clk, the ripple stages, hclk and pclk as generated clocks with explicit edges, one scenario per DIVSEL value. aon_clk, hclk and pclk are one synchronous family with no clock groups between them.

## 8 Timing

### 8.1 Clock relationship

refclk   ─┐ ┌─┐ ┌─┐ ┌─┐ ┌─┐ ┌─┐ ┌─┐ ┌─┐ ┌─┐ ┌─    2 ns
          └─┘ └─┘ └─┘ └─┘ └─┘ └─┘ └─┘ └─┘ └─┘

hclk     ─┐   ┌───┐   ┌───┐   ┌───┐   ┌───┐        4 ns, 50%
          └───┘   └───┘   └───┘   └───┘   └───

pclk     ─┐       ┌───────┐       ┌───────┐        8 ns, 50%
          └───────┘       └───────┘       └───

pclk_    ─────┐   ┌───┐       ┌───┐       ┌───     1 hclk cycle before
phase         └───┘   └───────┘   └───────┘        each pclk rising edge
              
              ▲       ▲       ▲       ▲
              └───────┴───────┴───────┴── every pclk edge lands on an hclk
                                          edge: this is why no CDC exists

### 8.2 Reset assertion and release

┌── supply stable
              │
ext_rst_n  ───┘‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾
           ___________
refclk     ___________┌┐┌┐┌┐┌┐┌┐┌┐┌┐┌┐┌┐┌┐┌┐┌┐┌┐┌┐┌┐┌┐┌┐
                      └┘└┘└┘└┘└┘└┘└┘└┘└┘└┘└┘└┘└┘└┘└┘└┘└┘

hclk       _______________┌──┐  ┌──┐  ┌──┐  ┌──┐  ┌──┐
                          └──┘  └──┘  └──┘  └──┘  └──┘
                          ▲
                          └── divider leaves raw reset

stretch    ──────────────< 1024 >< 1023 >< ... >< 1 >< 0 >
counter

hreset_n   ‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾┌────
           (asserted through the whole stretch)      └── 2 hclk after zero

preset_n   ‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾┌──
                                                        └── 2 pclk after

### 8.3 Watchdog reset — the case the stretch exists for

wdt_rst_req   ────────┐
(from timers) ________│‾│___________________________________
                        └─ one hclk pulse, then the watchdog
                           is itself reset and stops asserting

stretch       ─────────< 1024 >< 1023 >< ... >< 1 >< 0 >
counter                 ▲
                        └── loaded by the pulse; runs on refclk,
                            independent of the watchdog's state

hreset_n      ‾‾‾‾‾‾‾‾‾‾╲______________________________┌────
                                                       └── full 1024 cycles

RSTREASON.WDT ─────────────┌────────────────────────────────
              _____________┘  survives the reset it records

Without the counter, hreset_n would be a pulse the width of wdt_rst_req — one hclk cycle, and in silicon quite possibly less once the self-cancelling path settles.

## 9 Clock, reset and power

### 9.1 Domains

| Domain | Clock | Contents |
| --- | --- | --- |
| refclk | 500 MHz | The divider flop t1_q only. |
| aon_clk | 250 MHz | The stretch counter, request capture, RSTREASON (D-14). |
| hclk | 250 MHz | Core, DSU, AHB fabric, memories, DMA, CLIC, timers, DM. |
| pclk | 125 MHz | APB bridge peripheral side, all eight peripherals, reset_ctrl's APB registers. |
| tck | ≤20 MHz, async | JTAG TAP and DTM only. |

[N-9.1] hclk and pclk are synchronous to each other (§7.4). The only genuinely asynchronous boundary in the chip is tck ↔ hclk, inside the Debug Module.

### 9.2 Power

[N-9.2] clk_div provides no clock gating. The per-block gates (core WFI, DSU idle, DMA per-channel, APB per-window) are defined in their own specs and gate these root clocks locally.

[N-9.3] pclk at half hclk is itself a power measure: it halves the dynamic power of the entire peripheral clock tree and of every peripheral flop, which is the majority of the chip's flop count outside the core. This is the reason pclk exists as a real divided clock rather than an access-rate trick — an enable scheme would deliver the same interface rate while leaving every peripheral flop toggling at 250 MHz, saving nothing.

[N-9.4] No power domains, no isolation cells, no level shifters, no retention flops, no voltage scaling. DIVSEL provides frequency scaling as a coarse, firmware-driven fallback and is not a dynamic power-management mechanism.

## 10 Assertions

// --- reset duration: the property the stretch counter exists to guarantee
property p_reset_min_duration;
  @(posedge aon_clk_i) $fell(hreset_n_o) |-> ##[1:$] (!hreset_n_o [*1023]);
endproperty
a_reset_min_duration: assert property (p_reset_min_duration);

// --- no reset source resets its own requester (R-6)
a_wdt_req_survives_reset: assert property (
  @(posedge hclk_i) $rose(wdt_rst_req_i) |-> ##1 !$isunknown(wdt_rst_req_i));

// --- reason register survives every reset except external
a_reason_survives_wdt: assert property (
  @(posedge aon_clk_i) disable iff (!ext_rst_n_i)
  $fell(hreset_n_o) && rst_reason_q[1] |=> rst_reason_q[1]);

// --- exactly one cause bit set per reset event
a_reason_onehot: assert property (
  @(posedge pclk_i) disable iff (!preset_n_o)
  $onehot0(rst_reason_q[3:0]));

// --- synchronous release: reset deasserts only on a clock edge
a_sync_release_hclk: assert property (
  @(posedge hclk_i) $rose(hreset_n_o) |-> $rose(hreset_n_o));

// --- pclk edges are a subset of hclk edges (the no-CDC argument, checked)
a_pclk_subset_hclk: assert property (
  @(posedge pclk_o) $rose(hclk_o) || hclk_o);

// --- pclk_phase leads each pclk rising edge by exactly one hclk cycle
a_pclk_phase: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_o)
  pclk_phase_o |=> $rose(pclk_o));

// --- pclk is always exactly half of hclk
a_pclk_ratio: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_o)
  $rose(pclk_o) |=> ##1 $fell(pclk_o));

// --- glitch-free ratio change: no short pulse during DIVBUSY
a_div_no_glitch: assert property (
  @(posedge refclk_i) disable iff (!ext_rst_n_i)
  div_busy |-> ##[0:3] !$fell(hclk_o) or (hclk_low_count >= 2));

// --- preset_n never deasserts before hreset_n (release order, N-7.17)
a_release_order: assert property (
  @(posedge pclk_i) $rose(preset_n_o) |-> hreset_n_o);

// --- hartreset resets the core without touching the fabric
a_hartreset_scope: assert property (
  @(posedge hclk_i) hartreset_req_i |-> !core_rst_n_o && hreset_n_o);

## 11 Verification plan

| Req | Test | Oracle | Coverage goal | Status |
| --- | --- | --- | --- | --- |
| R-1 | t_clk_div2 | hclk period = 2× refclk, duty 50% ±2% | — | new |
| R-2 | t_pclk_div2 | pclk period = 2× hclk, duty 50% ±2% | — | new |
| R-3 | t_div_sel | each DIVSEL value gives the right ratio | all 4 values | new |
| R-3 | t_div_switch_glitch | a_div_no_glitch holds across all 12 ratio transitions | all ordered pairs | new |
| R-4 | t_no_por | chip resets correctly with ext_rst_n as the only source | — | new |
| R-5 | t_reset_stretch | every reset ≥1024 aon_clk cycles (2048 refclk) | all 4 sources | new |
| R-5 | t_wdt_self_cancel | watchdog single-cycle pulse → full-length reset | — | new |
| R-6 | t_reset_domains | each source's excluded blocks are not reset | all 4 sources | new |
| R-7 | t_sync_release | no flop in a domain leaves reset on a different cycle | — | new |
| R-8 | t_rst_reason | correct bit after each reset type; W1C works | all 5 bits | new |
| R-9 | t_ndmreset_dm_alive | DM and TAP respond during and after ndmreset | — | new |
| R-10 | — | not simulation-verifiable; checked at floorplan review | — | PD gate |
| §7.4 | t_no_cdc_proof | a_pclk_subset_hclk + a_pclk_phase over 10⁶ cycles | — | new |
| §7.13 | t_hartreset | core resets, fabric stays alive, SBA still works | — | new |
| §8.1 | t_reset_release_order | aon_clk → hclk → pclk ordering | — | new |

Formal recommended for: a_reset_min_duration, a_pclk_subset_hclk, a_pclk_phase. These are small, bounded properties on a handful of flops, and they are the properties the rest of the chip's no-CDC argument rests on. Proving them is cheap and worth more than any number of directed tests.

## 12 Design decisions

| Decision | ADR |
| --- | --- |
| 500 MHz reference, DIV=2 → 250 MHz, no PLL, DIV parameterised with DIV=4 fallback | ADR-0001 |
| Real 125 MHz pclk from a toggle flop, not an ICG, not an access-enable scheme | ADR-0002 |
| 1024-cycle stretch on aon_clk; no source inside its own reset domain | ADR-0003, D-14 |
| Divider flop abuts the pad; 500 MHz confined to that net | ADR-0018 |
| No analog POR; board supervisor drives ext_rst_n | ADR-0019 |

## 13 Not implemented

| Feature | Why not |
| --- | --- |
| PLL | The reference already exceeds the target. A PLL is a mixed-signal IP dependency with lock, jitter and bring-up risk for no gain. |
| Analog POR cell | Cannot be assumed present in the PDK, and depending on an absent cell is an unrecoverable hole. A board supervisor is a few rupees and strictly simpler. |
| Non-power-of-2 division | 500→200 MHz needs 2.5×, which needs either a PLL or a non-50%-duty divider. 250 MHz avoids the question. |
| Independent low-power oscillator | A separate always-on oscillator for a sleep domain, for a chip with no sleep domain. |
| Clock gating in clk_div | Gating belongs at the consumer, where the idle condition is known. |
| Glitch-free clock muxing | There is one clock source. Nothing to mux. |
| Reset-release debouncing on ext_rst_n | The board supervisor provides a clean edge; that is the requirement in R-4. |

## 14 Open items

None. Previous opens closed as follows:

| Was open | Closed as | ADR |
| --- | --- | --- |
| Is an analog POR available in the PDK? | Assume not. ext_rst_n mandatory. | ADR-0019 |
| Pad type and board SI for 500 MHz | Confine 500 MHz to a pad-adjacent net; standard pad. | ADR-0018 |
| Constraints for the 500 MHz divider flop | Stated in §7.21 with the SDC sketch. | ADR-0001 |

## 15 Errata

None. Neither block has RTL yet, which is precisely why the Rev 1.1 reset defects cost nothing to fix: they were caught while the document was still the only artefact.

Defects fixed in this revision, recorded so they are not reintroduced:

| ID | Symptom | Root cause | Fix |
| --- | --- | --- | --- |
| CR-1 | Watchdog reset would be a runt pulse of one flop delay. | All four sources ORed into one signal that reset the requesters. | Stretch counter, §7.2. |
| CR-2 | Debug reset would kill the debug session and clear dmcontrol.ndmreset before the debugger could observe it. | DM was inside the ndm_rst_n domain. | Domain table, §7.3. |
| CR-3 | Reset cause would be destroyed by the reset it recorded. | RSTREASON was in the general reset domain. | RSTREASON reset by ext_rst_n only. |
| CR-4 | A stretch counter on hclk would never advance. | hclk is stopped while reset is asserted. | Counter on aon_clk, [N-7.8], D-14. |
| CR-5 | pclk specified as 50% duty but generated by an ICG, which gives 25%. | Mechanism and waveform were specified independently. | Toggle flop, [N-7.2]. |