# GARUDA GPIO — Design Specification

## 0.1 Identity

| Field | Value |
| --- | --- |
| Document ID | GARUDA-GPIO-SPEC-001 |
| Revision | 1.0 |
| Date | 2026-09-23 |
| Status | Released for implementation |
| Block | 19 (gpio) |
| Owner | Team AeroSoC |
| Supersedes | — (first revision) |

## 0.2 Revision history

| Rev | Change | Driver |
| --- | --- | --- |
| 1.0 | First specification. pulp-platform/apb_gpio behind a GARUDA wrapper. | D-21, D-22, D-23 |

## 0.3 Normative references

GARUDA-SYS-001 Rev 4.0 · GARUDA-AHB2APB-SPEC-001 Rev 2.0 · GARUDA-CLIC-SPEC-001 Rev 2.0 §7.2 · Docs/DECISIONS.md D-14, D-17, D-21, D-22, D-23.

## 1 Purpose and scope

### 1.1 In scope

Two general-purpose bidirectional pins, gpio0 and gpio1, on APB window 7 (0x4000_7000), CLIC ID 20. Direction, output value, input read, and a per-pin edge/level interrupt.

### 1.2 Out of scope

- DMA. GPIO has no DMA channel in the Rev 4.0 map, and nothing to stream.

- Pad drive strength, slew and pull configuration. The IP has PADCFG registers for it, but GARUDA has no pad cells yet ([N-13.1]).

- Pin multiplexing. These two pins are GPIO and nothing else.

### 1.3 Where the RTL comes from

rtl/third_party/pulp/apb_gpio (Solderpad 0.51), unmodified, behind rtl/gpio/garuda_gpio_top.v. It is the closest thing to a drop-in of the four vendored IPs: already word-addressed (s_apb_addr = PADDR[6:2]), PREADY tied high, PSLVERR tied low, and it carries its own input synchronisers.

## 2 Requirements

| ID | Requirement | Source | Status |
| --- | --- | --- | --- |
| R-1 | Drive either pin high or low under firmware control. | SYS §6.4 | met |
| R-2 | Read the level on either pin, whether driven or not. | SYS §6.4 | met |
| R-3 | Per-pin direction control, independently. | SYS §6.4 | met |
| R-4 | Word-only APB, 32-bit registers at word-aligned offsets. | AHB2APB [N-7.12] | met |
| R-5 | Never stall the APB bus. | AHB2APB [N-7.15] | met |
| R-6 | One held level interrupt to CLIC ID 20, from a per-pin edge or level condition. | CLIC §7.2, D-17 | met ([N-7.3]) |
| R-7 | Pad inputs synchronised before use. | D-14 | met |
| R-8 | Both pins released (inputs) from reset until firmware acts. | Board safety | met |

## 3 Block diagram

APB window 7 (pclk 125 MHz)
        │
        ▼
  ┌──────────────── garuda_gpio_top ─────────────────┐
  │  garuda_apb_shim ── IP APB ──▶ apb_gpio (vendored)│
  │   0x00-0x7C pass-through   PADDIR GPIOEN PADIN    │
  │   0xFE0-0xFEC tail         PADOUT SET CLR INTEN   │
  │                            INTTYPE INTSTATUS      │
  │   interrupt (read-to-clear) ──▶ sticky IRQSTAT[0] │
  │   gpio_in ──▶ 2-flop sync                         │
  └───────────── gpio_i / gpio_o / gpio_oe ───────────┘

## 4 Sub-block inventory

| Name | Type | RTL file | Role |
| --- | --- | --- | --- |
| garuda_apb_shim | seq (pclk) | rtl/common/garuda_apb_shim.v | decode, PSLVERR, sticky IRQ, pad sync |
| apb_gpio | seq (pclk) | rtl/third_party/pulp/apb_gpio/src/ | register file, edge detect, interrupt (vendored) |
| garuda_gpio_top | wrapper | rtl/gpio/garuda_gpio_top.v | 2-pin mapping, direction polarity, sticky IRQ |

## 5 Interfaces

| Port | Dir | Width | Domain | Reset | Description |
| --- | --- | --- | --- | --- | --- |
| pclk_i, preset_n_i | in | 1 | pclk | — | 125 MHz |
| APB slave | — | — | pclk | — | window 7 |
| irq_o | out | 1 | pclk | 0 | CLIC ID 20, level |
| gpio_i | in | 2 | async | — | pad input, synchronised |
| gpio_o | out | 2 | pclk | 0 | pad output value |
| gpio_oe | out | 2 | pclk | 0 | 1 = drive the pad, 0 = input |

## 6 Register map — APB window 7 (0x4000_7000)

The register description Design_Docs/regs/gpio.rdl is written from this table and generates the UVM register model tb/gpio/uvm/gpio_reg_pkg.sv (tools/gen/gen_ral.py). The vendored register file is 32 pins wide; GARUDA has two, so only bits [1:0] of each register are meaningful and the rest read 0 ([N-6.2]).

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x00 | PADDIR | RW | 0 | [n] 1 = output, 0 = input |
| 0x04 | GPIOEN | RW | 0 | [n] 1 = enable the pin's input path and interrupt ([N-6.1]) |
| 0x08 | PADIN | RO | — | [n] the level on the pin |
| 0x0C | PADOUT | RW | 0 | [n] the value driven when PADDIR[n] is 1 |
| 0x10 | PADOUTSET | W | — | [n] 1 sets PADOUT[n] ([N-6.3]) |
| 0x14 | PADOUTCLR | W | — | [n] 1 clears PADOUT[n] ([N-6.3]) |
| 0x18 | INTEN | RW | 0 | [n] 1 = this pin may raise an interrupt |
| 0x1C | INTTYPE | RW | 0 | 2 bits per pin: 00 falling, 01 rising, 10 either, 11 level |
| 0x24 | INTSTATUS | R, clears | 0 | [n] 1 = this pin caused the interrupt ([N-7.3]) |
| 0x28 | PADCFG0 | RW | 0 | pad drive/pull — no effect ([N-13.1]) |
| 0xFE0 | IRQSTAT | W1C | 0 | [0] a GPIO interrupt occurred |
| 0xFE4 | IRQEN | RW | 0 | mask; irq_o = IRQSTAT[0] & IRQEN[0] |
| 0xFE8 | DMACTL | RW | 0 | present for uniformity; no DMA channel ([N-13.2]) |
| 0xFEC | ID | RO | — | {16'h6A5D, 8'd19, rev} |

[N-6.1] GPIOEN[n] gates the input path. A pin used only as an output does not need it; a pin read or used as an interrupt source does, and forgetting it is the usual reason a GPIO interrupt never arrives.

[N-6.2] Bits [31:2] of every register exist in the vendored RTL and are writable, but no pin is attached. Firmware must ignore them. t_gpio_width asserts that reads return 0 there.

[N-6.3] PADOUTSET and PADOUTCLR exist so two contexts can own one pin each without a read-modify-write race on PADOUT. On a chip with an interrupt handler toggling one pin and a control loop toggling the other, use them.

## 7 Functional description

### 7.1 Driving a pin

[N-7.1]

PADDIR    |= (1 << n)        make pin n an output
PADOUTSET  = (1 << n)        drive it high
PADOUTCLR  = (1 << n)        drive it low

### 7.2 Reading a pin

[N-7.2]

PADDIR &= ~(1 << n)          make it an input (this is also the reset state)
GPIOEN |=  (1 << n)          enable its input path  <- easy to forget
level = (PADIN >> n) & 1

PADIN reflects the pin, not PADOUT, so a pin configured as an output can be read back to confirm it is actually at the level driven — which is how a short to ground is detected.

### 7.3 Interrupts, and the one wrinkle

[N-7.3] The vendored block's interrupt output is a one-cycle pulse, not a level: assign interrupt = |s_is_int_all, and s_is_int_all is a combinational edge detect gated by INTEN/GPIOEN/INTTYPE. It is therefore exactly the SPI master's situation ([SPIM N-7.7]) and the shim's sticky IRQSTAT[0] is what turns it into the held level D-17 requires. The tail is not a formality here; without it a GPIO interrupt would be one pclk wide and the CLIC would miss it.

INTSTATUS is a separate, per-pin sticky record (r_status), and reading it clears it. So the two registers answer two different questions:

| Register | Says | Cleared by |
| --- | --- | --- |
| IRQSTAT[0] | "a GPIO interrupt happened" — the line the CLIC sees | W1C |
| INTSTATUS[n] | "it was pin n" | reading it |

A handler needs both:

cause = INTSTATUS        which pin - and clears the record for the next one
...handle it...
IRQSTAT = 1              W1C: drops the line to the CLIC

Either order works, because they are independent. Read INTSTATUS even if only one pin is enabled: leaving it set makes the next interrupt's cause ambiguous, since the bits accumulate with r_status | s_is_int_all.

[N-7.3a] For the same reason the wrapper must never poll INTSTATUS. The SPI and UART wrappers read a status register in idle cycles to recover FIFO state ([SPIM N-7.4]); doing that here would silently eat every interrupt. GPIO needs no such polling — it has no FIFO and no DMA — and the wrapper does none.

## 8 Timing

Combinational from PADOUT/PADDIR to the pins; input path is the shim's two flops plus the IP's own synchroniser. Nothing here is timing-critical.

## 9 Clock, reset and power

[N-9.1] Single domain, pclk, reset preset_n.

[N-9.2] gpio_oe is 0 out of reset — both pins are inputs until firmware says otherwise (R-8). On a flight controller a GPIO that powers up driving is a pin fighting whatever is on the other side of it.

[N-9.3] The IP's dft_cg_enable_i is tied low. It bypasses an internal clock gate in test mode; DFT insertion must revisit it ([N-14.1]).

## 10 Assertions

a_pready:   assert property (@(posedge pclk_i) disable iff (!preset_n_i)
              psel_i |-> pready_o);
a_oe_reset: assert property (@(posedge pclk_i) !preset_n_i |-> (gpio_oe_o == 2'b00));
a_irq_held: assert property (@(posedge pclk_i) disable iff (!preset_n_i)
              $fell(irq_o) |-> $past(irqstat_write));

## 11 Verification plan

make test_gpio — tb/gpio/tb_gpio.sv, 25 checks, 0 failures.

| Req | Test | Oracle | Status |
| --- | --- | --- | --- |
| R-1 | t_gpio_drive | each pin drives high and low, independently | pass |
| R-2 | t_gpio_read | PADIN follows an externally driven pin | pass |
| R-2 | t_gpio_readback | an output pin reads back the level it drives | pass |
| R-3 | t_gpio_dir | one pin output, one input, at the same time | pass |
| R-4 | t_gpio_regs | registers read back; PSLVERR on unmapped; [31:2] read 0 | pass |
| R-5 | t_gpio_pready | PREADY high in every cycle of every access | pass |
| R-6 | t_gpio_irq | rising-edge interrupt; held across 60 pclk though the IP pulses for 1; W1C drops it | pass |
| R-6 | t_gpio_irq_cause | INTSTATUS names the pin and self-clears on read | pass |
| R-6 | t_gpio_irq_masked | an edge on a pin with INTEN clear raises nothing | pass |
| R-6 | t_gpio_irq_type | falling, rising and either-edge each fire correctly | pass |
| R-7 | t_gpio_sync | an input change is seen, with synchroniser latency | pass |
| R-8 | t_gpio_reset | both gpio_oe are 0 out of reset | pass |
| — | t_chip_gpio | both pins from the core, through the real fabric | new |

## 12 Design decisions

| # | Decision | Alternative rejected |
| --- | --- | --- |
| 1 | PAD_NUM = 2, matching the pin count | the default 32 — ~150 flops of registers with no pins behind them |
| 2 | Sticky tail over the IP's read-to-clear interrupt | exposing the read-to-clear line to the CLIC, against D-17 |
| 3 | No idle-cycle polling in this wrapper | consistency with SPI/UART — it would eat every interrupt ([N-7.3a]) |

## 13 Not implemented

[N-13.1] PADCFG (drive strength, pull-up/down, slew) is writable and has no effect: GARUDA has no pad cells in RTL, so there is nothing to configure. When the pad ring lands, these bits become real and this note must go.

[N-13.2] DMACTL exists so every peripheral window has the same tail, but GPIO owns no DMA channel in the Rev 4.0 map. Writing it does nothing.

## 14 Open items

- OPEN-G1 ([N-9.3]) — dft_cg_enable_i is tied low. Scan insertion must drive it from the test-mode signal, which does not exist yet. Part of the DFT gap already recorded in Docs/HANDOFF.md.

## 15 Errata

None found in the vendored RTL.