# GARUDA Pin-out and Physical Constraints — Specification

## 0.1 Identity

| Field | Value |
| --- | --- |
| Document ID | GARUDA-PHYS-SPEC-001 |
| Revision | 1.0 |
| Date | 2026-09-18 |
| Status | Released for the GDSII handoff |
| Owner | Team AeroSoC |
| Audience | Physical design (1-TOPS mentor), board design |
| Supersedes | nothing — this document did not previously exist |

## 0.2 Why this document exists

No document in this project owned the physical constraints. The pin table lived in the TRM with GPIO defined as "the remaining budget", which is not a specification — it meant nobody could check the count, and the count turned out to be the one decision in the whole project that cannot be undone after the pad ring is fixed.

Four constraints in this document are not derivable from any block spec: the signal-pin budget, the pad-adjacent placement of the 500 MHz divider, the SDC for the reference clock, and the supply-pad distribution. A physical design engineer reading the eleven block specs would not find them. They are collected here so the GDSII handoff is one package.

## 0.3 Normative references

- GARUDA-SYS-001 Rev 4.0 — the pin allocation table is generated from pins.

- GARUDA-ADR-001 Rev 1.0 — ADR-0001, 0013, 0018, 0019, 0020.

- GARUDA-CLKRST-SPEC-001 Rev 2.0 §7.6 — the clock constraint this document expands.

## 1 Die and package

| Parameter | Value | Source |
| --- | --- | --- |
| Process | 28 nm CMOS | programme |
| Die edge | 1.45 mm | programme |
| Die area | 2.10 mm² | derived |
| Pad ring perimeter | 5.80 mm | derived |
| Total pads | 40 | programme |
| Signal pads | 28 | ADR-0020 |
| Supply pads | 12 (assumed) | ADR-0020 |

[N-1.1] At a 70 µm pad pitch, 5.80 mm of perimeter accommodates approximately 82 pads, so 40 pads is not perimeter-limited. The 40-pad figure is a programme constraint, presumably packaging, and the die has room to spare.

## 2 The signal-pin budget and why it is 28

[N-2.1] This project assumes the 40 pads include supply pads, leaving 28 for signals.

[N-2.2] The reasoning, stated because this is the one irreversible decision here. A chip switching 30,000-odd flops at 250 MHz in 28 nm needs supply pads both for average current and, more importantly, for distribution: a pad ring with too few supply connections, or with them clustered, produces local supply droop and ground bounce that no amount of firmware work can fix, and that does not appear in any pre-silicon simulation this project runs. Eight to twelve pads, spread around the ring, is the conventional figure for a die of this class.

[N-2.3] The asymmetry that decides it: if the 40 pads turn out to be signal-only, adding GPIO and the SPI slave back is a top-level wiring change and a pad-ring edit, both cheap and both before tape-out. If they are not, and the design assumed 40 signals, the discovery comes when the pad ring is being closed and there is no recovery. Designing for 28 and expanding is safe; the reverse is not.

[N-2.4] Action for the PD mentor: confirm against the programme's pad frame whether supply pads are counted in the 40. If they are not, the 36-pin expansion of §3.2 applies and it is additive only.

## 3 Pin allocation

### 3.1 The 28-pin configuration (baseline)

Generated from GARUDA-SYS-001 pins.allocation:

| # | Name | Dir | Type | Function | Block |
| --- | --- | --- | --- | --- | --- |
| 1 | refclk | in | fast/differential-capable | 500 MHz reference | 21 |
| 2 | ext_rst_n | in | CMOS, Schmitt | External reset from the board supervisor | 22 |
| 3 | tck | in | CMOS, Schmitt | JTAG clock, ≤20 MHz | 12 |
| 4 | tms | in | CMOS, pull-up | JTAG mode select | 12 |
| 5 | tdi | in | CMOS, pull-up | JTAG data in | 12 |
| 6 | tdo | out | CMOS, tri-state | JTAG data out | 12 |
| 7 | spim_sclk | out | CMOS | SPI master clock | 13 |
| 8 | spim_mosi | out | CMOS | SPI master out | 13 |
| 9 | spim_miso | in | CMOS | SPI master in | 13 |
| 10 | spim_cs_flash_n | out | CMOS, pull-up | Boot flash chip select | 13 |
| 11 | spim_cs_imu_n | out | CMOS, pull-up | IMU chip select | 13 |
| 12 | i2c_scl | bidir | open-drain | I²C clock | 15 |
| 13 | i2c_sda | bidir | open-drain | I²C data | 15 |
| 14 | uart0_rx | in | CMOS | GPS receive | 16 |
| 15 | uart0_tx | out | CMOS | GPS transmit | 16 |
| 16 | uart1_rx | in | CMOS | Ground-link receive | 17 |
| 17 | uart1_tx | out | CMOS | Ground-link transmit | 17 |
| 18 | uart2_rx | in | CMOS | Console receive | 18 |
| 19 | uart2_tx | out | CMOS | Console transmit | 18 |
| 20 | pwm0 | out | CMOS | ESC output 0 | 20 |
| 21 | pwm1 | out | CMOS | ESC output 1 | 20 |
| 22 | pwm2 | out | CMOS | ESC output 2 | 20 |
| 23 | pwm3 | out | CMOS | ESC output 3 | 20 |
| 24 | gpio0 | bidir | CMOS | Discrete IO | 19 |
| 25 | gpio1 | bidir | CMOS | Discrete IO | 19 |
| 26 | boot_sel | in | CMOS, pull-down | 0 = flash boot, 1 = JTAG recovery | 4 |
| — | spare ×2 | — | — | Reserved | — |

Total: 26 allocated, 2 spare, 28 budget.

[N-3.1] boot_sel has a pull-down so an unconnected pin boots from flash. A floating boot_sel that read high would leave every board spinning in the recovery loop, looking dead.

[N-3.2] tms and tdi have pull-ups per IEEE 1149.1, so an unconnected JTAG header cannot hold the TAP in an active state.

[N-3.3] i2c_scl/i2c_sda must be open-drain pads with no internal pull-up. I²C bus pull-ups are sized at board level against the bus capacitance; an internal pull-up of unknown value fights that.

[N-3.4] The four PWM pins are dedicated, not GPIO-multiplexed. An ESC output that a GPIO register write can steal is a flight-safety hazard, and four pins is affordable even in the 28-pin budget. This is the one place in the pin budget where safety beat economy.

[N-3.5] Two chip selects on the SPI master, because the boot flash and the IMU share the bus. No previous document accounted for the second one.

### 3.2 The 36-pin expansion (contingent)

[N-3.6] ESP-NOW is required (ADR-0020 Rev 2 — it is APF's neighbour-position source), so the SPI-slave port below is in scope, not contingent. How it is added depends on OPEN-2: if supply pads are outside the 40, these pins are additive and nothing in §3.1 moves; if they are inside the 40, 4 pins are reclaimed (baseline: 2 spares + console UART2 — see ADR-0020 Rev 2). OPEN-2 remains the PD mentor's call.

| # | Name | Dir | Function | Block |
| --- | --- | --- | --- | --- |
| 29–32 | spis_sclk, spis_mosi, spis_miso, spis_cs_n | mixed | ESP32 companion / ESP-NOW mesh | 14 |
| 33–38 | gpio2–gpio7 | bidir | Discrete IO | 19 |

[N-3.7] The design already supports this. Block 14 (rtl/spi_slave/) is built and is on window 0 (0x4000_0000), CLIC ID 14 and DMA channel 4. garuda_chip_top has its four pins, selected by the parameter WITH_SPI_SLAVE (default 1). WITH_SPI_SLAVE = 0 gives the exact 28-pin build. The expansion is therefore a pad-ring edit, not a redesign. This is what "closed in the recoverable direction" meant in ADR-0020.

## 4 Supply pads

[N-4.1] Twelve pads assumed: 6 VDD (1.0 V core) and 6 VSS, distributed around the ring with no more than about 1.2 mm of perimeter between adjacent supply pairs.

[N-4.2] Distribution matters more than count. Supply pads clustered on one edge leave the opposite corners of the die fed through the full width of the power grid, and at 250 MHz the resulting IR drop and ground bounce is a functional failure, not a margin question.

[N-4.3] If the pad library provides separate IO supplies (VDDIO/VSSIO), the split between core and IO supplies is the PD engineer's call against the pad library's requirements. This document does not constrain it beyond [N-4.1]'s distribution requirement.

[N-4.4] The refclk pad's supply should be on the quietest available segment, and ideally not shared with the PWM outputs, which switch hard into inductive loads. Jitter injected onto the reference propagates to every clock in the chip.

## 5 The 500 MHz reference clock — the critical physical constraint

[N-5.1] This section is the reason this document exists as more than a pin table.

### 5.1 Placement

[N-5.2] t1_q (u_clk_div) — the single divide-by-2 toggle flop on refclk — shall be placed abutting the refclk pad. The refclk net shall connect the pad's input buffer to that flop's clock input, and to nothing else.

[N-5.3] Target net length: under 100 µm. Nothing else may be placed on this net, and it should not be routed on a layer or in a channel shared with long parallel neighbours.

[N-5.4] Rationale. 500 MHz is a 2 ns period on a 1.45 mm die in a bond-wire package. A 500 MHz net routed across the die would need its own insertion-delay budget inside the clock tree, would couple into every neighbour it runs alongside, and would make the reference's duty distortion a chip-wide concern rather than a local one. Confining it to the pad's immediate neighbourhood means 500 MHz exists on roughly 50–100 µm of metal and every other net in the chip is 250 MHz or slower. This converts a chip-wide risk into a local, inspectable one.

### 5.2 Timing constraints

[N-5.5] The constraints are flow/constraints/garuda.sdc (TSMC 28 nm target, draft rev 2) and flow/constraints/asap7/ (exploratory, generated by gen_modes.py). The ASAP7 set was validated on a netlist and taken through place and route (Docs/PD_LOG_2026-10-13.md, Docs/PD_LOG_2026-10-14.md). The rules that follow are normative. The values in the files that are marked PROVISIONAL are not (Docs/DECISIONS.md D-27).

| Item | Rule |
| --- | --- |
| refclk | create_clock on the pin refclk, 2.0 ns, input transition 0.10 ns, setup uncertainty 0.15 ns. It reaches u_clk_div.t1_q only ([N-5.2]). |
| aon_clk | Generated clock on t1_q, ÷2 of refclk, always on (D-14). |
| hclk | Generated clock on the hclk_mux output. DIVSEL selects t1_q..t4_q, so there are four scenarios (DIV=2 primary; DIV=4, 8, 16). Each is fixed by case analysis on sel_act_q. |
| pclk | Generated clock on pclk_q, ÷2 of hclk. |
| Edges | Every derived clock is given its edges explicitly, counted in refclk edges (-edges). A plain -divide_by on an inverted-output pin put the clocks half a period out of phase and made hold optimistic (Docs/BUGS.md PD-6). |
| tck | create_clock, 50 ns (≤20 MHz). The paths between tck and the hclk family are bounded with set_max_delay of one hclk period and a hold false path; there is no clock group. |
| ext_rst_n | False path to its asynchronous reset/set pins only. |

[N-5.6] No set_clock_groups is declared between refclk, aon_clk, hclk and pclk. They come from one source with known edges, so the timing tool times every path between them.

[N-5.7] hclk and pclk must not be declared asynchronous or exclusive of each other. Every path between them is real, synchronous and must be timed. This is the whole basis of the chip's no-CDC architecture, and an incorrect set_clock_groups here would silently waive the paths the design depends on. It is called out because it is the single most likely constraint mistake in this design, and its symptom is silicon that fails intermittently at the bridge.

### 5.3 What to check after synthesis

[N-5.8] Three checks specific to this design, beyond normal closure:

| Check | Why |
| --- | --- |
| refclk fanout is exactly 1 | Confirms [N-5.2]. Any other load means 500 MHz escaped the pad. |
| hclk/pclk paths are timed, not waived | Confirms [N-5.7]. Look for unexpected set_clock_groups or false paths in the report. |
| WFI clock gate produces no runt pulse at gate level | The ICG's enable timing is invisible in RTL simulation (GARUDA-CORE-SPEC-001 [N-11.4]). |

## 6 Floorplan guidance

[N-6.1] Not constraints, except where marked. The PD engineer owns the floorplan; these are the adjacencies the design's timing depends on.

| Element | Placement | Binding? |
| --- | --- | --- |
| u_clk_div.t1_q | abutting the refclk pad | yes, §5.1 |
| ISRAM (64 KiB) | near the core's IF and the D-port LSU | no |
| DSRAM (4 × 16 KiB) | near the LSU; the four macros may be split | no |
| Boot ROM (4 KiB) | anywhere; accessed once per power-on | no |
| Core + DSU | one region; the EX critical path must not cross a macro | guidance, important |
| AHB interconnect | central to the four slaves | no |
| APB peripherals + bridge | perimeter, near their pads | no |
| JTAG TAP + DTM | near the JTAG pads | no |
| PWM | near the PWM pads, away from refclk | guidance, §4.4 |

[N-6.2] The core and DSU should be a contiguous region. The critical path is the 33×33 multiplier in series with the EX result multiplexer with the DSU in the same cone (GARUDA-CORE-SPEC-001 §7.9); routing that cone around a memory macro is how 4 ns becomes unachievable.

[N-6.3] Memory area estimate, for floorplanning: 132 KiB of SRAM in 28 nm is roughly 0.25–0.40 mm² depending on the compiler's density option, against 2.10 mm² of die. Memory is 12–19% of the die and the rest is logic and routing, so the floorplan is not memory-dominated.

## 7 Timing closure and the fallback

[N-7.1] Target 250 MHz. Closure is not asserted by any document in this project.

[N-7.2] If closure fails, the recovery is DIVSEL = 1 in clk_div — DIV=4, hclk = 125 MHz, pclk = 62.5 MHz. This is a register default change, no RTL edit, no floorplan change. The flight loop then has 125,000 cycles per iteration, which remains far more than the workload needs.

[N-7.3] The fallback is stated in advance deliberately. A timing miss found during PD is a schedule problem only if the recovery has to be invented at that point. Here it is a parameter the PD engineer can apply without consulting anyone.

[N-7.4] Order of remedies, if 250 MHz is close but missing:

- Confirm the EX cone is not routed around a macro ([N-6.2]).

- Retime or pipeline the 33×33 multiplier — this changes the core and needs re-verification, so it is a last resort, not a first.

- DIVSEL = 1. Costs nothing functional.

[N-7.5] Step 2 before step 3 only if there is a reason to want 250 MHz specifically. There is no functional reason in this design; the only reason is the programme's expectation of the core's performance.

## 8 Board-level requirements

[N-8.1] These are requirements on the board, arising from chip-level decisions. The board designer needs them and would not find them in a block spec.

| # | Requirement | Source |
| --- | --- | --- |
| B-1 | A supply supervisor shall hold ext_rst_n low until the core rail is stable. The chip has no analog power-on-reset. | ADR-0019 |
| B-2 | A 500 MHz reference oscillator, 45–55% duty, ≤200 ps peak-to-peak jitter. Not a crystal — a driven XO or equivalent. | ADR-0001 |
| B-3 | The refclk trace shall be short, impedance-controlled, with a continuous return path, and routed away from the PWM outputs. | ADR-0018, [N-4.4] |
| B-4 | I²C pull-ups sized at board level; the chip provides none. | [N-3.3] |
| B-5 | boot_sel may be left unconnected for flash boot; tie high for JTAG recovery. | [N-3.1] |
| B-6 | The ground-link radio is a board-level choice of one part — telemetry radio or LoRa — on uart1. The chip does not support both. | ADR-0008 |
| B-7 | Sonar shall be an I²C part. The design has no UART available for it. | ADR-0008 |
| B-8 | In the 28-pin configuration there is no ESP-NOW mesh link. | ADR-0020 |
| B-9 | Boot flash and IMU share the SPI master bus with separate chip selects. | [N-3.5] |

## 9 Handoff checklist

[N-9.1] For the GDSII package, in order:

| # | Item | Owner | Reference |
| --- | --- | --- | --- |
| 1 | Confirm whether supply pads are inside the 40 | PD mentor | §2, [N-2.4] |
| 2 | Pad ring per §3.1 (or §3.2 if item 1 says so) | PD | §3 |
| 3 | Supply pad distribution per [N-4.1] | PD | §4 |
| 4 | u_clk_div.t1_q abutting the refclk pad; refclk fanout = 1 | PD | §5.1, [N-5.8] |
| 5 | SDC per §5.2 (flow/constraints/garuda.sdc), including the aon_clk/hclk/pclk synchronous relationship | PD | §5.2, [N-5.7] |
| 6 | Core + DSU as a contiguous region | PD | [N-6.2] |
| 7 | WFI ICG checked at gate level for runt pulses | PD + verification | [N-5.8] |
| 8 | If 250 MHz misses, apply DIVSEL = 1 rather than escalating | PD | §7 |

## 10 Open items

[N-10.1] One remains, and it is the only irreversible item in this project:

| ID | Item | Owner | Consequence if the assumption is wrong |
| --- | --- | --- | --- |
| OPEN-2 | Are supply pads inside the 40? | PD mentor, against the programme's pad frame | If they are not: apply §3.2, additively, nothing moves. If they are: the baseline is already correct. Either way the design is safe — which is why it was closed this way. |

All other opens from the original eleven documents are closed. See GARUDA-ADR-001 "Open items — all closed."