# GARUDA SPI Master — Design Specification

## 0.1 Identity

| Field | Value |
| --- | --- |
| Document ID | GARUDA-SPIM-SPEC-001 |
| Revision | 1.0 |
| Date | 2026-09-22 |
| Status | Released for implementation |
| Block | 13 (spi_master) |
| Owner | Team AeroSoC |
| Supersedes | — (first revision; the block was sourced_ip with no document) |

## 0.2 Revision history

| Rev | Change | Driver |
| --- | --- | --- |
| 1.0 | First specification. The block is an adaptation of pulp-platform/apb_spi_master behind a GARUDA wrapper, not vendor IP. | D-22 |

## 0.3 Normative references

- GARUDA-SYS-001 Rev 4.0 — System Definition. All cross-block values come from it.

- GARUDA-AHB2APB-SPEC-001 Rev 2.0 — the APB contract this block must satisfy.

- GARUDA-MEM-SPEC-001 Rev 2.0 §8 — the boot sequence this block must support.

- GARUDA-DMA-SPEC-001 Rev 3.0 §7.3 — the request/acknowledge handshake.

- Docs/DECISIONS.md D-16, D-21, D-22.

## 1 Purpose and scope

### 1.1 In scope

One SPI master on APB window 1, serving two devices on one bus: the boot flash (cs_flash_n) and the IMU (cs_imu_n). It is the only block the Boot ROM needs beyond the core, the bus and the memories, so it is on the critical path of every board bring-up.

### 1.2 Out of scope

- The flash image format and the CRC — GARUDA-MEM-SPEC-001 §8.

- The IMU driver and its sample rate — firmware.

- Quad/dual SPI. The upstream IP supports them; this block does not use them ([N-7.6]).

### 1.3 Where the RTL comes from

rtl/third_party/pulp/apb_spi_master + rtl/third_party/pulp/axi_spi_master (Solderpad 0.51), unmodified, behind rtl/spi_master/garuda_spim_top.v. The upstream register model — a command/address/dummy/data length engine — is a flash read, which is why the boot path fits the 4 KiB ROM budget.

## 2 Requirements

| ID | Requirement | Source |
| --- | --- | --- |
| R-1 | Read the boot flash with a single-bit SPI mode-0 transfer: command 0x03, 24-bit address, streamed data. | MEM §8.2 |
| R-2 | Hold one chip select asserted across command, address and data phases of a transfer. | MEM §8.3 |
| R-3 | Two independent chip selects: cs_flash_n, cs_imu_n. | PHYS [N-3.5], ADR-0008 |
| R-4 | SCLK ≤ 20 MHz from pclk = 125 MHz. | MEM §8.2 step 3 |
| R-5 | Never stall the APB bus: the register interface is polled. | AHB2APB [N-7.15] |
| R-6 | One held, level interrupt to CLIC ID 15. | CLIC §7.2, D-17 |
| R-7 | DMA request/acknowledge on channel 0, one beat per assertion. | DMA §7.3, D-21 |
| R-8 | cs_*_n high and sclk low from reset until firmware acts. | Board safety |

## 3 Block diagram

APB window 1 (pclk 125 MHz)
        │
        ▼
  ┌──────────────────────── garuda_spim_top ────────────────────────┐
  │                                                                 │
  │  garuda_apb_shim ──── IP APB ────▶ apb_spi_master (vendored)    │
  │   0x000-0x0FF  pass-through          ├─ spi_master_apb_if       │
  │   0xFE0-0xFEC  IRQSTAT/IRQEN/        ├─ spi_master_controller   │
  │                DMACTL/ID             ├─ spi_master_clkgen       │
  │                                      ├─ spi_master_tx / _rx     │
  │  events ──▶ sticky IRQSTAT           └─ spi_master_fifo x2      │
  │  FIFO counts ──▶ dma_req (hold-off)         │                   │
  │  miso ──▶ 2-flop sync                       ▼                   │
  └──────────────────────────────────────── sclk, mosi, miso, csn ──┘
                                                 │
                                    cs_flash_n ──┴── cs_imu_n

## 4 Sub-block inventory

| Name | Type | RTL file | Role |
| --- | --- | --- | --- |
| garuda_apb_shim | seq (pclk) | rtl/common/garuda_apb_shim.v | window decode, PSLVERR, sticky IRQ, DMA request, pad sync |
| apb_spi_master | seq (pclk) | rtl/third_party/pulp/apb_spi_master/src/ | register file + SPI engine (vendored) |
| spi_master_controller | seq | rtl/third_party/pulp/axi_spi_master/src/ | command/address/data sequencer, chip select |
| spi_master_clkgen | seq | same | SCLK generation from CLKDIV |
| garuda_spim_top | wrapper | rtl/spi_master/garuda_spim_top.v | CS mapping, mode forcing, safe reset state |

## 5 Interfaces

| Port | Dir | Width | Domain | Reset | Description |
| --- | --- | --- | --- | --- | --- |
| pclk_i, preset_n_i | in | 1 | pclk | — | APB domain, 125 MHz |
| APB slave | — | — | pclk | — | window 1, 0x4000_1000 |
| irq_o | out | 1 | pclk | 0 | to CLIC ID 15, level |
| dma_req_o | out | 1 | pclk | 0 | DMA channel 0, level |
| dma_ack_i | in | 1 | pclk | — | 2 hclk = 1 pclk (D-16) |
| spim_sclk_o | out | 1 | pclk | 0 | SCLK |
| spim_mosi_o | out | 1 | pclk | 0 |  |
| spim_miso_i | in | 1 | async | — | synchronised in the wrapper ([N-9.2]) |
| spim_cs_flash_n_o | out | 1 | pclk | 1 | boot flash |
| spim_cs_imu_n_o | out | 1 | pclk | 1 | IMU |

## 6 Register map — APB window 1 (0x4000_1000)

No machine-readable register description exists for this block yet; this table is the register map. Offsets 0x00–0x2C are the vendored register file; 0xFE0–0xFEC are the GARUDA tail common to every peripheral (D-21).

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x00 | STATUS | RW | 0 | Write starts a transfer; read returns engine state ([N-6.1]) |
| 0x04 | CLKDIV | RW | 0 | SCLK divider ([N-6.2]) |
| 0x08 | SPICMD | RW | 0 | Command word, MSB-first |
| 0x0C | SPIADR | RW | 0 | Address word, MSB-first |
| 0x10 | SPILEN | RW | 0 | [5:0] cmd bits, [13:8] addr bits, [31:16] data bits |
| 0x14 | SPIDUM | RW | 0 | [15:0] dummy cycles before read, [31:16] before write |
| 0x18 | TXFIFO | W | — | Transmit data (32-bit) |
| 0x20 | RXFIFO | R | — | Receive data (32-bit) |
| 0x24 | INTCFG | RW | 0 | Upstream FIFO threshold configuration |
| 0x28 | INTSTA | R | 0 | Upstream event status |
| 0xFE0 | IRQSTAT | W1C | 0 | [0] transfer complete, [1] RX threshold, [2] TX threshold |
| 0xFE4 | IRQEN | RW | 0 | mask; irq_o = ¦(IRQSTAT & IRQEN) |
| 0xFE8 | DMACTL | RW | 0 | [0] request on RX data, [1] request on TX space |
| 0xFEC | ID | RO | — | {16'h6A5D, 8'd13, rev} |

### 6.1 STATUS (0x00)

Write — [0] start read, [1] start write, [4] soft reset, [11:8] chip-select register.

[N-6.1] [8] selects cs_flash_n and [9] selects cs_imu_n; [11:10] are unused and must be written 0. The upstream IP has four chip selects; the wrapper maps the low two to pins and ties the rest off.

Read — [0] engine idle, [20:16] RX FIFO occupancy, [28:24] TX FIFO occupancy.

[N-6.1a] Poll [20:16], not [0], to wait for read data. [0] means "the engine is in its IDLE state", which is also true in the cycles between the STATUS write and the engine acting on it: a poll loop tight enough to read STATUS before the engine has started sees idle, reads an empty RXFIFO and gets zero. Waiting for a word to arrive has no such race. For a transfer with no read data (a write, or a bare command), wait on IRQSTAT[0] instead — clear it, start the transfer, poll it — which is sticky and cannot be missed.

### 6.2 CLKDIV (0x04)

[N-6.2] SCLK = pclk / (2 × (CLKDIV + 1)). At pclk = 125 MHz, R-4 (≤20 MHz) requires CLKDIV ≥ 3, which gives 15.6 MHz. CLKDIV = 3 is the boot value. A DIVSEL change (CLKRST §6.2) changes pclk and therefore SCLK; firmware that lowers the chip frequency must re-program this register, and the Boot ROM runs at the reset DIVSEL, so the boot value is always valid.

### 6.3 Chip-select discipline

[N-6.3] The engine asserts the selected chip select for the whole transfer — command, address, dummy and data — and releases it at the end. This is R-2, and it is why a flash read works without software CS control. Firmware must not change [11:8] while STATUS[0] reads 0.

## 7 Functional description

### 7.1 A flash read, register by register

[N-7.1] The sequence the Boot ROM performs (sw/bootrom/spim.c), once per 32-bit word:

CLKDIV = 3                              once, at spim_init()
SPICMD = 0x03 << 24                     command 0x03 in the top 8 bits
SPIADR = byte_addr << 8                 24-bit address in the top 24 bits
SPILEN = (32 << 16) | (24 << 8) | 8     32 data bits, 24 addr bits, 8 cmd bits
STATUS = (1 << 8) | (1 << 0)            select cs_flash_n, start a read
poll STATUS until [20:16] != 0          a word has arrived ([N-6.1a])
word = RXFIFO

[N-7.2] SPICMD and SPIADR are shifted MSB-first from bit 31, which is why both are left-aligned. Getting this wrong is the classic flash bring-up failure: the command goes out as zeros and the flash returns all ones, which looks exactly like an erased device.

[N-7.3] RXFIFO returns the four bytes in the order they arrived, first byte in [31:24]. spim_read_word() therefore byte-swaps into the little-endian word the image header expects, and tools/mkbootimg.py writes the image in that same order. The two must agree; the test that proves they do is make test_chip_flash.

[N-7.3a] Boot time. One word per command costs 64 SCLK — 8 command, 24 address, 32 data — so half the bus time is per-word overhead. Measured: 4.1 µs per word at CLKDIV = 3, and make test_chip_flash boots a 700-byte image in 1.12 ms. Extrapolated to a full 64 KiB ISRAM image that is ~67 ms, which is a long time to hold a flight controller in reset.

The fix, when boot time becomes a requirement, is a burst read: the engine holds the chip select for the whole transfer, so one command with SPILEN.data = 32 × n streams n words into the RX FIFO back to back and drops the overhead to zero, taking the same image to ~34 ms. It is not done today because spim_read_word() is the interface boot.c is written against and the RX FIFO is only BUFFER_DEPTH (10) words deep, so a burst needs the ROM to drain it while the transfer runs. Left as a deliberate, measured trade: simple ROM now, a known 2× available later without a spec change.

### 7.2 Mode

[N-7.4] Mode 0 only: SCLK idles low, data is sampled on the rising edge and launched on the falling edge.

[N-7.5] The wrapper never asserts the upstream quad-mode controls, so the engine stays in single-bit mode (SPI_STD) and only mosi drives ([N-7.6]).

[N-7.6] The upstream lanes are quad-SPI pads, and single-bit mode uses two different lane numbers for the two directions: spi_master_tx drives IO0 (spi_sdo0 → mosi) and spi_master_rx shifts in IO1 (data_int_next = {data_int[30:0], sdi1}), which is where MISO sits on a quad-capable flash. So the wrapper connects miso to spi_sdi1, not spi_sdi0. spi_sdo1..3, spi_sdi0, spi_sdi2..3 are tied off and spi_mode is ignored: GARUDA has no pins for them (PHYS §3.1). Connecting MISO to sdi0 is a silent failure — the transfer runs, the chip select and SCLK look perfect on a scope, and every word reads back as zero.

### 7.3 Interrupt

[N-7.7] IRQSTAT[0] captures transfer completion, [1]/[2] the upstream RX/TX FIFO threshold events. All three are captured sticky by the shim, so a one-cycle upstream pulse becomes the held level the CLIC requires (D-17, D-21).

### 7.4 DMA

[N-7.8] With DMACTL[0] set, dma_req_o asserts while the RX FIFO holds at least one word, and drops for one pclk after each dma_ack_i — the one-beat rule of D-21. DMACTL[1] does the same for TX space.

[N-7.9] DMA beats are 32-bit words, so channel 0 must be configured with CR.SIZE = word. An IMU burst therefore moves four bytes per beat.

[N-7.10] The Boot ROM does not use DMA (ADR-0015).

## 8 Timing

### 8.1 Flash read, CLKDIV = 3

│ cmd 0x03 (8 bits) │ addr (24 bits) │ data (32 bits) │
cs_flash_n ──┐_________________________________________________┌───
sclk       ───┐_┌─┐_┌─┐_ ... ┌─┐_┌─┐_ ... ┌─┐_┌─┐_ ...  ┌─┐_┌───────
               (15.6 MHz: pclk/8)
mosi       ───< 0 0 0 0 0 0 1 1 >< A23 ... A0 ><        0s        >
miso       ─────────────────────────────────────< D31 ... D0     >
STATUS[0]  ──┐_______________________________________________┌─────
             (busy)                                    (idle again)

Total 64 SCLK cycles ≈ 4.1 µs per word at 15.6 MHz. A 64 KiB image is 16,384 words ≈ 67 ms, consistent with the ≈45 ms figure in MEM §8 (which assumes 20 MHz).

### 8.2 DMA request hold-off

rx fifo    ──┤ 1 word │ 1 word │ 1 word ├
dma_req    ──┌────────┐   ┌────┐   ┌────
dma_ack    ──────┌─┐──────────┌─┐───────   (1 pclk, from the DMA)
                 │            │
                 └── req drops here, for at least one pclk, or the
                     channel's taken_q latches and never re-arms (D-21)

## 9 Clock, reset and power

[N-9.1] One clock: pclk. irq_o and dma_req_o cross to hclk synchronously (D-5), no synchroniser.

[N-9.2] spim_miso_i is asynchronous to pclk and is passed through the shim's two-flop synchroniser before it reaches the engine.

[N-9.3] Reset state (R-8): cs_flash_n = cs_imu_n = 1, sclk = 0, mosi = 0. A board must not see a chip select asserted before firmware runs.

[N-9.4] Power: the window's PSEL is the natural clock-gate enable (AHB2APB [N-9.5]); no gating is implemented in this block.

## 10 Assertions

// PREADY is never deasserted: the bridge would time out (R-5)
a_pready:   assert property (@(posedge pclk_i) psel_i |-> pready_o);

// chip selects are mutually exclusive and idle high when the engine is idle
a_cs_excl:  assert property (@(posedge pclk_i) disable iff (!preset_n_i)
              !(!spim_cs_flash_n_o && !spim_cs_imu_n_o));
a_cs_idle:  assert property (@(posedge pclk_i) disable iff (!preset_n_i)
              status_idle |-> (spim_cs_flash_n_o && spim_cs_imu_n_o));

// the selected chip select stays asserted for the whole transfer (R-2)
a_cs_held:  assert property (@(posedge pclk_i) disable iff (!preset_n_i)
              $fell(spim_cs_flash_n_o) |-> !spim_cs_flash_n_o throughout
              (##[1:$] status_idle));

// SCLK never exceeds 20 MHz: at least 3 pclk between edges (R-4)
a_sclk_rate: assert property (@(posedge pclk_i) disable iff (!preset_n_i)
              $changed(spim_sclk_o) |=> !$changed(spim_sclk_o)[*3]);

// the DMA one-beat rule (D-21)
a_dma_drop: assert property (@(posedge pclk_i) disable iff (!preset_n_i)
              dma_ack_i |=> !dma_req_o);

## 11 Verification plan

make test_spim — tb/spi_master/tb_spim.sv, 24 checks, 0 failures.

| Req | Test | Oracle | Status |
| --- | --- | --- | --- |
| R-1, R-2 | t_spim_flash_read | spi_flash_model returns the programmed bytes | pass |
| R-3 | t_spim_cs | only the selected CS falls; never both | pass |
| R-4 | t_spim_sclk_rate | measured SCLK period ≥ 8 pclk at CLKDIV=3 | pass |
| R-5 | t_spim_pready | PREADY high in every cycle of every access | pass |
| R-6 | t_spim_irq | held after a one-cycle event; cleared by W1C only | pass |
| R-7 | t_spim_dma | dma_req_checker sees no stuck request | pass |
| R-8 | t_spim_reset | pins at safe idle out of reset | pass |
| §7.1 | test_chip_flash | the real Boot ROM boots an image out of the flash model | pass |

R-1 is the one that earns its keep. Every other row above passed while MISO was connected to the wrong upstream lane and every word read back as zero (D-23). A register-level test cannot find that; only comparing bytes against a flash programmed with a known pattern can.

## 12 Design decisions

| Decision | ADR / ruling |
| --- | --- |
| Adapt PULP IP rather than write or wait | D-22 |
| Uniform shim: held IRQ, DMA hold-off, word-only decode | D-21 |
| Two chip selects on one bus (flash + IMU) | ADR-0008, PHYS [N-3.5] |
| No DMA in the boot path | ADR-0015 |

## 13 Not implemented

| Feature | Why not |
| --- | --- |
| Quad / dual SPI | No pins (PHYS §3.1). The upstream engine supports it; the wrapper does not expose it. |
| SPI slave mode | Block 14 was dropped for the pin budget (ADR-0020). |
| Automatic flash programming | Boot is read-only; programming is a debugger/JTAG job (DEBUG §7.6). |
| Per-byte CS toggling | Would break a flash read (R-2). |

## 14 Open items

OPEN-13: the IMU's SPI mode and maximum SCLK are a board decision; CLKDIV for the IMU is firmware's to set. Nothing in this block depends on the answer.

## 15 Errata

None. First revision.