# GARUDA SPI Slave — Design Specification

## 0.1 Identity

| Field | Value |
| --- | --- |
| Document ID | GARUDA-SPIS-SPEC-001 |
| Revision | 1.0 |
| Date | 2026-09-26 |
| Status | Released for implementation |
| Block | 14 (spi_slave) |
| Owner | Team AeroSoC |
| Supersedes | — (first revision; the block was reserved with no document) |

## 0.2 Revision history

| Rev | Change | Driver |
| --- | --- | --- |
| 1.0 | First specification. In-house RTL. The block was held as a reserved number by ADR-0020 and restored as required by ADR-0020 Rev 2. | ADR-0020 Rev 2, D-25 |

## 0.3 Normative references

GARUDA-SYS-001 Rev 4.0 · GARUDA-AHB2APB-SPEC-001 Rev 2.0 · GARUDA-DMA-SPEC-001 Rev 3.0 §7.3 · GARUDA-CLIC-SPEC-001 Rev 2.0 §7.2 · GARUDA-PHYS-SPEC-001 §3.2 · GARUDA-DEBUG-SPEC-001 [N-7.16] · Docs/DECISIONS.md D-14, D-17, D-21, D-25.

## 1 Purpose and scope

### 1.1 In scope

One SPI slave on APB window 0 (0x4000_0000), CLIC ID 14, DMA channel 4, pins 29–32 spis_sclk / spis_mosi / spis_miso / spis_cs_n. The master is an ESP32 companion running ESP-NOW.

Why this block exists at all, in one line, because it was nearly deleted: ESP-NOW is the neighbour-position source for APF collision avoidance — the workload the DSU exists to accelerate. ADR-0020 Rev 1 cut it as "the least essential system function"; Rev 2 reversed that, because without it the swarm feature has no input. It is load-bearing.

### 1.2 Out of scope

- Master mode. That is block 13 (GARUDA-SPIM-SPEC-001).

- Quad/dual SPI, and CPOL/CPHA other than mode 0 ([N-7.2]).

- The ESP-NOW protocol, packet format and framing above the byte — firmware's.

### 1.3 Where the RTL comes from

In-house, rtl/spi_slave/garuda_spis_core.v + garuda_spis_top.v. Written rather than adapted, and the reason is [N-7.1] rather than taste: the interesting decision in an SPI slave is how SCLK enters the chip, and that is an architectural choice this design has already made elsewhere. Vendoring a slave would import someone else's answer to it.

## 2 Requirements

| ID | Requirement | Source | Status |
| --- | --- | --- | --- |
| R-1 | Receive bytes from an external SPI master, MSB first, mode 0. | ADR-0020 Rev 2 | met |
| R-2 | Transmit a response byte on the same transfer. | ADR-0020 Rev 2 | met |
| R-3 | Frame a packet on cs_n: a falling edge starts one, a rising edge ends it. | [N-7.3] | met |
| R-4 | Add no new asynchronous clock domain to the chip. | DEBUG [N-7.16], D-14 | met ([N-7.1]) |
| R-5 | Word-only APB, never stalls. | AHB2APB [N-7.12], [N-7.15] | met |
| R-6 | One held, level interrupt to CLIC ID 14. | CLIC §7.2, D-17 | met |
| R-7 | DMA request/acknowledge on channel 4, one beat per assertion, byte in [7:0]. | DMA §7.3, D-21 | met |
| R-8 | Report an RX overrun rather than dropping a byte silently. | [N-7.5] | met |
| R-9 | miso released and all state idle out of reset; a partial packet must not be presented as data. | Board safety | met |

## 3 Block diagram

APB window 0 (pclk 125 MHz)
        │
        ▼
  ┌──────────────── garuda_spis_top ─────────────────┐
  │  garuda_apb_shim ──▶ register file               │
  │   0x00-0x1C  RXDATA TXDATA CTRL STATUS           │
  │   0xFE0-0xFEC  tail                              │
  │                     │                            │
  │                     ▼   garuda_spis_core         │
  │   sclk/mosi/cs_n ─▶ 2-flop sync ─▶ edge detect   │
  │                     ─▶ 8-bit shifter ─▶ RX FIFO  │
  │                     ◀─ TX holding reg ─▶ miso    │
  └──────────── sclk, mosi, miso, cs_n ──────────────┘
        ▲ all sampled in pclk - no second clock domain

## 4 Sub-block inventory

| Name | Type | RTL file | Role |
| --- | --- | --- | --- |
| garuda_apb_shim | seq (pclk) | rtl/common/garuda_apb_shim.v | decode, PSLVERR, sticky IRQ, DMA, pad sync |
| garuda_spis_core | seq (pclk) | rtl/spi_slave/garuda_spis_core.v | oversampled shifter, CS framing, byte assembly |
| garuda_spis_top | seq (pclk) | rtl/spi_slave/garuda_spis_top.v | register file, FIFO, events |

## 5 Interfaces

| Port | Dir | Width | Domain | Reset | Description |
| --- | --- | --- | --- | --- | --- |
| pclk_i, preset_n_i | in | 1 | pclk | — | 125 MHz |
| APB slave | — | — | pclk | — | window 0, 0x4000_0000 |
| irq_o | out | 1 | pclk | 0 | CLIC ID 14, level |
| dma_req_o / dma_ack_i | — | 1 | pclk | 0 | DMA channel 4 |
| spis_sclk_i | in | 1 | async | — | driven by the ESP32; oversampled ([N-7.1]) |
| spis_mosi_i | in | 1 | async | — | oversampled |
| spis_cs_n_i | in | 1 | async | — | oversampled |
| spis_miso_o | out | 1 | pclk | 0 | response data |
| spis_miso_oe_o | out | 1 | pclk | 0 | 0 = released; driven only while selected ([N-9.2]) |

## 6 Register map — APB window 0 (0x4000_0000)

[N-6.0] Window 0 was deliberately left unmapped by ADR-0020 rather than compacting the window map when this block was cut, precisely so it could come back without moving anything. This is the block it was held for.

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x00 | RXDATA | RO | 0 | [7:0] received byte; reading pops the FIFO ([N-6.1]) |
| 0x04 | TXDATA | RW | 0 | [7:0] byte shifted out on the next transfer ([N-6.2]) |
| 0x08 | CTRL | RW | 0 | [0] EN, [1] RXFLUSH (self-clearing) |
| 0x0C | STATUS | RO | 0 | [0] RXVALID, [1] CS active, [2] OVERRUN (sticky), [3] packet done (sticky), [8:4] RX FIFO level |
| 0xFE0 | IRQSTAT | W1C | 0 | [0] byte received, [1] packet complete (CS rose), [2] overrun |
| 0xFE4 | IRQEN | RW | 0 | mask; irq_o = ¦(IRQSTAT & IRQEN) |
| 0xFE8 | DMACTL | RW | 0 | [0] request while a received byte is waiting |
| 0xFEC | ID | RO | — | {16'h6A5D, 8'd14, rev} |

[N-6.1] Reading RXDATA pops one byte and drops the DMA request for it. [31:8] read 0.

[N-6.2] TXDATA is not double-buffered per byte: it is loaded into the shifter at the start of each byte. Firmware or the DMA must write the next response before the master clocks the following byte, which at the [N-7.2] maximum SCLK leaves at least 8 × 6 = 48 pclk. If nothing is written, the previous value is re-sent — a defined, repeatable value, not an X.

## 7 Functional description

### 7.1 SCLK does not get its own clock domain — and that is the design

[N-7.1] An SPI slave's shift clock is generated by the other end of the wire. The obvious implementation clocks the shift register on sclk directly and hands bytes to pclk through an asynchronous FIFO. This design does not do that, for a reason that is about the chip and not about this block:

GARUDA-DEBUG-SPEC-001 [N-7.16] states that the JTAG dmi_cdc crossing is the only asynchronous crossing in the chip. That single sentence is worth a great deal at signoff — one CDC to constrain, one to review, one to argue about with a mentor. Adding a second, on a peripheral, to save a handful of flops, spends it.

So sclk, mosi and cs_n are synchronised and oversampled in pclk. Edges are detected on the synchronised sclk, and the shifter advances on a detected rising edge. The chip keeps exactly one asynchronous boundary, and this block needs no CDC review, no asynchronous FIFO and no clock constraint of its own.

[N-7.1a] The price, stated plainly. Oversampling imposes a maximum SCLK. Two pclk of synchroniser plus edge detection need the half-period to be at least three pclk to be seen reliably, so SCLK ≤ pclk / 6 = 20.8 MHz, and the specification limit is SCLK ≤ 20 MHz with pclk at 125 MHz. An ESP32 SPI master defaults well below this; it is a configuration constraint on the companion, recorded here and checked by t_spis_rate.

[N-7.1b] If DIVSEL lowers pclk, the SCLK limit scales with it. At the DIV=4 timing fallback (pclk = 62.5 MHz) the limit is 10 MHz. Firmware that changes the chip frequency must re-negotiate the companion's SPI clock, exactly as it must re-program the UART divisors.

### 7.2 Mode

[N-7.2] Mode 0 only: SCLK idles low, mosi is sampled on the rising edge and miso changes on the falling edge, MSB first, 8 bits per byte. Matching block 13's mode ([SPIM N-7.4]) keeps one convention in the chip.

### 7.3 Packet framing

[N-7.3] cs_n falling starts a packet: the bit counter and shifter reset, so a packet always begins byte-aligned regardless of what preceded it. cs_n rising ends it and sets IRQSTAT[1] and STATUS[3].

[N-7.3a] A packet whose bit count is not a multiple of 8 leaves a partial byte in the shifter. That partial byte is discarded, never pushed. A truncated ESP-NOW frame must not appear to firmware as a short but valid one; the packet-complete interrupt plus the FIFO level is how firmware sees the truncation.

### 7.4 Interrupts

[N-7.4] Three sources into the standard sticky tail (D-21): [0] a byte arrived, [1] the packet completed, [2] overrun. The intended handler waits on [1], then drains the FIFO — one interrupt per packet rather than per byte.

### 7.5 Overrun

[N-7.5] If a byte completes while the RX FIFO is full, the new byte is dropped, STATUS[2] latches and IRQSTAT[2] sets. Dropping the newest rather than the oldest keeps the bytes already accepted contiguous, so a packet is truncated rather than interleaved — a truncated packet fails its checksum, an interleaved one might not.

An overrun means firmware or the DMA did not keep up; with the FIFO depth of [N-8.1] and DMA channel 4 enabled it should not occur, which is why it is reported rather than tolerated.

### 7.6 DMA

[N-7.6] DMACTL[0] requests a beat while STATUS[0] (RXVALID) is set. Word-sized beats, byte in [7:0], request drops for at least one pclk after every dma_ack_i — the shim's hold-off, for the dma_chan.v taken_q rule (D-16, D-21). Channel 4 is this block's; dma_req_i[4] was tied low at the SoC top for as long as the block did not exist, and un-tying it is part of landing this specification.

## 8 Timing

[N-8.1] RX FIFO depth is 16 bytes, matching the UART's. At the maximum SCLK an 8-bit byte takes 400 ns, so a full FIFO represents 6.4 µs of slack for the DMA or firmware to respond — comfortably more than an interrupt latency.

cs_n  ‾‾‾╲______________________________________╱‾‾‾
sclk  ______╱‾╲_╱‾╲_╱‾╲_ ... _╱‾╲______________________
mosi  ----< b7 X b6 X b5 X ... X b0 >------------------
           ▲ sampled on the rising edge (oversampled in pclk)
                                    ▲ byte pushed here
                                                   ▲ packet done

## 9 Clock, reset and power

[N-9.1] One clock domain: pclk. No CDC — that is [N-7.1] and it is the block's main design property. The three pad inputs pass through the shim's two-flop synchronisers.

[N-9.2] spis_miso_oe_o is 0 out of reset and is asserted only while cs_n is active and CTRL.EN is set. On a bus the ESP32 may share, a slave that drives MISO while not selected is a contention fault, not a protocol one.

[N-9.2a] There is a bounded release tail, and it is deliberate. cs_n reaches the block through the shim's two-flop synchroniser, so the output enable drops 2 pclk (16 ns at 125 MHz) after the pin rises, measured in tb_spis. Releasing from the raw pin instead would remove the tail but put a combinational path from an asynchronous pad input onto a pad output enable — trading a bounded, analysable 16 ns against an unconstrainable path, on the one block whose entire design premise ([N-7.1]) is that it adds no asynchronous timing to the chip. The tail is accepted and bounded: t_spis_reset fails if it ever exceeds 3 pclk. At the maximum SCLK a bit is 50 ns, so no master can reselect inside it.

[N-9.3] All framing state — bit counter, shifter, byte-assembly — resets with preset_n and again on every cs_n falling edge ([N-7.3]), so the block cannot come up mid-byte after a reset that lands during a transfer.

## 10 Assertions

a_miso_only_when_selected: assert property (@(posedge pclk_i) disable iff (!preset_n_i)
    spis_miso_oe_o |-> (cs_active && en_q));
a_pready:   assert property (@(posedge pclk_i) disable iff (!preset_n_i)
    psel_i |-> pready_o);
a_dma_drop: assert property (@(posedge pclk_i) disable iff (!preset_n_i)
    dma_ack_i |=> !dma_req_o);
a_no_partial_push: assert property (@(posedge pclk_i) disable iff (!preset_n_i)
    cs_rise |-> !rx_push);

## 11 Verification plan

make test_spis — tb/spi_slave/tb_spis.sv against tb/models/spi_master_model.sv, 32 checks, 0 failures.

| Req | Test | Oracle | Status |
| --- | --- | --- | --- |
| R-1 | t_spis_rx | bytes the model sends appear in RXDATA, in order | pass |
| R-2 | t_spis_tx | the model receives what TXDATA was loaded with | pass |
| R-3 | t_spis_packet | cs_n framing: bit counter resets, IRQSTAT[1] on the rising edge | pass |
| R-3 | t_spis_partial | a 12-bit packet delivers one byte, not two ([N-7.3a]) | pass |
| R-4 | t_spis_rate | correct at every half period from 49 ns down to 9 ns, none of them a pclk multiple. Caveat: ideal simulation edges cannot find the real limit, which is set by metastability and edge rates — the pclk/6 figure is an analysis result the sweep confirms, not one it derives | pass |
| R-5 | t_spis_pready | PREADY high in every cycle of every access | pass |
| R-6 | t_spis_irq | held ≥50 pclk, drops only on W1C | pass |
| R-7 | t_spis_dma | dma_req_checker; request drops after ack | pass |
| R-8 | t_spis_overrun | 20 bytes into a 16-deep FIFO: 16 kept, IRQSTAT[2] set, no interleave | pass |
| R-9 | t_spis_reset | miso_oe low out of reset and while deselected | pass |
| — | t_chip_spis | the block from the core, through the real fabric | new |

## 12 Design decisions

| # | Decision | Alternative rejected |
| --- | --- | --- |
| 1 | Oversample SCLK in pclk | a real sclk domain + async FIFO — a second CDC in the chip, for one peripheral ([N-7.1]) |
| 2 | Drop the newest byte on overrun | dropping the oldest — truncates rather than interleaves ([N-7.5]) |
| 3 | Discard a partial byte at cs_n rise | pushing it — a truncated frame must not look valid ([N-7.3a]) |
| 4 | TXDATA single-buffered | per-byte double buffering — 48 pclk of slack is ample, and it re-sends a defined value |
| 5 | Window 0 | compacting the map — ADR-0020 held this slot for exactly this |

## 13 Not implemented

[N-13.1] CPOL/CPHA other than mode 0, quad/dual, 16-bit words, hardware packet length or CRC. The ESP-NOW frame format is firmware's.

[N-13.2] No TX FIFO: one holding register ([N-6.2]).

## 14 Open items

- OPEN-S1 — bonding, not RTL. Whether pins 29–32 are additive or reclaimed is OPEN-2, the PD mentor's call against the pad frame (ADR-0020 Rev 2, PHYS [N-3.6]). The RTL, the window, the CLIC ID and the DMA channel are all in place either way; what remains is a pad-ring decision. WITH_SPI_SLAVE on garuda_chip_top is the switch.

## 15 Errata

None. This block has no vendored RTL.