# GARUDA Timers and Watchdog — Design Specification

## 0.1 Identity

| Field | Value |
| --- | --- |
| Document ID | GARUDA-TIMERS-SPEC-001 |
| Revision | 2.0 |
| Date | 2026-09-18 |
| Status | Released for implementation |
| Block | 11 (timers) |
| Owner | Team AeroSoC |
| Supersedes | GARUDA_Timers_Design_Spec_v1_0 |

## 0.2 Revision history

| Rev | Change | Driver |
| --- | --- | --- |
| 1.0 | Initial. 64-bit mtime/mtimecmp exported to the core; mtip routed via CLIC ID 13 | — |
| 2.0 | The 64-bit comparison moves into this block; a single mtip wire goes to the core, removing 128 top-level signals and a 64-bit comparator from the core's timing cone. Watchdog request flop moved to the external-reset domain. Early-warning interrupt added on CLIC ID 22. | ADR-0010, 0003 |

## 0.3 Normative references

- RISC-V Privileged Architecture v1.12 — mtime, mtimecmp, mip.MTIP.

- GARUDA-SYS-001 Rev 4.0.

- GARUDA-ADR-001 Rev 1.0.

- GARUDA-CLKRST-SPEC-001 Rev 2.0 — the watchdog reset path and its domain.

- GARUDA-CLIC-SPEC-001 Rev 2.0 — ID 22 for the early warning; ID 13 reserved.

## 1 Purpose and scope

### 1.1 In scope

Two counters:

- Machine timer — the RISC-V architectural timer. 64-bit mtime, 64-bit mtimecmp, and the comparison that produces mtip. Drives the 1 kHz flight loop.

- Watchdog — 32-bit down counter with an early-warning interrupt and a reset on timeout.

### 1.2 Out of scope

- The reset stretching and domain logic for wdt_rst_req (GARUDA-CLKRST-SPEC-001 §7).

- The trap entry for mtip (core).

### 1.3 The change that matters in Rev 2.0

Rev 1.0 exported mtime and mtimecmp as 64-bit buses to the core, and additionally routed mtip through CLIC ID 13. That meant:

- 128 wires crossing the top level for one bit of information.

- A 64-bit comparator inside the core (garuda_core_top.v:126), in a design whose critical path is already the 33×33 multiplier in EX.

- Two delivery mechanisms for one interrupt, because the core implements mip.MTIP directly and three documents said it arrived via the CLIC.

Rev 2.0 does the comparison here and sends one wire. The core keeps its existing, verified mip.MTIP path; the 64-bit ports and the in-core comparator are deleted (RTL delta R4).

## 2 Requirements

| ID | Requirement | Source |
| --- | --- | --- |
| R-1 | 64-bit monotonic mtime counter, readable by firmware. | Priv spec |
| R-2 | 64-bit mtimecmp, writable by firmware. | Priv spec |
| R-3 | mtip shall be delivered to the core as a single wire. | ADR-0010 |
| R-4 | A 32-bit read of mtime's halves shall never return a torn value. | Priv spec |
| R-5 | Writing mtimecmp shall not produce a spurious mtip. | Priv spec |
| R-6 | 32-bit watchdog with a programmable timeout and a magic-value kick. | Safety |
| R-7 | The watchdog shall raise an interrupt before it resets, so firmware can record why. | Safety |
| R-8 | The watchdog's reset request shall not be cleared by the reset it causes. | ADR-0003 |
| R-9 | The watchdog shall not be disableable once enabled. | Safety |

## 3 Block diagram

┌────────────────────────────────────────────────────────────┐
   │                     timers  (block 11)                     │
   │                                                            │
   │  ┌──────────────────────────────────────────────────┐      │
   │  │  MACHINE TIMER                                   │      │
   │  │                                                  │      │
   │  │  ┌────────────┐      ┌──────────────┐            │      │
   │  │  │ mtime      │      │ mtimecmp     │            │      │
   │  │  │ 64-bit up  │      │ 64-bit       │◀── APB     │      │
   │  │  └─────┬──────┘      └──────┬───────┘            │      │
   │  │        │                    │                    │      │
   │  │        │  ┌─────────────────▼──────┐             │      │
   │  │        └─▶│  64-bit comparator     │             │      │
   │  │           │  mtime >= mtimecmp     ├─────────────┼──────┼──▶ mtip_o
   │  │           └────────────────────────┘             │      │   (1 wire)
   │  │        │                                         │      │
   │  │        ├─▶ ┌──────────────┐                      │      │
   │  │        │   │ hi shadow    │── coherent 64-bit    │      │
   │  │        │   │ latch        │   read  (§7.2)       │      │
   │  │        │   └──────────────┘                      │      │
   │  └──────────────────────────────────────────────────┘      │
   │                                                            │
   │  ┌──────────────────────────────────────────────────┐      │
   │  │  WATCHDOG                                        │      │
   │  │                                                  │      │
   │  │  ┌────────────┐   ┌──────────┐  ┌─────────────┐  │      │
   │  │  │ wdt 32-bit │──▶│ == WARN  │─▶│ warn irq    ├──┼──────┼──▶ CLIC 22
   │  │  │ down ctr   │   └──────────┘  └─────────────┘  │      │
   │  │  │            │   ┌──────────┐  ┌─────────────┐  │      │
   │  │  │  kick ◀────┼───│ == 0     │─▶│ rst req flop│──┼──────┼──▶ wdt_rst_req
   │  │  └────────────┘   └──────────┘  │ (ext-reset  │  │      │
   │  │                                 │  domain!)   │  │      │
   │  │                                 └─────────────┘  │      │
   │  └──────────────────────────────────────────────────┘      │
   │                                                            │
   │  ┌──────────────────────────────────────────────────┐      │
   │  │  timers_apb  — window 11 (0x4000_B000)           │◀─ APB │
   │  └──────────────────────────────────────────────────┘      │
   └────────────────────────────────────────────────────────────┘

## 4 Sub-block inventory

| Name | Type | RTL file | Role |
| --- | --- | --- | --- |
| mtime_ctr | seq (hclk) | mtime.v | 64-bit up counter. |
| mtime_shadow | seq (hclk) | mtime.v | Upper-word holding register for coherent reads. |
| mtimecmp_reg | seq (hclk) | mtime.v | 64-bit compare value. |
| mtime_compare | comb | mtime.v | 64-bit unsigned >=. Produces mtip. |
| wdt_ctr | seq (hclk) | wdt.v | 32-bit down counter. |
| wdt_warn | comb | wdt.v | Early-warning threshold comparison. |
| wdt_rst_req_flop | seq (ext-reset domain) | wdt.v | Holds the reset request. See §7.6. |
| timers_apb | seq (pclk) | timers_apb.v | Window 11 register interface. |

## 5 Interfaces

| Port | Dir | Width | Domain | Reset | Description |
| --- | --- | --- | --- | --- | --- |
| hclk_i | in | 1 | hclk | — |  |
| hreset_n_i | in | 1 | hclk | — | Resets mtime, mtimecmp, the watchdog counter. |
| ext_rst_n_i | in | 1 | async | — | Resets only wdt_rst_req_flop. See §7.6. |
| mtip_o | out | 1 | hclk | 0 | To the core's mip.MTIP. The only timer signal the core sees. |
| wdt_warn_irq_o | out | 1 | hclk | 0 | To CLIC ID 22. |
| wdt_rst_req_o | out | 1 | hclk | 0 | To reset_ctrl. |
| APB slave | — | — | pclk | — | Window 11. |

[N-5.1] mtime_i and mtimecmp_i as 64-bit inputs to the core are deleted. The core's port list loses 128 signals and its in-core comparator; see RTL delta R4.

## 6 Register map — APB window 11 (0x4000_B000)

(Window number corrected 2026-09-26: was stated as 10, inherited from the 0-based table in GARUDA-AHB2APB-SPEC-001. The hardware decodes window n at 0x4000_0000 + 0x1000 x n (haddr[15:12]), so the base address quoted here was always right and only the index was wrong.)

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x00 | MTIME_LO | RW | 0 | mtime[31:0]. A read latches mtime[63:32] into the shadow. |
| 0x04 | MTIME_HI | RW | 0 | Reads the shadow, not the live upper word. |
| 0x08 | MTIMECMP_LO | RW | 0xFFFF_FFFF | mtimecmp[31:0]. |
| 0x0C | MTIMECMP_HI | RW | 0xFFFF_FFFF | mtimecmp[63:32]. |
| 0x10 | WDTCTL | RW | 0 | Watchdog control. |
| 0x14 | WDTLOAD | RW | 0xFFFF_FFFF | Reload value. |
| 0x18 | WDTVAL | RO | — | Current count. |
| 0x1C | WDTKICK | W | — | Magic-value write to reload. |
| 0x20 | WDTWARN | RW | 0 | Early-warning threshold. |

### 6.1 MTIMECMP reset value

[N-6.1] mtimecmp resets to all ones, not zero. A zero reset value would satisfy mtime >= mtimecmp on the first cycle after reset and assert mtip before firmware has installed a handler. All-ones means mtip cannot assert for 2⁶⁴ cycles — about 2,300 years at 250 MHz — which is to say never until firmware sets it.

### 6.2 WDTCTL (0x10)

| Bit | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0 | EN | RW1S | 0 | Enable. Write 1 to set. Cannot be cleared by software. |
| 1 | WARNEN | RW | 0 | Enable the early-warning interrupt. |
| 31:2 | reserved | — | 0 |  |

[N-6.2] EN is write-1-to-set and sticky, clearable only by reset. Rationale: a watchdog that firmware can disable is a watchdog that a hung firmware path can disable. The canonical failure is a stuck loop that happens to include a register write that clears the enable. On a flight controller, a watchdog that can be switched off by the bug it exists to catch is worse than no watchdog, because it creates false confidence.

[N-6.3] Once EN is set, firmware must kick within WDTLOAD cycles forever. There is no way back. This is deliberate and must be stated in the firmware's startup documentation.

### 6.3 WDTKICK (0x1C)

[N-6.4] Writing 0x5A5A_C3C3 reloads the counter from WDTLOAD. Any other value is ignored and sets no error.

[N-6.5] A magic value rather than any write: a wild pointer sweeping through the peripheral address space, or a runaway memset, would otherwise kick the watchdog and mask exactly the failure it should catch. A specific 32-bit constant makes an accidental kick improbable.

### 6.4 WDTWARN (0x20)

[N-6.6] When WDTVAL equals WDTWARN and WARNEN is set, wdt_warn_irq_o asserts (CLIC ID 22).

[N-6.7] Purpose: the warning fires while the core is still running, so the handler can record diagnostic state — the current task, a stack pointer, a loop counter — into DSRAM before the reset arrives. After the reset, RSTREASON.WDT says the watchdog fired and the DSRAM record says what the firmware was doing. Without the warning, a watchdog reset tells you only that something hung.

[N-6.8] The warning is level-triggered like every CLIC source, and asserts while WDTVAL == WDTWARN, which is one hclk cycle unless the counter is stopped. Firmware's handler must be short. Setting WDTWARN to roughly 10% of WDTLOAD gives the handler 90% of the timeout budget to run in.

## 7 Functional description

### 7.1 mtime

[N-7.1] mtime is a 64-bit up counter incrementing every hclk cycle. At 250 MHz one tick is 4 ns and the counter wraps after about 2,300 years.

[N-7.2] mtime increments on hclk, not on a divided tick. Rev 1.0's rate was hclk-derived too, so nothing changes, but it is worth noting the consequence: mtime is a cycle counter, and a DIVSEL change (GARUDA-CLKRST-SPEC-001 §6.2) changes its rate. Firmware that uses mtime for wall-clock timing must recompute its scaling after any frequency change. The 1 kHz loop reload value is GARUDA_HCLK_HZ / 1000, taken from the generated header, so it follows the frequency automatically at compile time but not at runtime.

[N-7.3] mtime is writable, as the privileged specification requires, so firmware can set the epoch. A write takes effect immediately and can therefore cause mtip to assert or deassert on the next cycle. That is correct behaviour and firmware should write mtime only during initialisation.

### 7.2 Coherent 64-bit read

[N-7.4] A 32-bit bus cannot read 64 bits atomically. If firmware reads the low word, the counter carries into the high word, and then firmware reads the high word, the combination is wrong by 2³².

[N-7.5] The mechanism: reading MTIME_LO latches mtime[63:32] into mtime_shadow in the same cycle. A subsequent read of MTIME_HI returns the shadow. So the sequence read-LO then read-HI returns a coherent 64-bit value.

[N-7.6] Firmware must read LO first. Reading HI alone returns whatever was last latched, which may be stale. This is stated in the register description and must be in the firmware's timer accessor, not left to whoever writes the next driver.

[N-7.7] The same technique is used for the DSU accumulator taps (GARUDA-DEBUG-SPEC-001 §6.8).

### 7.3 mtip generation

[N-7.8] mtip_o = (mtime >= mtimecmp), a 64-bit unsigned comparison, combinational from the two registers, registered once before leaving the block.

[N-7.9] The comparison is >=, not ==. With ==, a mtimecmp write to a value already passed would never match and the interrupt would be lost for 2⁶⁴ cycles. >= means a late write asserts mtip immediately, which is the recoverable behaviour.

[N-7.10] mtip_o is level, not a pulse. It stays asserted until firmware advances mtimecmp past mtime. This is the privileged specification's contract: the timer handler clears the interrupt by writing the next deadline. A handler that returns without advancing mtimecmp re-enters immediately.

### 7.4 Writing mtimecmp safely

[N-7.11] Writing a 64-bit compare value through 32-bit registers can transiently create a value that is neither the old nor the new one. Writing LO first can make the pair momentarily smaller than mtime, asserting a spurious mtip.

[N-7.12] The required firmware sequence, which the privileged specification also prescribes:

// 1. Set LO to all-ones. The pair is now unreachable, so mtip cannot assert.
MTIMECMP_LO = 0xFFFFFFFF;
// 2. Write the real HI. Safe: LO still blocks any match.
MTIMECMP_HI = (uint32_t)(next >> 32);
// 3. Write the real LO. The pair becomes valid atomically from mtip's view.
MTIMECMP_LO = (uint32_t)next;

[N-7.13] Hardware does not enforce this and cannot: it has no way to know a two-register write is one logical operation. A write-enable side register could serialise it, but that adds a register and a firmware step for a sequence the architecture already specifies. The sequence is therefore a firmware requirement, stated here and in the driver.

[N-7.14] For the 1 kHz loop the deadline advances within the low word almost always (GARUDA_HCLK_HZ / 1000 = 250,000 ticks), so step 2 is only needed on a low-word wrap, roughly every 17 seconds. The driver should still perform all three steps unconditionally; the saving is not worth a conditional path that is exercised once every 17 seconds and therefore almost never tested.

### 7.5 Watchdog counting

[N-7.15] wdt_ctr is a 32-bit down counter, decrementing every hclk cycle while EN is set. At 250 MHz a full 32-bit count is about 17 seconds, so any practical timeout is representable.

[N-7.16] A WDTKICK magic write reloads from WDTLOAD. Reaching zero asserts the reset request.

[N-7.17] The counter does not run while EN is clear, and WDTVAL reads WDTLOAD.

[N-7.18] WDTLOAD is writable while running. The new value takes effect at the next kick, not immediately, so a firmware bug that writes a tiny WDTLOAD does not cause an instant reset — it causes one at the next kick, which is at least diagnosable.

### 7.6 The watchdog reset request and its domain

[N-7.19] wdt_rst_req_flop is reset by ext_rst_n_i, not by hreset_n_i.

[N-7.20] This is the requirement of ADR-0003 seen from the watchdog's side, and it is the single most important sentence in this document. The watchdog's reset request causes hreset_n to assert. If the request flop were in the hreset_n domain, that reset would clear the flop, the request would deassert, and hreset_n would deassert — one flop delay after it asserted. The result is a runt pulse that does not reliably reset anything, and it is invisible in RTL simulation where reset is modelled as instantaneous.

[N-7.21] With the flop in the ext_rst_n domain, the request persists through the reset it caused. reset_ctrl's stretch counter (GARUDA-CLKRST-SPEC-001 §7.2) then holds the reset for its full 1024 reference cycles regardless of the requester's state, and the request is cleared when the counter expires, by an explicit clear from reset_ctrl rather than by the reset itself.

[N-7.22] RSTREASON.WDT is likewise in the ext_rst_n domain, so firmware can read after the reset that the watchdog caused it.

### 7.7 Interaction with debug

[N-7.23] The watchdog continues counting while the core is held in hartreset by the debugger. A debug session that stops the core for longer than the timeout will therefore trigger a watchdog reset.

[N-7.24] This is accepted rather than fixed. The alternatives are worse: a debug-mode freeze bit would need the DM to signal this block, and more importantly a watchdog that stops for one reason will eventually be stopped for another. The practical workaround is that the watchdog is not enabled until late in firmware initialisation, and a debug session that needs to stop the core for a long time uses boot_sel = 1 so firmware never runs. This must be in the bring-up notes, because the symptom — the chip resetting itself every few seconds during a debug session — is confusing if unexpected.

## 8 Timing

### 8.1 mtip assertion and clearing

│        │        │        │        │
hclk        ──┘‾┐_┌‾┐_┌‾┐_┌‾┐_┌‾┐_┌‾┐_┌‾┐_┌‾┐_┌

mtime       ──┤ N-2 │ N-1 │  N  │ N+1 │ N+2 │ N+3 ├
mtimecmp    ──┤         N         │    N+250000   ├
                                  ▲
                                  └── handler writes next deadline

mtip        ──────────────┌───────────┐──────────────
                          ▲           ▲
                          │           └── deasserts when mtimecmp > mtime
                          └── mtime >= mtimecmp  [N-7.8]

(in core)
mip.MTIP    ──────────────────┌───────┐──────────────
take        ──────────────────────┌─┐────────────────

### 8.2 Coherent 64-bit read across a carry

mtime[31:0]  ──┤ FFFF_FFFE │ FFFF_FFFF │ 0000_0000 │ 0000_0001 ├
mtime[63:32] ──┤        0000_0007      │      0000_0008        ├
                                        ▲
                                        └── carry into the high word

read MTIME_LO ──┌─┐
                 │  returns FFFF_FFFF
                 └── latches shadow <= 0000_0007

shadow       ──┤ ???? │   0000_0007 (held)                     ├

read MTIME_HI ──────────────────────────┌─┐
                                         │  returns 0000_0007,
                                         │  NOT the live 0000_0008
                                         └── result: 0000_0007_FFFF_FFFF ✓

Without the shadow, the pair would read 0000_0008_FFFF_FFFF — high by 2³².

### 8.3 Watchdog: warning then reset

WDTLOAD = 250_000_000  (1 s),  WDTWARN = 25_000_000  (100 ms left)

WDTVAL      ──┤ ... │ 25_000_001 │ 25_000_000 │ 24_999_999 │ ... │ 1 │ 0 ├
                                        ▲                                ▲
warn_irq    ────────────────────────────┌──┐                             │
                                        │                                │
                                        └── handler writes diagnostics   │
                                            to DSRAM                     │
                                                                         ▼
wdt_rst_req ────────────────────────────────────────────────────────────┌────
                                                                        │
                        ═══ ext_rst_n domain: survives the reset ═══     │
                                                                        │
hreset_n    ‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾╲___
                                                                   (1024 refclk)

### 8.4 The bug that [N-7.19] prevents

IF wdt_rst_req_flop were in the hreset_n domain:

wdt_rst_req ────────────────┌─┐──────────  asserts on WDTVAL == 0
hreset_n    ‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾╲_┌‾‾‾‾‾‾‾‾‾‾  ◀── runt pulse, ~1 hclk
                              │
                              └── the reset cleared the flop, which
                                  deasserted the request, which
                                  released the reset

Result: the chip is not reset. It continues from a hung state with
        RSTREASON possibly clear. Invisible in RTL simulation.

## 9 Clock, reset and power

[N-9.1] Both counters and the comparator are in hclk. The APB interface is in pclk. Register values cross synchronously, no CDC.

[N-9.2] Reset domains:

| Element | Reset by | Why |
| --- | --- | --- |
| mtime, mtimecmp, shadow | hreset_n | Ordinary state. |
| wdt_ctr, WDTCTL, WDTLOAD, WDTWARN | hreset_n | The watchdog restarts disabled after any reset, so firmware must re-enable it deliberately. |
| wdt_rst_req_flop | ext_rst_n only | §7.6. |

[N-9.3] mtime resetting to 0 on a watchdog reset means firmware cannot measure how long it was hung. Accepted: preserving mtime across reset would need it in the ext_rst_n domain, 64 flops in a second reset domain, and the DSRAM diagnostic record from the early warning already gives the useful information.

[N-9.4] Power: neither counter can be clock gated in any useful way. mtime must increment every cycle by definition, and the watchdog must count while the core is asleep — indeed especially then, since a core that never wakes from WFI is exactly what the watchdog catches. 96 flops toggling continuously is the cost of having a timer.

[N-9.5] This block is therefore one of the few that stays fully active during WFI. The core's WFI clock gate does not extend here.

## 10 Assertions

// --- mtime is monotonic except on an explicit write
a_mtime_monotonic: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  !mtime_wr |=> (mtime == $past(mtime) + 1));

// --- mtip is exactly the comparison (R-3, N-7.8)
a_mtip_correct: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  mtip_o == (mtime_q >= mtimecmp_q));

// --- mtip is level, not a pulse: it holds until mtimecmp advances
a_mtip_level: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (mtip_o && !mtimecmp_wr) |=> mtip_o);

// --- mtimecmp resets to all-ones so mtip cannot assert at reset (N-6.1)
a_mtimecmp_reset: assert property (
  @(posedge hclk_i) !hreset_n_i |=> (mtimecmp_q == 64'hFFFF_FFFF_FFFF_FFFF && !mtip_o));

// --- coherent read: MTIME_HI returns the shadow, never the live value (R-4)
a_shadow_latch: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  (apb_read && apb_addr == 12'h000) |=> (shadow_q == $past(mtime_q[63:32])));
a_hi_returns_shadow: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  (apb_read && apb_addr == 12'h004) |-> (apb_rdata == shadow_q));

// --- THE watchdog domain property (R-8, N-7.19)
a_wdt_req_survives_hreset: assert property (
  @(posedge hclk_i) (wdt_rst_req_o && !hreset_n_i) |=> wdt_rst_req_o);
a_wdt_req_cleared_only_by_ctrl: assert property (
  @(posedge hclk_i) disable iff (!ext_rst_n_i)
  $fell(wdt_rst_req_o) |-> $past(wdt_req_clear));

// --- watchdog enable is sticky (R-9, N-6.2)
a_wdt_en_sticky: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i) $fell(wdt_en) |-> 1'b0);

// --- only the magic value kicks (N-6.5)
a_wdt_magic_only: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  (apb_write && apb_addr == 12'h01C && apb_wdata != 32'h5A5A_C3C3)
    |=> $stable(wdt_ctr));
a_wdt_magic_reloads: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  (apb_write && apb_addr == 12'h01C && apb_wdata == 32'h5A5A_C3C3)
    |=> ##[0:2] (wdt_ctr == wdt_load));

// --- the counter only runs when enabled
a_wdt_stopped: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i) !wdt_en |=> $stable(wdt_ctr));

// --- reset request only at zero
a_wdt_req_at_zero: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  $rose(wdt_rst_req_o) |-> ($past(wdt_ctr) == 32'd0));

// --- the warning precedes the reset (R-7)
a_wdt_warn_before_reset: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (wdt_en && wdt_warn_en && wdt_warn_q != 0) ##1 $rose(wdt_rst_req_o)
    |-> $past(wdt_warn_irq_o, 1, 1, wdt_warn_q));

// --- warning asserts at exactly the threshold
a_wdt_warn_at_thresh: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (wdt_en && wdt_warn_en) |-> (wdt_warn_irq_o == (wdt_ctr == wdt_warn_q)));

// --- no 64-bit timer bus leaves this block (R-3)
// structural, checked at integration review, not simulation

## 11 Verification plan

| Req | Test | Oracle | Coverage goal | Status |
| --- | --- | --- | --- | --- |
| R-1 | t_mtime_count | increments every cycle; 32-bit boundary crossing | low-word wrap | new |
| R-1 | t_mtime_write | write takes effect immediately | — | new |
| R-2 | t_mtimecmp_rw | both halves readable and writable | — | new |
| R-3 | t_mtip_single_wire | mtip reaches mip.MTIP; no CLIC ID involved | — | new |
| R-4 | t_mtime_coherent | read LO then HI across a carry returns the right value | carry at the exact read cycle | new |
| R-4 | t_mtime_hi_alone | reading HI alone returns the shadow, documented as such | — | new |
| R-5 | t_mtimecmp_seq | the three-step sequence produces no spurious mtip | — | new |
| R-5 | t_mtimecmp_naive | the naive one-step write does produce a spurious mtip, confirming why the sequence is required | — | new |
| R-6 | t_wdt_count | counts down, kicks reload, zero asserts the request | — | new |
| R-6 | t_wdt_magic | only 0x5A5A_C3C3 kicks; sweep of other values | ≥100 random values | new |
| R-7 | t_wdt_warn | warning fires at the threshold, before the reset | 3 threshold values | new |
| R-8 | t_wdt_req_survives | request holds through hreset_n; full-length reset results | — | critical, new |
| R-9 | t_wdt_no_disable | no write clears EN | — | new |
| §7.3 | t_mtip_late_write | mtimecmp set to a past value asserts mtip immediately | — | new |
| §7.7 | t_wdt_during_hartreset | watchdog fires while the core is debug-held, as documented | — | new |
| — | t_timers_1khz | full 1 kHz loop with the real reload value, 1000 iterations, no drift | — | new |

[N-11.1] t_wdt_req_survives is the most important test in this document. It is the regression guard for the runt-pulse class of bug described in §8.4, which RTL simulation would otherwise hide because reset is modelled as instantaneous. The test must check the duration of hreset_n, not merely that it asserted.

[N-11.2] t_mtimecmp_naive deliberately verifies that the wrong firmware sequence fails. This is unusual but valuable: it documents in executable form why [N-7.12] exists, so a future engineer who "simplifies" the driver sees a test fail rather than an intermittent spurious interrupt at 17-second intervals.

## 12 Design decisions

| Decision | ADR |
| --- | --- |
| Compare in this block; single mtip wire to the core; 64-bit buses and the in-core comparator deleted | ADR-0010 |
| CLIC ID 13 released | ADR-0010 |
| wdt_rst_req_flop in the ext_rst_n domain | ADR-0003 |

## 13 Not implemented

| Feature | Why not |
| --- | --- |
| mtime as a CLIC source | ADR-0010. The core's mip.MTIP path is implemented and verified. |
| A prescaler on mtime | It is a cycle counter. A prescaler adds a register and a frequency question for no benefit at 4 ns resolution. |
| A second general-purpose timer | The 1 kHz loop needs one deadline. Additional software timers are a firmware list keyed off mtime. |
| Timer capture or compare outputs | No input capture requirement; PWM owns the motor outputs. |
| Watchdog windowing (kick-too-early detection) | Catches a specific runaway-loop shape. Worth having in v2; not worth the register and the firmware-timing constraint now. |
| A watchdog freeze-on-debug bit | §7.24. Needs a DM connection, and a watchdog that stops for one reason will be stopped for another. |
| mtime preserved across reset | §9.3. 64 flops in a second reset domain for information the early-warning record already provides. |
| A software-clearable watchdog enable | §6.2. A watchdog the hung firmware can disable is worse than none. |

## 14 Open items

- OPEN-T1 — closed 2026-09-26. The APB side now runs on pclk, with the registers still on hclk as ADR-0002 Rev 2 asks. PRDATA is registered on pclk at the setup-to-access edge, so it is stable for the whole access phase: the 4 ns hclk-to-pclk hop still exists but is now local to the block, a few gates from the source flop, and what crosses to the bridge is pclk-to-pclk with a full period. tb_timers gained a permanent monitor that counts any PRDATA movement during an access phase; it reports 3 movements against the pre-migration RTL and 0 after, so the check discriminates rather than merely passing.

## 15 Errata

No RTL for this block yet. The core's mip.MTIP path is implemented and verified; its 64-bit comparator and ports are deleted by RTL delta R4.

Defects fixed in this revision:

| ID | Symptom | Root cause | Fix |
| --- | --- | --- | --- |
| TM-1 | Two delivery paths for one interrupt: mip.MTIP in the core RTL and CLIC ID 13 in three specs. | Routing decided independently in three documents. | ADR-0010; §7.3; ID 13 reserved. |
| TM-2 | 128 top-level wires and a 64-bit comparator in the core's critical timing cone, for one bit of information. | The comparison was placed at the consumer rather than the producer. | Comparator moves here; mtip_o is one wire. |
| TM-3 | Watchdog reset would be a runt pulse of roughly one hclk cycle, leaving the chip un-reset and possibly with a clear reason register. | The reset request flop was in the domain its own request reset. | §7.6; ext_rst_n domain; t_wdt_req_survives. |
| TM-4 | A watchdog reset gave no diagnostic information at all. | No early warning existed. | WDTWARN and CLIC ID 22, §6.4. |
| TM-5 | mtimecmp reset value of 0 would assert mtip immediately after reset, before any handler existed. | Reset value chosen as the default zero. | All-ones, §6.1. |