# GARUDA UART — Design Specification

## 0.1 Identity

| Field | Value |
| --- | --- |
| Document ID | GARUDA-UART-SPEC-001 |
| Revision | 1.0 |
| Date | 2026-09-23 |
| Status | Released for implementation |
| Blocks | 16 (uart0), 17 (uart1), 18 (uart2) — one design, three instances |
| Owner | Team AeroSoC |
| Supersedes | — (first revision; the blocks were sourced_ip with no document) |

## 0.2 Revision history

| Rev | Change | Driver |
| --- | --- | --- |
| 1.0 | First specification. An adaptation of pulp-platform/apb_uart_sv behind a GARUDA wrapper, not vendor IP. The interrupt model is GARUDA's, not the 16550's ([N-7.5]); the vendored RTL carries one recorded patch, for receive-error reporting and an inferred latch (§15). | D-21, D-22, D-23, D-24 |

## 0.3 Normative references

- GARUDA-SYS-001 Rev 4.0 — System Definition. All cross-block values come from it.

- GARUDA-AHB2APB-SPEC-001 Rev 2.0 — the APB contract this block must satisfy.

- GARUDA-DMA-SPEC-001 Rev 3.0 §7.3 — the request/acknowledge handshake.

- GARUDA-CLIC-SPEC-001 Rev 2.0 §7.2 — level interrupts.

- Docs/DECISIONS.md D-16, D-21, D-22, D-23, D-24 (why this IP is patched).

- GARUDA-SPIM-SPEC-001 Rev 1.0 — the first block built this way; the shared tail is identical.

## 1 Purpose and scope

### 1.1 In scope

Three independent asynchronous serial ports, one design instantiated three times. Each is 8-bit, one start bit, configurable parity and stop bits, with a 16-byte transmit and a 16-byte receive FIFO.

| Instance | Block | APB window | Base | CLIC ID | DMA ch | Pins | Intended use |
| --- | --- | --- | --- | --- | --- | --- | --- |
| uart0 | 16 | 3 | 0x4000_3000 | 17 | 2 | uart0_rx/tx | telemetry link |
| uart1 | 17 | 4 | 0x4000_4000 | 18 | 3 | uart1_rx/tx | GNSS receiver |
| uart2 | 18 | 6 | 0x4000_6000 | 19 | 5 | uart2_rx/tx | console |

### 1.2 Out of scope

- Modem control and flow control. GARUDA has no RTS/CTS/DTR/DSR pins (PHYS §3.1), so MCR/MSR exist but control nothing ([N-13.1]).

- Auto-baud detection, IrDA, RS-485 direction control, 9-bit/address mode.

- The console protocol and any firmware framing — firmware's business.

### 1.3 Where the RTL comes from

rtl/third_party/pulp/apb_uart_sv (Solderpad 0.51), behind rtl/uart/garuda_uart_top.v. The upstream block is a 16550-style register file over a plain start/data/parity/stop shifter pair with 16-entry FIFOs.

It is the one modified vendored IP in the design — see below and §15.

What had to be taken over, and why, is the substance of this document. Upstream's interrupt unit is not usable as it stands ([N-7.5], §15 ERR-U1), so GARUDA derives its interrupts from the Line Status Register instead — a wrapper change, in line with D-22.

One defect could not be handled in the wrapper: upstream cannot report a receive error at all (§15 ERR-U2), and the wrapper sees only the APB side and the raw rx pin, so detecting one there would mean reimplementing the receiver. R-7 is therefore met by a recorded patch to the vendored RTL — patches/0001-error-reporting-and-latch-fix.patch, governed by D-24, verified by tools/vendor_sync.py --check, and declared in Docs/THIRD_PARTY_NOTICES.md as Solderpad 0.51 requires.

The same patch also removes an inferred latch that has nothing to do with R-7 (§15 ERR-U3) — upstream left fifo_tx_data without a default, which cost 24 latches across the three instances. make synth found it; no simulation could have.

## 2 Requirements

| ID | Requirement | Source | Status |
| --- | --- | --- | --- |
| R-1 | Transmit and receive 8N1 frames at a programmable baud rate, byte-accurate in both directions. | SYS §6.4 | met |
| R-2 | 115200 baud from pclk = 125 MHz with ≤1% baud error. | TRM §3.5 | met |
| R-3 | Word-only APB: every register is 32 bits at a word-aligned offset, even though upstream is byte-addressed. | AHB2APB [N-7.12] | met |
| R-4 | Never stall the APB bus. | AHB2APB [N-7.15] | met |
| R-5 | One held, level interrupt per instance to its CLIC ID. | CLIC §7.2, D-17 | met |
| R-6 | DMA request/acknowledge on the instance's channel, one beat per assertion, word-sized beats carrying one byte in [7:0]. | DMA §7.3, D-21 | met |
| R-7 | Report parity and framing errors to firmware without losing received data. | SYS §6.4 | met, via vendored patch 0001 (D-24) |
| R-8 | tx high (line idle) from reset until firmware acts; rx synchronised before use. | Board safety, D-14 | met |
| R-9 | Three instances are independent: no shared register, interrupt, DMA channel or clock enable. | §1.1 | met |

## 3 Block diagram

APB window 3 / 4 / 6  (pclk 125 MHz)
        │
        ▼
  ┌──────────────────────── garuda_uart_top ────────────────────────┐
  │                                                                 │
  │  garuda_apb_shim ──── IP APB ────▶ apb_uart_sv (vendored)       │
  │   ADDR_SHIFT = 2      (byte addr)   ├─ uart_tx  ─┐              │
  │   0x00-0x1C  pass-through           ├─ uart_rx  ─┤ 16-byte      │
  │   0xFE0-0xFEC  IRQSTAT/IRQEN/       ├─ io_generic_fifo x2       │
  │                DMACTL/ID            └─ uart_interrupt (UNUSED)  │
  │                                              │                  │
  │  LSR poll (idle cycles) ──▶ lsr_q ──▶ evt[2:0], dma_req         │
  │  LCR[7] write snoop     ──▶ dlab_q ──▶ gates dma_req            │
  │  rx ──▶ 2-flop sync                          ▼                  │
  └──────────────────────────────────────────── tx, rx ─────────────┘

## 4 Sub-block inventory

| Name | Type | RTL file | Role |
| --- | --- | --- | --- |
| garuda_apb_shim | seq (pclk) | rtl/common/garuda_apb_shim.v | word decode, PSLVERR, sticky IRQ, DMA request, pad sync |
| apb_uart_sv | seq (pclk) | rtl/third_party/pulp/apb_uart_sv/src/ | 16550 register file + FIFOs (vendored) |
| uart_tx, uart_rx | seq | same | shifters and baud counters |
| io_generic_fifo | seq | same | 16-byte TX and RX FIFOs |
| uart_interrupt | seq | same | instantiated but its output is unused ([N-7.5]) |
| garuda_uart_top | wrapper | rtl/uart/garuda_uart_top.v | address shift, LSR poll, DLAB snoop, event derivation |

## 5 Interfaces

| Port | Dir | Width | Domain | Reset | Description |
| --- | --- | --- | --- | --- | --- |
| pclk_i, preset_n_i | in | 1 | pclk | — | APB domain, 125 MHz |
| APB slave | — | — | pclk | — | window 3, 4 or 6 |
| irq_o | out | 1 | pclk | 0 | to CLIC ID 17/18/19, level |
| dma_req_o | out | 1 | pclk | 0 | DMA channel 2/3/5, level |
| dma_ack_i | in | 1 | pclk | — | 2 hclk = 1 pclk (D-16) |
| uart_tx_o | out | 1 | pclk | 1 | line idle is high |
| uart_rx_i | in | 1 | async | — | synchronised in the wrapper ([N-9.2]) |

BLOCK_NUM is a parameter: 16, 17 or 18, so ID distinguishes the three instances at run time.

## 6 Register map — one 4 KiB window per instance

No machine-readable register description exists for this block yet; this table is the register map. Offsets 0x00–0x1C are the vendored 16550 register file at word offsets; 0xFE0–0xFEC are the GARUDA tail common to every peripheral (D-21).

| Offset | Name (DLAB=0) | Name (DLAB=1) | Access | Reset | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | RBR / THR | DLL | R / W | 0 | receive byte [7:0] / transmit byte [7:0] |
| 0x04 | IER | DLM | RW | 0 | upstream interrupt enables — leave 0 ([N-7.5]) |
| 0x08 | IIR (R) / FCR (W) | — | R / W | 0 | upstream interrupt ID / FIFO control |
| 0x0C | LCR | — | RW | 0 | line control: word length, stop bits, parity, DLAB |
| 0x10 | MCR | — | — | 0 | not implemented ([N-13.1]) |
| 0x14 | LSR | — | RO | 0x60 | line status ([N-6.3]) — [3] exists only because of patch 0001 |
| 0x18 | MSR | — | — | 0 | not implemented ([N-13.1]) |
| 0x1C | SCR | — | — | 0 | not implemented ([N-13.2]) |
| 0xFE0 | IRQSTAT |  | W1C | 0 | [0] RX data available, [1] TX holding empty, [2] line error |
| 0xFE4 | IRQEN |  | RW | 0 | mask; irq_o = ¦(IRQSTAT & IRQEN) |
| 0xFE8 | DMACTL |  | RW | 0 | [0] request on RX data, [1] request on TX space |
| 0xFEC | ID |  | RO | — | {16'h6A5D, BLOCK_NUM, rev} |

### 6.1 Word offsets over a byte-addressed IP

[N-6.1] Upstream decodes register_adr = PADDR[2:0] — eight registers at byte offsets 0–7. GARUDA's bridge rejects sub-word accesses before they reach a peripheral (AHB2APB [N-7.12]), so a byte-addressed register file cannot be reached at all. The shim is configured with ADDR_SHIFT = 2, which presents our word offset 4n to the IP as byte offset n. Nothing else changes: the register contents are the 16550's.

[N-6.2] PRDATA[31:8] reads 0, not X: upstream zero-extends every read (PRDATA = {24'b0, …}). Firmware may read a whole word and mask.

### 6.2 LCR (0x0C) and the DLAB discipline

[N-6.2a] LCR[7] is the Divisor Latch Access Bit. While it is set, offsets 0x00 and 0x04 address the baud divisor (DLL, DLM) instead of the data and interrupt-enable registers. This is the 16550's original sin and it is preserved here because the register file is.

Rule: firmware sets DLAB only to program the divisor and clears it immediately afterwards. The hardware enforces the dangerous half of that: the wrapper snoops writes to LCR and holds a shadow of LCR[7], and dma_req_o is forced low while the shadow is set ([N-7.6]). A DMA transfer cannot therefore be pointed at the divisor latch by a driver that forgot to clear DLAB — the request simply never comes, which is a stall a developer can see, rather than a baud rate silently overwritten with payload bytes.

### 6.3 LSR (0x14)

[N-6.3] Bits GARUDA relies on. Bits 0, 5 and 6 are levels recomputed every pclk from FIFO occupancy; bits 2 and 3 describe the byte currently at the head of the RX FIFO and travel with it:

| Bit | Name | Meaning |
| --- | --- | --- |
| 0 | DR | RX FIFO is not empty — a byte can be read |
| 2 | PE | parity error on the byte currently at the head of the RX FIFO |
| 3 | FE | framing error (stop bit low) on that same byte |
| 5 | THRE | TX FIFO is empty — bytes can be written |
| 6 | TEMT | TX FIFO and the shift register are empty — the line is idle |

[N-6.3a] Reading LSR has no effect on data. Upstream's read path asserts clr_int into uart_interrupt and nothing else — it does not pop a FIFO and does not alter regs_q[LSR]. This is what makes the wrapper's idle-cycle poll ([N-7.4]) safe, and it was verified by reading the upstream read mux, not by assuming (D-23).

[N-6.3b] TEMT (bit 6), not THRE (bit 5), is the bit to poll before cutting power or changing the baud rate. THRE goes high as soon as the last byte leaves the FIFO for the shift register, up to ten bit-times before the stop bit is actually on the wire.

## 7 Functional description

### 7.1 Baud rate

[N-7.1] The divisor is a plain per-bit counter — there is no 16× oversampling:

baud = pclk / (divisor + 1)          divisor = {DLM, DLL}, 16 bits
divisor = (pclk / baud) - 1

At pclk = 125 MHz:

| Baud | divisor | actual | error |
| --- | --- | --- | --- |
| 115200 | 1084 | 115207 | +0.006% |
| 57600 | 2169 | 57603 | +0.005% |
| 9600 | 13020 | 9600.0 | +0.000% |
| 921600 | 134 | 925926 | +0.47% |

R-2 is met with room to spare. DIVSEL (CLKRST §6.2) changes pclk and therefore every baud rate: firmware that changes the chip frequency must re-program DLL/DLM on all three instances.

### 7.2 Sending a byte

[N-7.2]

LCR  = 0x83            DLAB=1, 8 bits, 1 stop, no parity
DLL  = 1084 & 0xFF     divisor low
DLM  = 1084 >> 8       divisor high
LCR  = 0x03            DLAB=0 - do not skip this ([N-6.2a])
poll LSR until [5]     THRE: the TX FIFO has room
THR  = byte

### 7.3 Receiving a byte

[N-7.3]

poll LSR until [0]     DR: a byte is waiting
byte = RBR & 0xFF      reading RBR pops the FIFO

Read LSR before RBR, never after. LSR[2] (parity error) describes the byte at the head of the FIFO — the one the next RBR read will return. Read RBR first and the error bit you then read belongs to the following byte.

### 7.4 How the wrapper knows what the FIFOs are doing

[N-7.4] Upstream exposes tx_elements and rx_elements only as internal signals — they are not ports. The wrapper needs them as levels for both the interrupt and the DMA request, so, exactly as the SPI master does with its STATUS register (SPIM [N-7.4]), the wrapper — the only APB master into the IP — reads LSR in the cycles when the SoC is not accessing this window and latches the result. No upstream change, no hierarchical reference, and the status is never more than one pclk stale.

This is sound only because of [N-6.3a]: an LSR read has no data side effect. The one thing it does do — pulse clr_int into uart_interrupt — is harmless here precisely because that unit's output is not used ([N-7.5]).

### 7.5 Interrupts: GARUDA's model, not the 16550's

[N-7.5] uart_interrupt's output is left unconnected, and firmware must leave IER (offset 0x04) at 0. The block's interrupt is built in the wrapper from the LSR levels of [N-7.4] and delivered through the standard sticky tail (D-21):

| IRQSTAT bit | Set by | Cleared by |
| --- | --- | --- |
| 0 | LSR[0] — RX data available | W1C, and it re-arms while data remains |
| 1 | LSR[5] — TX holding register empty | W1C |
| 2 | LSR[2] ¦ LSR[3] — parity or framing error at the FIFO head | W1C |

[N-7.5a] Bit 2 is one "line error" event for both causes: the interrupt only has to say the byte at the head of the FIFO is suspect. Firmware reads LSR to tell them apart, and the distinction is worth having — a parity error means noise on the line, a framing error means the baud rate is wrong.

irq_o = |(IRQSTAT & IRQEN), held until firmware writes IRQSTAT — which is the CLIC's requirement (D-17) and is not what upstream provides.

Why, and this is a correctness matter, not a style preference: upstream wires the receiver-data-available input of its interrupt unit to the wrong LSR bit — .RDA_i(regs_n[LSR][5]), which is THRE, the transmit FIFO empty flag, where regs_n[LSR][0] (DR) was intended. With IER[0] set, a 16550 driver would take a "received data available" interrupt whenever the transmitter drained, and IIR would report 0b1000 for it. The only remaining route to a genuine RX interrupt is trigger_level_reached, which upstream computes with == rather than >=, so it is missed unless the occupancy is sampled at exactly the trigger value, and CTI_i (character timeout) is tied to zero, so a part-full FIFO below the trigger level never raises anything.

Recorded as ERR-U1 in §15 and in Docs/BUGS.md. Deriving our own events from LSR costs about fifteen lines in the wrapper, needs no patch to third-party RTL, and gives firmware the same interrupt model as every other GARUDA peripheral. That is the trade D-22 exists to make.

### 7.6 DMA

[N-7.6] DMACTL[0] requests a beat while LSR[0] is set (a byte is waiting); DMACTL[1] requests while LSR[5] is set (the TX FIFO has room). The request drops for at least one pclk after every dma_ack_i — the shim's hold-off — because dma_chan.v takes one beat per assertion and a request that stays high deadlocks the channel after the first beat (D-16, D-21).

[N-7.6a] Beats are word-sized; the byte lives in [7:0] and [31:8] reads 0. Firmware sets the DMA channel's SIZE to word and the peripheral address to this window's offset 0x00.

[N-7.6b] dma_req_o is forced low while the DLAB shadow is set ([N-6.2a]).

## 8 Timing

### 8.1 One 8N1 frame at 115200 baud

│← start →│← b0 →│ … │← b7 →│← stop →│
tx   ───┐         ┌──────┐   ┌──────┐        ┌────────
        └─────────┘      └───┘      └────────┘
         8.68 us each, 10 bits = 86.8 us per byte

A 16-byte FIFO therefore holds 1.39 ms of traffic at 115200 baud — the budget firmware has to service an RX interrupt before an overrun.

### 8.2 Where the receiver samples

[N-8.1] Upstream samples each bit at its centre. On the falling edge of the start bit the baud counter runs to cfg_div_i[15:1] — a half bit period — rather than the full cfg_div_i it uses for every later bit:

if      (!start_bit && (baud_cnt == cfg_div_i))          // data bits: full
else if ( start_bit && (baud_cnt == {1'b0,cfg_div_i[15:1]}))   // start: half

That half-bit step is what aligns every subsequent sample to a bit centre. Measured in tb_uart, with the DUT at divisor 124 the samples land 1.54, 2.54, 3.54 … bit periods after the falling edge, against ideal centres of 1.5, 2.5, 3.5 — the 0.04 bit of lag is the pad synchronisers, and it is the same three flops that delayed the edge detection, so it very nearly cancels.

Measured receiver baud tolerance: at least ±6%, which is the full range t_uart_baud_tol sweeps; the receiver did not fail at either end, so the true figure is wider. 115200 baud against a partner with a ±2% crystal-less clock is comfortable.

(Rev 1.0 draft of this section asserted bit-boundary sampling and a tolerance "smaller than ±5%". That was wrong — it read the data-bit branch of the baud counter and missed the start-bit branch. The measurement above replaces it, and D-23's rule earned its keep twice: the first reading of upstream was wrong, and only running real bytes through it settled the question.)

### 8.3 DMA request hold-off

As SPIM §8.2 — identical mechanism, identical timing.

## 9 Clock, reset and power

[N-9.1] Single clock domain: pclk, 125 MHz, from preset_n. The IP is fully synchronous to it; there is no CDC inside the block.

[N-9.2] uart_rx_i is a pad input asynchronous to pclk and is passed through the shim's two-flop synchroniser before it reaches the IP. Upstream adds three more flops of its own (reg_rx_sync), which is harmless — five flops of latency on a signal whose bit period is 1085 clocks.

[N-9.3] uart_tx_o is high out of reset (R-8). Upstream resets its transmitter to the idle line state, and the wrapper adds nothing: a UART line held low is a continuous break condition to whatever is on the other end.

The receive side needs the same care and did not get it for free. The shim's pad synchronisers reset to SYNC_RESET, and this block passes SYNC_RESET = 1 because an idle serial line is high. With the shim's original reset-to-zero, the receiver saw a low rx for two pclk out of reset, framed that as a start bit, and left a garbage byte at the head of the RX FIFO — after which every read returned the previous byte, for ever. It cost an hour and it would have looked like an off-by-one in the driver on a board. t_uart_rxidle covers it.

## 10 Assertions

// PREADY is never low - the 16-pclk bridge timeout would fault the access
a_pready: assert property (@(posedge pclk_i) disable iff (!preset_n_i)
            psel_i |-> pready_o);

// the interrupt is a level: it may only fall on an IRQSTAT write
a_irq_held: assert property (@(posedge pclk_i) disable iff (!preset_n_i)
              $fell(irq_o) |-> $past(irqstat_write));

// one beat per request (D-16)
a_dma_drop: assert property (@(posedge pclk_i) disable iff (!preset_n_i)
              dma_ack_i |=> !dma_req_o);

// the divisor latch is never a DMA target
a_dma_dlab: assert property (@(posedge pclk_i) disable iff (!preset_n_i)
              dlab_q |-> !dma_req_o);

## 11 Verification plan

make test_uart — tb/uart/tb_uart.sv against tb/models/uart_model.sv, 53 checks, 0 failures.

| Req | Test | Oracle | Status |
| --- | --- | --- | --- |
| R-1 | t_uart_tx_byte | the model decodes what the DUT transmits | pass |
| R-1 | t_uart_rx_byte | the DUT receives what the model transmits | pass |
| R-1 | t_uart_loopback | 256 bytes both ways, no loss or reorder | pass |
| R-2 | t_uart_baud | measured bit period at divisor 1084 is 8.68 µs ±1% | pass |
| — | t_uart_baud_tol | measures the mismatch the receiver tolerates: at least ±6% ([N-8.1]) | pass |
| R-3 | t_uart_wordmap | each word offset reaches the intended 16550 register | pass |
| R-3 | t_uart_dlab | 0x00/0x04 bank correctly, and PRDATA[31:8] is 0 | pass |
| R-4 | t_uart_pready | PREADY high in every cycle of every access, FIFO full and empty | pass |
| R-5 | t_uart_irq | held ≥50 pclk, drops only on W1C | pass |
| R-6 | t_uart_dma | dma_req_checker; and no request while DLAB is set | pass |
| R-1 | t_uart_parity | a clean 8E1 frame carries data and raises no error | pass |
| R-7 | t_uart_parity_err | a wrong-parity frame sets LSR[2] and IRQSTAT[2], and still delivers the byte | pass |
| R-7 | t_uart_frame_err | a low stop bit sets LSR[3] and the same event | pass |
| R-7 | t_uart_err_noleak | neither flag leaks into the following byte | pass |
| — | t_uart_b2b | 16 frames with no inter-frame gap — the timing patch 0001 changes ([N-8.2]) | pass |
| R-8 | t_uart_reset | tx high out of reset | pass |
| R-9 | t_chip_uart | three instances, three windows, three CLIC IDs, no crosstalk | new |
| — | t_uart_rxidle | rx synchroniser resets high, so no garbage byte at power-on ([N-9.3]) | pass |

The test that earns its keep is t_uart_loopback. As D-23 records, a register-level test suite passes happily with the data path connected to the wrong pin. Only bytes compared end to end catch that.

## 12 Design decisions

| # | Decision | Alternative rejected |
| --- | --- | --- |
| 1 | One module, three instances, BLOCK_NUM parameterised | three copies — three places for a fix to be missed |
| 2 | Interrupts derived in the wrapper from LSR | using upstream's event_o — it is wired to the wrong bit (ERR-U1) |
| 3 | LSR polled in idle cycles for FIFO state | patching upstream to export rx_elements/tx_elements — D-22 prefers an empty patches/ |
| 4 | Hardware gates DMA while DLAB is set | documenting the rule and trusting the driver |
| 5 | ADDR_SHIFT = 2 in the shim | re-implementing the register file at word offsets |

## 13 Not implemented

[N-13.1] MCR (0x10) and MSR (0x18) are not implemented at all: upstream's write case list is THR/IER/LCR/FCR and its read mux covers RBR/LSR/LCR/IER/IIR; everything else falls to default. They decode without PSLVERR and read 0. No modem-control pin exists on GARUDA anyway (PHYS §3.1), so firmware must not use hardware flow control. t_uart_wordmap asserts this, so a re-vendor that implements them fails the test rather than changing behaviour quietly.

[N-13.2] SCR (0x1C), the 16550 scratch register, is likewise not implemented — writes are dropped and reads return 0. Do not use it as a scratchpad; it is not one.

[N-13.2a] Upstream's FCR trigger-level field is writable and feeds uart_interrupt, which GARUDA does not use ([N-7.5]). It therefore has no observable effect. It is left writable rather than blocked so that the vendored register file is untouched.

[N-13.3] No break generation or detection, no auto-baud, no RS-485 direction pin, no 9-bit address mode.

## 14 Open items

- OPEN-U1 — closed 2026-09-23. Measured at at least ±6% ([N-8.1]), the full range the sweep covers; the receiver did not fail at either end. No board-level constraint is needed.

- OPEN-U3 — closed 2026-09-23. R-7 is met by vendored patch 0001 (D-24). The recommendation at the time was to accept the gap and rely on protocol checksums; that was overruled by the owner, correctly — a checksum tells firmware to drop a packet, it does not say why, and error counters are how a flaky connector is told from EMI in the field.

- OPEN-U2 — no RX FIFO overrun status. Upstream's LSR[1] (overrun) is never written by the register file, so a byte lost to a full FIFO is silent. Options: derive an overrun event in the wrapper from a write to a full FIFO, or accept it and rely on the DMA keeping the FIFO drained. Decide before the telemetry link is brought up at 921600.

## 15 Errata

ERR-U1 — upstream's receiver-data-available interrupt is wired to the transmit flag. apb_uart.sv connects .RDA_i(regs_n[LSR][5]) (THRE) where regs_n[LSR][0] (DR) belongs. Compounded by trigger_level_reached using == instead of >= and CTI_i being tied to zero, the upstream interrupt path cannot reliably signal received data at all.

Not fixed in the vendored file (D-22: upstream is never edited). GARUDA does not use uart_interrupt; the wrapper derives all three events from LSR ([N-7.5]). Firmware must leave IER at 0. Recorded in Docs/BUGS.md.

If this block is ever re-vendored from a newer upstream, re-check this: if upstream has fixed it, the wrapper still works unchanged, because it does not depend on the unit either way.

ERR-U2 — parity errors could never be reported, for two independent reasons. FIXED by vendored patch 0001 (D-24). Recorded here because a re-vendor must re-apply or re-derive it.

- apb_uart.sv instantiated the receiver with .err_clr_i(1'b1). In uart_rx the error flop is if (err_clr_i) err_o <= 0; else if (set_error) err_o <= 1; — with the clear tied high the set_error branch was unreachable and err_o a constant 0.

- The ordering was wrong regardless. uart_rx pushed the byte to the RX FIFO in SAVE_DATA with data_i = {parity_error, rx_data}, and only then advanced to PARITY to check the bit, so the flag stored beside a byte belonged to the previous frame.

LSR[2] is driven from that FIFO bit, so it never set. Framing errors were not detected at all — the stop bit was never examined — and apb_uart.sv never drove LSR[3].

Why this one was patched and ERR-U1 was not. ERR-U1 is a miswired input whose effect the wrapper can simply decline to use. ERR-U2 cannot be worked around from outside: the wrapper sees the APB side and the raw rx pin, and nothing else, so detecting a parity error there would mean reimplementing the receiver. That is the unavoidable case D-22 kept patches/ for, and D-24 records the ruling.

What the patch changes, and the risk it carries. The push moves into the STOP_BIT bit_done cycle — after both checks, but not into a later state, because s_rx_fall is true for exactly one clock and an extra state makes the receiver miss the start bit of a frame that follows with no gap. The error outputs are combinational (flop OR set-strobe) so they are valid in the cycle they are captured. SAVE_DATA survives for the FIFO-full case only.

The consequence worth stating plainly: we are the only people who have run this receiver. t_uart_b2b — sixteen frames with no inter-frame gap — exists for that reason and for no other. It is the test that would fail if the timing argument above is wrong.

ERR-U3 — an inferred latch, 24 of them. apb_uart.sv gave fifo_tx_data no default in the register-write always_comb, assigning it only inside the THR branch, so synthesis inferred an 8-bit latch per instance. FIXED by the same patch 0001. Functionally harmless — the TX FIFO samples the bus only when fifo_tx_valid is high, which is exactly when that branch assigns it — but latches break scan insertion and must be constrained by hand at STA.

Found by make synth, which reported 25 latches against a budget of 1. No simulation would ever have found it, which is the argument for running synthesis as each block lands rather than once at the end.

(During bring-up t_uart_b2b did fail, 1 frame of 16. The cause was a testbench bug — both fork branches incremented the same module-level loop index — not the patch. Worth recording: the first instinct was to blame the new RTL, and that instinct would have led to "fixing" working logic.)