# GARUDA Block Design Specification — Template

GARUDA-DOC-001 Rev 1.0. Every block spec in this project has exactly these sections, in this order, with these numbers. A reviewer, a new engineer and a script all know where to look, and a missing section is visible as a gap rather than hidden in prose.

Status of the tooling this template names (2026-10-14). gen/tables.md, tools/check_specs.py and spec/regs/<block>.yaml were planned and do not exist. What exists: Design_Docs/garuda_system.yaml with tools/garuda_gen.py (address map, windows, CLIC IDs and DMA channels into rtl/include/garuda_map.vh and sw/common/garuda_map.h; --check fails on drift); SystemRDL register descriptions in Design_Docs/regs/ for the CLIC, clock/reset, GPIO, PWM and timers, written from each spec's §6 and used to generate the UVM register models (tools/gen/gen_ral.py); and tools/check_docs.py (make check_docs) for .docx exports. Rules 1 and the register generation in "Machine-readable sources" below are therefore targets, not checks.

## Rules that apply to every spec

- No cross-block facts are written by hand. Frequencies, addresses, block numbers, CLIC IDs, DMA channels, pins and APB windows come from GARUDA-SYS-001 via {{include: gen/tables.md#T-n}}. If a value appears literally in a spec's prose, that is a defect, and tools/check_specs.py fails on it.

- No edit history in the body. No "(was 200 MHz)", no "corrected from". History lives in §0.2 and in the ADR that made the change. The body states only what is true now.

- Every normative statement is numbered and testable. [N-3.2] style. Section 11's verification matrix references those numbers, so an untested requirement is visible.

- Every design choice points at an ADR. If a choice has no ADR, either it is not a choice (it follows from a requirement) or the ADR is missing.

- Timing is drawn, not described. Any multi-cycle protocol needs a waveform. Blocks whose specs described handshakes in prose instead of waveforms are where I-1 and D-1 came from.

- Assertions are in the spec, in SVA. §10 holds the assertions that the testbench binds. The spec is the source; the testbench imports it.

## Section structure

| § | Title | Content |
| --- | --- | --- |
| 0.1 | Identity | Doc ID, revision, date, status, owner, block number (generated) |
| 0.2 | Revision history | One row per revision: what changed and which ADR drove it |
| 0.3 | Normative references | With revisions. GARUDA-SYS-001 is always first |
| 1 | Purpose and scope | What the block does, what it explicitly does not |
| 2 | Requirements | [R-n] numbered. Where each comes from (system need, protocol, ADR) |
| 3 | Block diagram | Boundary, sub-blocks, every port on the boundary |
| 4 | Sub-block inventory | Table: name, seq/comb, RTL file, one-line role |
| 5 | Interfaces | Every port: name, direction, width, domain, reset value, description |
| 6 | Register map | Offset, name, width, access, reset value, field-by-field. Machine-readable source in spec/regs/<block>.yaml |
| 7 | Functional description | [N-n] normative statements. State machines as explicit state tables |
| 8 | Timing | Waveforms for every protocol phase, wait state, back-to-back and error case |
| 9 | Clock, reset and power | Domains, CDC (or a statement that there is none and why), reset domain, reset values |
| 10 | Assertions | SVA the testbench binds. Each maps to a [N-n] |
| 11 | Verification plan | Matrix: requirement → test name → oracle → coverage goal → status |
| 12 | Design decisions | Table of ADR references. No rationale restated, only the pointer |
| 13 | Not implemented | What was considered and excluded, with why. Prevents re-litigation |
| 14 | Open items | OPEN-n with owner and what unblocks it. Empty is a valid section |
| 15 | Errata | Fixed bugs: symptom, root cause, fix, test that catches a regression |

## Machine readability

Each spec ships three machine-readable companions, and the prose is generated from or checked against them:

- spec/regs/<block>.yaml — the register map. Generates the §6 table, the RTL register bank's field decode, sw/common/<block>_regs.h, and a UVM-style register model for the testbench. A register that exists in RTL but not here fails CI.

- spec/ports/<block>.yaml — the port list. Generates the §5 table and an instantiation template, and is checked against the RTL module header, which is how the dma_top naming mismatch and the clic_mintthresh_o ghost port would have been caught.

- spec/verif/<block>.yaml — the §11 matrix. Test names are checked to exist in the Makefile, so a requirement cannot silently have no test.

## Human readability

- A reader who needs one fact reads one table. A reader who needs the design reads §1, §3, §7 in order and nothing else.

- Rationale is in §12 by pointer, never inline. The old specs buried decisions in paragraphs of narrative, which is why the same decision got made three different ways in three documents.

- No section exceeds two screens without a table or a waveform breaking it up.