# GARUDA AHB-to-APB Bridge — Design Specification

## 0.1 Identity

| Field | Value |
| --- | --- |
| Document ID | GARUDA-AHB2APB-SPEC-001 |
| Revision | 2.0 |
| Date | 2026-09-18 |
| Status | Released for implementation |
| Blocks | 8 (ahb2apb), 7 (apb_fabric, generated inside block 8) |
| Owner | Team AeroSoC |
| Supersedes | GARUDA_AHB2APB_Bridge_Design_Spec_v1_4 |

## 0.2 Revision history

| Rev | Change | Driver |
| --- | --- | --- |
| 1.0–1.4 | Bridge with pclk generated by an ICG at 125 MHz, claimed 50% duty and "no CDC" | — |
| 2.0 | pclk generation moves to clk_div as a toggle flop (an ICG cannot produce 50% duty). The no-CDC claim is replaced with a proof. Per-window APB_DIV. Window map regenerated from GARUDA-SYS-001 after the peripheral reassignment. | ADR-0002, 0008, 0020 |
| 2.0a | [N-7.20] states the single PSLVERR rule: only for an offset that is not a register; a write to a read-only register is ignored (Track A task 0.5, 2026-10-12). | AUD-13 |

## 0.3 Normative references

- AMBA 3 APB Protocol Specification, ARM IHI 0024B.

- AMBA 3 AHB-Lite Protocol Specification, ARM IHI 0033A.

- GARUDA-SYS-001 Rev 4.0 — System Definition. Window map and APB_DIV defaults generated from it.

- GARUDA-ADR-001 Rev 1.0 — Architecture Decision Record.

- GARUDA-CLKRST-SPEC-001 Rev 2.0 — pclk generation and the phase indicator. The no-CDC argument lives there; this document consumes it.

- GARUDA-AHB-SPEC-001 Rev 4.0 — the AHB side.

## 1 Purpose and scope

### 1.1 In scope

One AHB-Lite slave that converts AHB transfers in the 0x4000_0000 region into APB3 transfers on twelve peripheral windows (APB-3: was "eleven"), and returns the response. Includes the window decode, the AHB↔APB state machine, the per-window access-rate divider, sub-word rejection, and the peripheral timeout.

### 1.2 Out of scope

- pclk generation — GARUDA-CLKRST-SPEC-001 §7.1.

- The peripherals themselves.

- The DMA's own APB config port, which is a peripheral on window 4 like any other.

### 1.3 What Rev 1.4 got wrong

Two things, and they are worth stating because both were internally contradictory rather than merely suboptimal.

The clock. Rev 1.4 generated pclk by passing every second hclk pulse through an integrated clock gate, and specified the result as 125 MHz with 50% duty. A gate that passes alternate pulses of a 4 ns clock produces a 2 ns high time in an 8 ns period — 25% duty. The stated mechanism cannot produce the stated waveform. A 25%-duty clock halves the usable setup window for anything sensitive to the low phase and distorts hold margins throughout the peripheral domain.

The crossing. Rev 1.4 asserted "no CDC is required" and gave no reason, while GARUDA_DMA_Controller_Design_Spec_v2_2 §15.3 implemented gray-code counters, a toggle handshake and level synchronisers against that same pclk. Both documents cannot be right. The conclusion of Rev 1.4 was correct and its reasoning absent; the DMA's implementation was solving a problem that does not exist. §7.2 of this document supplies the proof, and GARUDA-DMA-SPEC-001 Rev 3.0 deletes the synchronisers.

## 2 Requirements

| ID | Requirement | Source |
| --- | --- | --- |
| R-1 | Convert AHB-Lite transfers in the APB region to APB3 transfers. | Protocol |
| R-2 | Decode twelve 4 KiB windows (0-11; 12-15 unmapped). | ADR-0008, 0020 |
| R-3 | Peripherals shall be clocked at 125 MHz, not 250 MHz, to halve peripheral-domain dynamic power. | ADR-0002 |
| R-4 | No clock-domain-crossing synchronisers shall be required between hclk and pclk, and the reason shall be stated as a checkable property rather than asserted. | ADR-0002 |
| R-5 | The APB access rate shall be a per-window parameter, so an IP with a lower maximum frequency can be accommodated without a second clock. | ADR-0002 |
| R-6 | Byte and halfword accesses shall be rejected with an error, not silently widened. | System |
| R-7 | A peripheral that never asserts PREADY shall not hang the bus. | System |
| R-8 | An access to an unmapped window offset shall return an error. | System |

## 3 Block diagram

AHB-Lite shared layer
                            │ hsel_apb
                            ▼
   ┌──────────────────────────────────────────────────────────┐
   │                      ahb2apb  (block 8)                  │
   │                                                          │
   │   ── hclk domain ──────────┼── pclk domain ──────────    │
   │                            │                             │
   │  ┌──────────────┐          │      ┌──────────────────┐   │
   │  │ ahb_slave_if │          │      │  apb_fsm         │   │
   │  │              │          │      │  IDLE→SETUP→     │   │
   │  │ addr/ctl     ├──────────┼─────▶│  ACCESS→DONE     │   │
   │  │ capture      │  pclk_   │      │                  │   │
   │  │              │  phase   │      │  per-window      │   │
   │  │ hreadyout    │◀─────────┼──────┤  APB_DIV rate    │   │
   │  │ hresp        │          │      └────┬─────────────┘   │
   │  └──────────────┘          │           │                 │
   │         │                  │           ▼                 │
   │  ┌──────▼───────┐          │      ┌──────────────────┐   │
   │  │ win_decode   │          │      │ timeout_ctr      │   │
   │  │ haddr[15:12] │          │      │ 16 pclk cycles   │   │
   │  └──────┬───────┘          │      └──────────────────┘   │
   │         │                  │           │                 │
   │  ┌──────▼───────┐          │           ▼                 │
   │  │ size_check   │          │      ┌──────────────────┐   │
   │  │ word only    │          │      │ apb_fabric (7)   │   │
   │  └──────────────┘          │      │ PSEL[10:0] fanout│   │
   │                            │      └──┬──┬──┬─────┬───┘   │
   └────────────────────────────┼─────────┼──┼──┼─────┼───────┘
                                          ▼  ▼  ▼     ▼
                                       W0 W1 W2 ... W10
                                       (11 peripheral windows)

   The dashed boundary is a frequency boundary, not a CDC boundary: see §7.2.

## 4 Sub-block inventory

| Name | Type | RTL file | Role |
| --- | --- | --- | --- |
| ahb_slave_if | seq (hclk) | ahb2apb_top.v | Captures address/control, drives hreadyout/hresp. |
| win_decode | comb | ahb2apb_top.v | haddr[15:12] → PSEL one-hot; flags unmapped. |
| size_check | comb | ahb2apb_top.v | Rejects non-word hsize. |
| apb_fsm | seq (pclk) | apb_fsm.v | IDLE / SETUP / ACCESS / DONE. |
| rate_div | seq (pclk) | apb_fsm.v | Per-window APB_DIV stretching of the ACCESS phase. |
| timeout_ctr | seq (pclk) | apb_fsm.v | 16-cycle peripheral timeout. |
| apb_fabric | comb | apb_fabric.v | PSEL fanout, PRDATA/PREADY/PSLVERR collection. Block 7. |

## 5 Interfaces

### 5.1 AHB slave side (hclk)

| Port | Dir | Width | Reset | Description |
| --- | --- | --- | --- | --- |
| hclk_i, hreset_n_i | in | 1 | — |  |
| pclk_i, preset_n_i | in | 1 | — |  |
| pclk_phase_i | in | 1 | 0 | From clk_div. High on the hclk cycle preceding each pclk rising edge. |
| hsel_i | in | 1 | 0 |  |
| haddr_i | in | 32 | — |  |
| htrans_i | in | 2 | — |  |
| hwrite_i, hsize_i | in | 1/3 | — |  |
| hwdata_i | in | 32 | — |  |
| hrdata_o | out | 32 | 0 |  |
| hreadyout_o | out | 1 | 0 | See [N-9.2]. |
| hresp_o | out | 1 | 0 |  |

### 5.2 APB master side (pclk)

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| psel_o | out | 11 | One-hot per window. |
| penable_o | out | 1 | Shared. |
| paddr_o | out | 12 | Word offset within the window. |
| pwrite_o | out | 1 |  |
| pwdata_o | out | 32 |  |
| prdata_i | in | 32×11 |  |
| pready_i | in | 11 |  |
| pslverr_i | in | 11 |  |

### 5.3 Parameters

| Parameter | Width | Default | Description |
| --- | --- | --- | --- |
| APB_DIV[10:0][1:0] | 2 per window | 2'b00 (÷1) | Additional ACCESS-phase stretching per window. 00=÷1 (125 MHz), 01=÷2 (62.5 MHz), 10=÷4, 11=÷8. |

[N-5.1] APB_DIV is a synthesis parameter, not a register. The value is a property of the peripheral IP instantiated in that window, fixed at integration, and a register would let firmware set a value the IP cannot meet.

## 6 Register map

[N-6.1] The bridge has no registers. It is transparent apart from the error responses of §7.5–§7.7, which surface as precise faults in the accessing master. mem_ctl's MEMCTL register occupies offset 0x20 of window 8 and belongs to GARUDA-MEM-SPEC-001, not to the bridge.

### 6.1 Window map

Generated from GARUDA-SYS-001 apb.windows.

The window number IS the address nibble: window n is at 0x4000_0000 + 0x1000 x n, and PSEL bit n is haddr[15:12], exactly as ahb2apb_bridge.v decodes it (wire [3:0] win = haddr_i[15:12]). Window 0 (0x4000_0000) is block 14, the SPI slave — ADR-0020 held the slot unmapped rather than compacting the map when the block was cut, and ADR-0020 Rev 2 restored the block into it. Windows 12–15 (0xC–0xF) never exist and fault.

(Corrected 2026-09-26: this table previously numbered the rows 0..10 against the same base addresses, making every window number one lower than the hardware's. GARUDA-DMA/CLIC/TIMERS/CLKRST-SPEC-001 each inherited that error. The base addresses were right throughout; only the index was wrong.)

| Window | Base | Block | Peripheral | APB_DIV default |
| --- | --- | --- | --- | --- |
| 0 | 0x4000_0000 | 14 | spi_slave | ÷1 |
| 1 | 0x4000_1000 | 13 | spi_master | ÷1 |
| 2 | 0x4000_2000 | 15 | i2c | ÷1 |
| 3 | 0x4000_3000 | 16 | uart0 | ÷1 |
| 4 | 0x4000_4000 | 17 | uart1 | ÷1 |
| 5 | 0x4000_5000 | 9 | dma_cfg | ÷1 |
| 6 | 0x4000_6000 | 18 | uart2 | ÷1 |
| 7 | 0x4000_7000 | 19 | gpio | ÷1 |
| 8 | 0x4000_8000 | 20 | pwm | ÷1 |
| 9 | 0x4000_9000 | 22 | reset_ctrl + mem_ctl | ÷1 |
| 10 | 0x4000_A000 | 10 | clic_cfg | ÷1 |
| 11 | 0x4000_B000 | 11 | timers_cfg | ÷1 |

[N-6.2] 0x4000_0000 (window 0) is the SPI slave (block 14). ADR-0020 Rev 1 had removed it for the pin budget and kept the address unmapped rather than compacting the map; ADR-0020 Rev 2 restored it. It answers when the chip is built with WITH_SPI_SLAVE = 1 (garuda_chip_top, the default); with WITH_SPI_SLAVE = 0 its mask bit is clear and an access returns the error of §7.7. (APB-3, 2026-10-14: this paragraph still described the Rev 1 map, window 0 unmapped.)

## 7 Functional description

### 7.1 Transfer conversion

[N-7.1] An AHB transfer with hsel_i high, htrans_i = NONSEQ or SEQ, and a legal size starts an APB transfer. The FSM runs IDLE → SETUP → ACCESS → DONE.

| State | PSEL | PENABLE | Duration |
| --- | --- | --- | --- |
| IDLE | 0 | 0 | until a request |
| SETUP | 1 | 0 | 1 pclk |
| ACCESS | 1 | 1 | 1 pclk × APB_DIV, extended while PREADY is low |
| DONE | 0 | 0 | 1 pclk, returns the response to hclk |

[N-7.2] hreadyout_o is held low from acceptance until DONE, stalling the AHB master.

[N-7.3] Minimum latency is 4 pclk cycles = 8 hclk cycles = 32 ns at APB_DIV=÷1. A peripheral register access therefore costs the core about 8 cycles. At 7% bus utilisation and a few dozen peripheral accesses per flight-loop iteration this is immaterial; DMA exists so that per-byte peripheral traffic does not pay it.

### 7.2 Why no CDC is required — the proof

[N-7.4] pclk is generated by a toggle flop clocked by hclk (GARUDA-CLKRST-SPEC-001 [N-7.3]). Therefore:

- Every pclk rising edge occurs simultaneously with an hclk rising edge.

- The set of pclk edges is a proper subset of the set of hclk edges.

- The phase relationship is fixed, known at design time, and constant — there is no drift, because both clocks derive from the same source with an integer ratio.

- pclk_phase_i identifies, in the hclk domain, exactly which hclk cycle precedes a pclk edge.

[N-7.5] Consequently every transfer between the domains is sampled on an edge both domains share. There is no window in which a pclk flop samples an hclk flop mid-transition, because the two never transition at different times. Metastability requires a setup violation from an unknown phase relationship; here the phase relationship is known and constrained by static timing analysis like any other path.

[N-7.6] STA handles this with one create_clock and two create_generated_clock statements. The hclk→pclk paths are timed as ordinary multi-cycle synchronous paths. No synchronisers, no metastability, no CDC waivers, no gray coding.

[N-7.7] This is different in kind from the tck↔hclk boundary in the Debug Module, which is a genuine asynchronous crossing with an unrelated clock source and does require synchronisers.

[N-7.8] The hclk-side capture registers load on any hclk edge, and the pclk-side FSM advances only on pclk edges. The bridge uses pclk_phase_i to guarantee that the hclk side does not change the captured address or write data during the hclk cycle in which the pclk side samples it. Assertion a_capture_stable_at_phase (§10) checks exactly this.

### 7.3 Per-window access rate

[N-7.9] APB_DIV extends the ACCESS phase for a window by holding PENABLE asserted for 1, 2, 4 or 8 pclk cycles before sampling PREADY.

[N-7.10] This is an access-rate mechanism, not a clocking mechanism. The peripheral's flops still receive pclk at 125 MHz. It therefore accommodates an IP whose interface timing needs longer phases, but not an IP whose internal logic cannot meet the 8 ns pclk period. For that case the only remedies are DIVSEL on clk_div (GARUDA-CLKRST-SPEC-001 §6.2), which slows the whole chip, or a different IP. This distinction is stated because it is easy to assume APB_DIV solves both, and it does not.

[N-7.11] All twelve windows default to ÷1 on the assumption that a simple UART, SPI or I²C block in a 28 nm process meets 125 MHz. If an IP datasheet later shows otherwise, that window's APB_DIV changes and nothing else in the chip is affected.

### 7.4 Word-only access

[N-7.12] Only hsize_i = WORD (3'b010) is accepted. BYTE and HALFWORD accesses produce a two-cycle AHB ERROR without generating any APB transfer.

[N-7.13] Rationale: APB3 has no byte enables. A bridge that accepted a byte write would have to read the word, merge, and write back — three transfers for one, and a read-modify-write against a peripheral whose registers may have read side effects, clear-on-read fields or write-1-to-clear semantics. Silently doing that is worse than refusing. Rejecting it means the fault is precise and points at the instruction that caused it.

[N-7.14] Every peripheral register in this chip is 32 bits and word-aligned, so no correct firmware performs a sub-word peripheral access.

### 7.5 Timeout

[N-7.15] If PREADY for the selected window is not asserted within 16 pclk cycles of entering ACCESS, the bridge abandons the transfer, deasserts PSEL and PENABLE, and returns a two-cycle AHB ERROR.

[N-7.16] 16 cycles is chosen as comfortably longer than any legitimate wait in this peripheral set — none of them inserts more than 2 — while being short enough that a hung peripheral costs 128 ns rather than a stalled flight loop.

[N-7.17] The timeout is the only thing standing between a defective or unclocked peripheral and a permanently stalled bus. GARUDA-AHB-SPEC-001 [N-7.18] relies on it: the interconnect has no timeout of its own precisely because the only slave that can stall indefinitely has one here.

[N-7.18] A timeout leaves the peripheral in an undefined state — it may have latched a write. No recovery is attempted. The reasonable firmware response is to treat the access fault as fatal and reset.

### 7.6 Unmapped windows

[N-7.19] haddr[15:12] values with no window — 0x0 (the removed SPI slave) and 0xB–0xF — assert no PSEL and return a two-cycle AHB ERROR.

### 7.7 SLVERR

[N-7.20] A window asserting PSLVERR with PREADY causes the bridge to return a two-cycle AHB ERROR. One rule holds for every peripheral (AUD-13, ruled 2026-10-11 in the two-track plan): PSLVERR is raised only for an offset that is not a register in the window; a write to a read-only register is ignored and raises no error. (Until 2026-10-12 this note left the choice to each peripheral's specification, and the timers and the DMA answered PSLVERR to a read-only write while the CLIC, the reset controller and the shim ignored it.)

[N-7.21] All three error sources — sub-word, unmapped window, PSLVERR — produce the identical AHB two-cycle ERROR. The master cannot distinguish them, and does not need to: all three are firmware bugs, and mepc identifies the instruction.

## 8 Timing

### 8.1 Word write, APB_DIV = ÷1

│ h0 │ h1 │ h2 │ h3 │ h4 │ h5 │ h6 │ h7 │ h8 │
hclk       ──┘‾┐_┌‾┐_┌‾┐_┌‾┐_┌‾┐_┌‾┐_┌‾┐_┌‾┐_┌‾┐_┌
pclk       ──┘‾‾‾┐___┌‾‾‾┐___┌‾‾‾┐___┌‾‾‾┐___┌‾‾‾
pclk_phase ──┌─┐   ┌─┐   ┌─┐   ┌─┐   ┌─┐   ┌─┐
             └─┘   └─┘   └─┘   └─┘   └─┘   └─┘
               ▲     ▲     ▲     ▲
               └─────┴─────┴─────┴── every pclk edge is an hclk edge

hsel       ──┌──────┐──────────────────────────────
htrans     ──┤NONSEQ├─IDLE─────────────────────────
hsize      ──┤ WORD ├──────────────────────────────
hwdata     ──┤  D   ├──────────────────────────────

fsm         IDLE│ SETUP  │ ACCESS │ DONE  │IDLE
psel       ──────┌───────────────────┐─────────────
penable    ──────────────┌───────────┐─────────────
pwdata     ──────┤       D           ├─────────────
pready     ──────────────────┌───────┐─────────────

hreadyout  ‾‾‾‾‾╲________________________┌─────────
                                         └── 8 hclk total
hresp      ────────────────────────────────────────  OKAY

### 8.2 Read with one wait state, then APB_DIV = ÷2 comparison

ACCESS phase, ÷1, peripheral inserts one wait:
pclk       ──┘‾‾‾┐___┌‾‾‾┐___┌‾‾‾┐___┌‾‾‾
penable    ──────┌───────────────────┐────
pready     ──────────┐       ┌───────┐────   low for one pclk
                     └───────┘
                             ▲
                             └── ACCESS extended until PREADY

ACCESS phase, ÷2, peripheral always ready:
penable    ──────┌───────────────────┐────
                 │◀── 2 pclk ──▶│          PENABLE held 2 cycles
pready     ──────┌───────────────────┐────   before being sampled

### 8.3 Timeout

pclk       ──┘‾┐_┌‾┐_┌‾┐ ... ┌‾┐_┌‾┐_┌‾┐_┌
fsm         SETUP│ ACCESS ..................│ DONE
psel       ──┌───────────────────────────┐──────
penable    ──────┌─────────────────────┐────────
pready     ─────────────────────────────────────  never asserted
timeout    ──────< 1 >< 2 > ... < 15 >< 16 >
                                        ▲
                                        └── abandon
hresp      ────────────────────────────────┌──┐──  ERROR, two cycles
hreadyout  ‾‾‾╲────────────────────────────┐  ┌──
                                           └──┘

### 8.4 Sub-word access rejected

hsel       ──┌──────┐──────────
htrans     ──┤NONSEQ├─IDLE────
hsize      ──┤ BYTE ├──────────
psel       ────────────────────  no APB transfer at all
hresp      ──────┌──────┐──────  ERROR, two cycles
hreadyout  ‾‾‾‾‾╲       ┌──────
                 └──────┘

## 9 Clock, reset and power

[N-9.1] The bridge is the only block in the chip spanning hclk and pclk. Both are synchronous (§7.2). tck does not appear here.

[N-9.2] hreadyout_o resets to 0, not 1. The bridge spans two reset releases: hreset_n deasserts two hclk cycles after the stretch counter expires, preset_n two pclk cycles after that (GARUDA-CLKRST-SPEC-001 §7.17). Between those two moments the AHB side is live and the APB side is still in reset. Resetting hreadyout_o low holds any AHB master that addresses the APB region during that window, rather than letting it see a bogus OKAY from a bridge whose peripheral side cannot yet respond.

[N-9.3] hreadyout_o is released only when both hreset_n_i and the synchronised preset_n_i are deasserted. This is the requirement GARUDA-CLKRST-SPEC-001 §5.2 refers to.

[N-9.4] Power: the pclk domain at half hclk halves the dynamic power of the peripheral clock tree and of every peripheral flop, which together are the largest flop population outside the core. This is the whole reason pclk exists as a real divided clock rather than an access-rate trick — APB_DIV alone would deliver the same interface rate while leaving every peripheral flop toggling at 250 MHz, saving nothing.

[N-9.5] Each window's PSEL is a natural enable term for a per-peripheral clock gate. The gate belongs to the peripheral, not the bridge, so that a peripheral with internal timers — the PWM, for instance — can keep running while unselected. The bridge exposes psel_o and makes no gating decision.

## 10 Assertions

// --- APB3 protocol: SETUP is exactly one cycle with PSEL and no PENABLE
a_apb_setup: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  $rose(|psel_o) |-> !penable_o ##1 penable_o);

// --- PENABLE never asserts without PSEL
a_penable_needs_psel: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i) penable_o |-> |psel_o);

// --- PSEL is one-hot or zero
a_psel_onehot: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i) $onehot0(psel_o));

// --- address and write data stable for the whole APB transfer
a_apb_addr_stable: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  (|psel_o && !done) |-> $stable(paddr_o) && $stable(pwrite_o));
a_apb_wdata_stable: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  (penable_o && pwrite_o && !pready_sel) |=> $stable(pwdata_o));

// --- THE no-CDC property: the hclk-side capture is stable on the shared edge
a_capture_stable_at_phase: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  pclk_phase_i |-> $stable(addr_cap) && $stable(wdata_cap) && $stable(ctl_cap));

// --- every pclk edge coincides with an hclk edge (checked, not assumed)
a_pclk_on_hclk: assert property (@(posedge pclk_i) hclk_i);

// --- word-only: no APB transfer is ever generated for a sub-word access
a_subword_no_apb: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (hsel_i && htrans_i != IDLE && hsize_i != WORD) |-> ##[1:4] !$rose(|psel_o));

// --- sub-word access returns ERROR
a_subword_error: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (hsel_i && htrans_i != IDLE && hsize_i != WORD) |-> ##[1:2] hresp_o);

// --- every error is exactly the AHB two-cycle sequence
a_error_two_cycle: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (hresp_o && !hreadyout_o) |=> (hresp_o && hreadyout_o));

// --- timeout fires at exactly 16 and abandons the transfer
a_timeout_at_16: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  (penable_o && !pready_sel)[*16] |=> (!penable_o && !|psel_o));

// --- timeout always terminates the AHB transfer: the bus cannot hang (R-7)
a_no_hang: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (hsel_i && htrans_i != IDLE) |-> ##[1:40] hreadyout_o);

// --- unmapped window asserts no PSEL and errors
a_unmapped: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (hsel_i && htrans_i != IDLE && haddr_i[15:12] inside {4'h0, [4'hB:4'hF]})
    |-> ##[1:2] hresp_o);

// --- PSLVERR propagates as an AHB error
a_slverr_prop: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  (pready_sel && pslverr_sel) |-> ##[1:4] hresp_pending);

// --- hreadyout stays low until both resets have released (N-9.3)
a_dual_reset_hold: assert property (
  @(posedge hclk_i) (!hreset_n_i || !preset_n_sync) |-> !hreadyout_o);

// --- APB_DIV: PENABLE held for the configured number of cycles
a_apb_div: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  $rose(penable_o) |-> penable_o[*(1 << apb_div_sel)]);

## 11 Verification plan

| Req | Test | Oracle | Coverage goal | Status |
| --- | --- | --- | --- | --- |
| R-1 | t_apb_protocol | APB3 protocol checker, zero violations | — | new |
| R-1 | t_apb_rw | word read and write to every window | all 12 windows | new |
| R-2 | t_apb_decode | every haddr[15:12] value hits the right window or errors | all 16 | new |
| R-3 | t_apb_pclk_rate | peripherals observe 125 MHz | — | new |
| R-4 | t_apb_no_cdc | a_capture_stable_at_phase + a_pclk_on_hclk over 10⁶ cycles | — | new |
| R-5 | t_apb_div | each APB_DIV value stretches ACCESS correctly | all 4 values | new |
| R-6 | t_apb_subword | BYTE and HALF rejected, no APB transfer generated | both sizes × 4 offsets | new |
| R-7 | t_apb_timeout | stuck PREADY → ERROR at 16 cycles, bus recovers | each window | new |
| R-7 | t_apb_no_hang | a_no_hang under a randomly stalling peripheral model | — | new |
| R-8 | t_apb_unmapped | window 0x0 and 0xB–0xF error | all 6 | new |
| §7.7 | t_apb_slverr | PSLVERR becomes an AHB ERROR and a precise fault | — | new |
| §9.2 | t_apb_reset_skew | AHB access during the hreset/preset gap is held, not falsely acked | — | new |
| §7.1 | t_apb_backtoback | consecutive transfers to the same and different windows | same/different | new |

[N-11.1] t_apb_no_cdc and t_apb_reset_skew are the two tests specific to this bridge's architecture rather than to APB3 in general. The first checks the property the whole chip's clocking rests on; the second covers a window that exists only because two reset releases are staggered, and that no protocol checker would look for.

[N-11.2] Formal is recommended for a_no_hang and a_capture_stable_at_phase. Both are small, bounded, and load-bearing for the rest of the design.

## 12 Design decisions

| Decision | ADR |
| --- | --- |
| Real 125 MHz pclk from a toggle flop in clk_div, not an ICG in the bridge | ADR-0002 |
| No CDC, with the subset-of-edges proof rather than an assertion | ADR-0002 |
| Per-window APB_DIV as the remedy for a slow IP, not a second clock | ADR-0002 |
| Window map after the peripheral reassignment | ADR-0008 |
| Window 0x0 left unmapped rather than compacting the map | ADR-0020 |

## 13 Not implemented

| Feature | Why not |
| --- | --- |
| pclk generated by an ICG | §1.3: produces 25% duty, not the specified 50%. |
| CDC synchronisers | §7.2: the two clocks share edges. Synchronisers would add latency and imply an asynchrony that does not exist. |
| Byte/halfword support via read-modify-write | §7.13: three transfers for one, against registers with read side effects. |
| APB4 (PPROT, PSTRB) | No privilege model, no byte strobes needed. |
| Multiple outstanding APB transfers | APB is inherently one at a time. |
| Write buffering / posted writes | Would make a peripheral write's fault imprecise, so a bad write's mepc would point at the wrong instruction. |
| A software-visible timeout register | The timeout produces a precise fault; a status bit read afterwards adds nothing. |
| APB_DIV as a runtime register | §5.1: it is a property of the instantiated IP, not a firmware choice. |
| Per-window reset | All peripherals share preset_n. No use case for resetting one peripheral alone. |

## 14 Open items

None. OPEN-1 (peripheral IP maximum frequency) is closed by design rather than by an answer: APB_DIV defaults to ÷1 and is a per-window parameter, so if an IP's interface turns out to need more time, one parameter changes. [N-7.10] states the limit of that remedy honestly — an IP that cannot meet 8 ns internally needs DIVSEL, not APB_DIV.

## 15 Errata

No RTL yet.

Defects fixed in this revision:

| ID | Symptom | Root cause | Fix |
| --- | --- | --- | --- |
| BR-1 | pclk specified as 50% duty but generated by a mechanism that produces 25%. | Waveform and mechanism specified independently, neither checked against the other. | pclk moves to a toggle flop in clk_div; a_pclk_on_hclk checks it. |
| BR-2 | "No CDC required" asserted with no reason, while the DMA spec implemented full CDC against the same clock. | Two documents reasoning independently about one boundary. | §7.2 states the proof; a_capture_stable_at_phase checks it; DMA Rev 3.0 deletes the synchronisers. |
| BR-3 | An AHB access during the gap between hreset_n and preset_n release would be acknowledged by a bridge whose APB side was still in reset. | hreadyout reset value was 1, and the staggered release was not considered. | hreadyout resets to 0 and is gated on both resets ([N-9.2]). |
| BR-4 | Window map contained an SPI-slave window and a peripheral assignment contradicting two other documents. | Window map maintained by hand in three places. | Generated from GARUDA-SYS-001. |