# GARUDA DMA Controller — Design Specification

## 0.1 Identity

| Field | Value |
| --- | --- |
| Document ID | GARUDA-DMA-SPEC-001 |
| Revision | 3.0 |
| Date | 2026-09-18 |
| Status | Released for implementation |
| Block | 9 (dma) |
| Owner | Team AeroSoC |
| Supersedes | GARUDA_DMA_Controller_Design_Spec_v2_2 |

## 0.2 Revision history

| Rev | Change | Driver |
| --- | --- | --- |
| 1.0–2.2 | Initial through six channels, with CDC primitives on the config port and a channel assignment contradicting two other documents | — |
| 3.0 | All CDC logic deleted — the hclk/pclk boundary is synchronous. Channel assignment regenerated from GARUDA-SYS-001; no channel is shared. The ISRAM path and the boot-time role are removed. Block and CLIC numbering corrected. | ADR-0002, 0008, 0015, 0011 |
| 3.0a | Text aligned with the RTL and the yaml (Track A, 2026-10-12; tasks 0.5 and 0.7): CR reset 0x0000_0040 and SIZE = 3 refused (AUD-12 item 7); error interrupts at CLIC IDs 8–13, ID 7 reserved (INT-2); channel 4 = spi_slave, wired (yaml dma.assignment, [N-6.4]); new [N-6.6] PSLVERR rule: a write to STAT/GSTAT is ignored without an error (AUD-13, ruled in the two-track plan); [N-7.8] ack is 2 hclk wide (D-16); [N-7.20] reach: the fabric is unrestricted and the DMA refuses (AUD-12 item 1, D-16); §10 a_one_beat_per_req in a sound form, a_ch4_tied removed. Open, pending rulings: DMA-9 to DMA-14 (§14), the per-channel clock gate of [N-9.3] (AUD-12 item 2). | AUD-12, AUD-13, INT-2 |

## 0.3 Normative references

- AMBA 3 AHB-Lite Protocol Specification, ARM IHI 0033A.

- GARUDA-SYS-001 Rev 4.0 — channel assignment and priorities generated from dma.assignment.

- GARUDA-ADR-001 Rev 1.0.

- GARUDA-AHB-SPEC-001 Rev 4.0 — the DMA is master M3, highest priority.

- GARUDA-AHB2APB-SPEC-001 Rev 2.0 — the no-CDC proof this revision depends on.

- GARUDA-CLIC-SPEC-001 Rev 2.0 — IDs 1–6 (completion) and 8–13 (error); ID 7 is reserved (INT-2).

## 1 Purpose and scope

### 1.1 In scope

A six-channel DMA controller: per-channel configuration, request/acknowledge handshake with peripherals, fixed-priority channel arbitration, AHB master M3, and completion and error interrupts.

### 1.2 Out of scope

- The peripherals' own FIFOs and request generation.

- Boot. The DMA has no boot-time role in Rev 3.0 (§1.3).

### 1.3 What changed and why

The CDC logic is gone. Rev 2.2 §15.3 implemented gray-code counters, a toggle handshake and level synchronisers between the hclk core logic and the pclk config port. Meanwhile GARUDA_AHB2APB_Bridge_Design_Spec_v1_4 asserted that no CDC was required across the same boundary. Both could not be right. The bridge's conclusion was correct and its reasoning was missing; GARUDA-AHB2APB-SPEC-001 §7.2 now supplies the proof — pclk edges are a proper subset of hclk edges, so both sides always sample on a shared edge. The synchronisers were therefore solving a problem that does not exist, while adding 3–4 cycles of latency to every register access and an entire class of apparent-CDC that a reviewer had to trust. dma_cdc_gray.v, dma_cdc_pulse.v and dma_cdc_sync.v are deleted (RTL deltas R1, R2).

The channel assignment is fixed. Three documents disagreed:

|  | Rev 2.2 §5 | TRM §13 | TRM §14 |
| --- | --- | --- | --- |
| UART0 | sonar | GPS | — |
| UART1 | GPS | telemetry | telemetry |
| UART2 | LoRa | debug (no DMA) | LoRa |
| CH5 | GPIO/SPI2 | shared GPIO + SPI slave | — |

"SPI2" named a block that does not exist; a GPIO controller cannot be a DMA source; and a channel cannot be shared by two peripherals, because a channel has exactly one dma_req/dma_ack pair. The assignment now lives once, in GARUDA-SYS-001.

The boot role is removed. Rev 2.2 gave the DMA a flash→ISRAM copy path during boot, and GARUDA_AHB_Bus_Design_Spec_v3_0 added a dedicated DMA→ISRAM wire for it. The bootloader is now polled PIO (ADR-0015), so the DMA has no boot-time role and no ISRAM path.

## 2 Requirements

| ID | Requirement | Source |
| --- | --- | --- |
| R-1 | Six independent channels, one peripheral each. | ADR-0008 |
| R-2 | P2M, M2P and M2M transfer modes. | System |
| R-3 | No CDC logic on the configuration interface. | ADR-0002 |
| R-4 | Fixed-priority channel arbitration, re-evaluated per beat. | ADR-0004 |
| R-5 | Per-channel completion and error interrupts. | System |
| R-6 | A channel shall report a consistent transfer count and completion status. | Erratum D-1 |
| R-7 | A hardware completion shall not be lost to a simultaneous software write. | Erratum D-2 |
| R-8 | An AHB ERROR shall abort only the affected channel. | System |
| R-9 | The DMA shall have no access to Boot ROM or ISRAM. | ADR-0005, 0015 |

## 3 Block diagram

peripheral req/ack (6 pairs)
   ┌──┬──┬──┬──┬──┬──┐
   ▼  ▼  ▼  ▼  ▼  ▼  │
  ┌─────────────────────────────────────────────────────────┐
  │                    dma  (block 9)                       │
  │                                                         │
  │  ┌────────────────────────────────────────────────┐     │
  │  │ channel 0 │ 1 │ 2 │ 3 │ 4 │ 5                  │     │
  │  │  ┌──────────────────────────────────────────┐  │     │
  │  │  │ CR  SAR  DAR  CNT  STAT   per channel    │  │     │
  │  │  └──────────────────────────────────────────┘  │     │
  │  └───────────────────┬────────────────────────────┘     │
  │                      │                                  │
  │              ┌───────▼────────┐                         │
  │              │ ch_arbiter     │  fixed priority         │
  │              │ per-beat       │  ch0 > ch4 > ch1 >      │
  │              └───────┬────────┘  ch3 > ch2 > ch5        │
  │                      │                                  │
  │              ┌───────▼────────┐                         │
  │              │ xfer_engine    │  one beat at a time     │
  │              │ read → write   │                         │
  │              └───────┬────────┘                         │
  │                      │                                  │
  │              ┌───────▼────────┐                         │
  │              │ ahb_master     ├─────────────────────────┼──▶ AHB M3
  │              └────────────────┘                         │
  │                                                         │
  │  ┌──────────────────────────┐                           │
  │  │ dma_apb  window 5        │◀── APB, pclk              │
  │  │ NO CDC  (ADR-0002)       │                           │
  │  └──────────────────────────┘                           │
  │                                                         │
  │  complete[5:0] ──────────────────────────────────────────┼──▶ CLIC 1–6
  │  error[5:0]    ──────────────────────────────────────────┼──▶ CLIC 8–13
  └─────────────────────────────────────────────────────────┘

  No ISRAM path. No Boot ROM path. (R-9)

## 4 Sub-block inventory

| Name | Type | RTL file | Role |
| --- | --- | --- | --- |
| dma_chan ×6 | seq (hclk) | dma_chan.v | Per-channel registers and state machine. |
| ch_arbiter | comb | dma_arbiter.v | Fixed-priority select among requesting channels. |
| xfer_engine | seq (hclk) | dma_engine.v | Read beat, then write beat. |
| dma_ahb_master | seq (hclk) | dma_ahb_master.v | AHB-Lite master M3. |
| dma_apb | seq (pclk) | dma_apb_slave.v | Window 5. No CDC. |

[N-4.1] Deleted in Rev 3.0: dma_cdc_gray.v, dma_cdc_pulse.v, dma_cdc_sync.v.

## 5 Interfaces

| Port | Dir | Width | Domain | Reset | Description |
| --- | --- | --- | --- | --- | --- |
| hclk_i, hreset_n_i | in | 1 | hclk | — |  |
| pclk_i, preset_n_i | in | 1 | pclk | — | APB side only. |
| dma_req_i | in | 6 | hclk | — | One per channel, from its peripheral. |
| dma_ack_o | out | 6 | hclk | 0 | One per channel. |
| dma_complete_o | out | 6 | hclk | 0 | To CLIC IDs 1–6. |
| dma_error_o | out | 6 | hclk | 0 | To CLIC IDs 8–13 (ID 7 is reserved, INT-2). |
| AHB M3 | — | — | hclk | — | Per GARUDA-AHB-SPEC-001 §5.1. |
| APB slave | — | — | pclk | — | Window 5. |

[N-5.1] pclk_i and preset_n_i remain as ports because the APB side genuinely runs on pclk. What is deleted is the synchroniser logic between the domains, not the second clock. The register file's hclk-visible values are sampled directly, per GARUDA-AHB2APB-SPEC-001 §7.2.

## 6 Register map — APB window 5 (0x4000_5000)

(Window number corrected 2026-09-26: was stated as 4, inherited from the 0-based table in GARUDA-AHB2APB-SPEC-001. The hardware decodes window n at 0x4000_0000 + 0x1000 x n (haddr[15:12]), so the base address quoted here was always right and only the index was wrong.)

Per channel n (0–5), base offset 0x20 × n:

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| +0x00 | CR | RW | 0x0000_0040 | Control. SIZE resets to 2 (word); every other field resets to 0 (§6.1). |
| +0x04 | SAR | RW | 0 | Source address. |
| +0x08 | DAR | RW | 0 | Destination address. |
| +0x0C | CNT | RW | 0 | Transfer count in beats. |
| +0x10 | STAT | RO | 0 | Status, including beats remaining. |
| +0x14 | ICLR | W1C | — | Interrupt clear. |

Global, at 0x100:

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x100 | GSTAT | RO | Per-channel active, complete and error summary. |

### 6.1 CR (+0x00)

| Bit | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0 | EN | RW | 0 | Channel enable. Cleared by hardware on completion or error. |
| 2:1 | MODE | RW | 0 | 0 = P2M, 1 = M2P, 2 = M2M, 3 reserved. |
| 3 | SINC | RW | 0 | Increment SAR after each beat. |
| 4 | DINC | RW | 0 | Increment DAR after each beat. |
| 6:5 | SIZE | RW | 2 | 0 = byte, 1 = halfword, 2 = word, 3 reserved: a write of 3 leaves SIZE unchanged, as for MODE ([N-6.1]). |
| 7 | IE_COMP | RW | 0 | Completion interrupt enable. |
| 8 | IE_ERR | RW | 0 | Error interrupt enable. |
| 31:9 | reserved | — | 0 |  |

[N-6.1] MODE = 3 is reserved and a write of 3 leaves MODE unchanged, rather than being accepted and behaving as one of the legal modes. An illegal mode that silently aliases is worse than one that refuses.

### 6.2 STAT (+0x10)

| Bits | Name | Description |
| --- | --- | --- |
| 15:0 | REMAINING | Beats not yet transferred. |
| 16 | ACTIVE | Channel is enabled and has beats remaining. |
| 17 | COMPLETE | All beats transferred. Sticky until ICLR. |
| 18 | ERROR | AHB ERROR encountered. Sticky until ICLR. |
| 21:19 | ERRPHASE | 0 = none, 1 = read, 2 = write. |

[N-6.2] REMAINING and COMPLETE are updated in the same cycle by the same logic, so they are never inconsistent (R-6). Erratum D-1 was exactly this inconsistency, caused by the two fields being updated on opposite sides of the deleted clock crossing: COMPLETE arrived through a pulse synchroniser while REMAINING arrived through a gray-coded counter, with different latencies, so firmware could read COMPLETE = 1 with REMAINING non-zero. Removing the crossing removes the mechanism, not merely the symptom.

[N-6.3] ERRPHASE distinguishes a failed source read from a failed destination write. Without it, an error on an M2M transfer leaves firmware unable to tell which address was bad, and the source and destination are both still in the registers, so the distinction costs three bits and saves a debugging session.

### 6.3 Channel assignment

Generated from GARUDA-SYS-001 dma.assignment:

| Channel | Peripheral | Function | Priority | CLIC complete | CLIC error |
| --- | --- | --- | --- | --- | --- |
| 0 | spi_master | IMU | 5 (highest) | 1 | 8 |
| 1 | i2c | baro, mag, sonar | 3 | 2 | 9 |
| 2 | uart0 | GPS | 1 | 3 | 10 |
| 3 | uart1 | ground link | 2 | 4 | 11 |
| 4 | spi_slave | companion link (ESP32 / ESP-NOW) | 4 | 5 | 12 |
| 5 | uart2 | console | 0 (lowest) | 6 | 13 |

[N-6.4] Channel 4 serves the SPI slave (ESP32 / ESP-NOW mesh), restored and required per ADR-0020 Rev 2 (APF's neighbour-position source). Its registers, interrupts and arbiter position already exist and work, and rtl/spi_slave/ is built with its request wired to dma_req_i[4] (garuda_chip_top.v), so CH4 carries traffic. (Updated 2026-10-12: the text said the line was still tied low.) The final pin outcome is gated on OPEN-2 (ADR-0020 Rev 2).

[N-6.5] The priority order is by consequence of not being serviced. The IMU is the control loop's hard-real-time input and is highest. The console is lowest because nothing depends on it — dropping a debug character is free. GPS at 115200 baud is one beat every ~21,700 hclk cycles, so its low priority costs nothing.

### 6.4 Register-port errors

[N-6.6] PSLVERR is raised only for an offset that is not a register: channels 6 and 7 (0xC0–0xFC), +0x18 and +0x1C in each channel, 0x104 and above, and unaligned offsets. A write to a read-only register (STAT, GSTAT) is ignored and raises no error; ICLR reads 0. This is the single rule of GARUDA-AHB2APB-SPEC-001 [N-7.20] (AUD-13, ruled in the two-track plan, 2026-10-11). Until 2026-10-12 the DMA answered PSLVERR to a write to STAT or GSTAT.

## 7 Functional description

### 7.1 Channel operation

[N-7.1] Firmware writes SAR, DAR, CNT and MODE, then sets EN. The channel waits for dma_req_i[n], performs one beat, decrements REMAINING, and asserts dma_ack_o[n].

[N-7.2] A beat is a read followed by a write, as two separate AHB SINGLE transfers. The DMA does not use bursts.

[N-7.3] When REMAINING reaches zero: COMPLETE sets, ACTIVE clears, hardware clears EN, and dma_complete_o[n] asserts if IE_COMP is set.

[N-7.4] dma_complete_o[n] is level and stays asserted until ICLR clears COMPLETE. This matches the CLIC's level-triggered contract (GARUDA-CLIC-SPEC-001 §7.2).

### 7.2 Modes

| Mode | SAR | DAR | Typical use |
| --- | --- | --- | --- |
| P2M | peripheral data register, SINC=0 | memory, DINC=1 | IMU burst into a DSRAM buffer |
| M2P | memory, SINC=1 | peripheral, DINC=0 | telemetry frame out of DSRAM |
| M2M | memory, SINC=1 | memory, DINC=1 | buffer copy |

[N-7.5] M2M has no peripheral request. It runs continuously while enabled, taking a beat whenever it wins arbitration. Because the DMA is the highest-priority AHB master (M3), an M2M transfer will stall the core for the duration.

[N-7.6] That is a real hazard and firmware must treat it as one: an M2M transfer of 16,384 beats holds M3 requesting for ~32,768 cycles, during which the core progresses only in the gaps. There is no rate limiting in hardware. The mitigation is firmware discipline — break large M2M transfers into chunks — and it is stated here because the hardware gives no protection. No use case in the current firmware needs M2M at all; it exists because it costs nothing over the P2M/M2P datapath.

### 7.3 The request/acknowledge handshake

[N-7.7] dma_req_i[n] is level: the peripheral asserts it while it has data (P2M) or space (M2P), and deasserts when the beat has been taken.

[N-7.8] dma_ack_o[n] is a pulse two hclk cycles wide (one pclk period; D-16, erratum DMA-7), asserted when a beat completes successfully. The pclk-domain peripheral sees it high at exactly one pclk edge, whichever hclk phase the beat ends on.

[N-7.9] All six peripherals' request lines are in the pclk domain and cross to hclk. This crossing is synchronous (GARUDA-AHB2APB-SPEC-001 §7.2) and requires no synchroniser. The peripheral asserts on a pclk edge, which is also an hclk edge, so the hclk sample is of a stable value.

[N-7.10] Because pclk is half hclk, a request asserted for one pclk cycle is visible for two hclk cycles. The channel state machine takes at most one beat per request assertion, tracked by an edge-qualified taken flag, so a two-cycle-visible request does not produce two beats.

### 7.4 Channel arbitration

[N-7.11] Fixed priority per GARUDA-SYS-001, re-evaluated at each beat boundary. A channel that wins arbitration completes its full read-write beat pair before arbitration is re-evaluated; a beat is never split between channels.

[N-7.12] Starvation: channel 5 can be starved while higher channels request continuously. With real peripheral rates — the IMU at 8 kHz, the others far slower — the aggregate request rate is a few thousand beats per second against 125 million available, so starvation is unreachable.

### 7.5 Error handling

[N-7.13] An AHB ERROR on either the read or the write beat: ERROR sets, ERRPHASE records which, EN clears, ACTIVE clears, REMAINING freezes at its value at the point of failure, and dma_error_o[n] asserts if IE_ERR is set.

[N-7.14] Only the affected channel aborts (R-8). Other channels continue. There is no global error state.

[N-7.15] REMAINING freezing rather than zeroing means firmware can compute exactly which address failed: SAR and DAR also hold their values at the point of failure, since they are only incremented on a successful beat.

[N-7.16] The channel is not automatically retried. A bus error from a DMA channel means a bad address in SAR/DAR, which is a firmware bug, and retrying a bad address forever is worse than stopping.

### 7.6 Simultaneous hardware and software register updates

[N-7.17] Hardware clears EN on completion or error. Firmware may write CR in the same cycle. Hardware wins for the EN bit; the rest of the write takes effect.

[N-7.18] This is erratum D-2. Previously the software write took the whole register, including a stale EN = 1, so a channel that had just completed was re-enabled with REMAINING = 0 and never completed again — it sat ACTIVE with nothing to do, and firmware waiting on COMPLETE hung. The fix is a per-bit priority on EN only.

[N-7.19] The general rule, which applies to every hardware-updated field in this block: where hardware and software write the same bit in the same cycle, hardware wins for bits hardware owns (EN, COMPLETE, ERROR, REMAINING, ACTIVE), and software wins for bits it owns (MODE, SINC, DINC, SIZE, IE_*, SAR, DAR, CNT). Stating it as a rule rather than a fix for one bit is what prevents the next instance.

### 7.7 Reachability

[N-7.20] The DMA reaches DSRAM and the APB region, and never ISRAM or Boot ROM (R-9). The interconnect is unrestricted (ADR-0005): GARUDA-SYS-001 ahb.master_reachability and GARUDA-AHB-SPEC-001 let master M3 address every region. R-9 is therefore enforced inside the DMA (DECISIONS D-16): the engine refuses a beat whose SAR or DAR lies in ISRAM (0x0xxx_xxxx) or Boot ROM (0x1xxx_xxxx) with ERROR and ERRPHASE = read or write, and issues no AHB transfer to the forbidden address. (Reworded 2026-10-12, AUD-12 item 1. Open: a bad DAR is refused only after the source read, DMA-10.)

[N-7.21] The ISRAM path existed in Rev 2.2 only to support the DMA boot copy. With the polled-PIO bootloader (ADR-0015) nothing needs it, and removing it means a DMA channel misconfigured with an ISRAM address gets a clean ERROR rather than corrupting instruction memory. This is an omission of an unused path that happens to also be a safety property; the ISRAM write lock (ADR-0006) covers the case more generally.

## 8 Timing

### 8.1 P2M beat: IMU into DSRAM

│ T0 │ T1 │ T2 │ T3 │ T4 │ T5 │
hclk        ──┘‾┐_┌‾┐_┌‾┐_┌‾┐_┌‾┐_┌‾┐_┌

dma_req[0]  ──┌──────────────────┐──────────  SPI RX FIFO not empty
                                              (pclk-asserted, hclk-visible
                                               for 2 cycles  [N-7.10])
grant M3    ──────┌──────────────────────┐──

              ── read beat ──  ── write beat ──
haddr       ──────┤ SAR      ├──┤ DAR      ├──
hwrite      ──────┤ 0        ├──┤ 1        ├──
hrdata      ───────────┤ data ├──────────────
hwdata      ────────────────────┤ data     ├──

dma_ack[0]  ────────────────────────┌─┐──────  1-cycle pulse
REMAINING   ──┤  N              │  N-1     ├──
DAR         ──┤  A              │  A+4     ├──  DINC=1

### 8.2 Completion

REMAINING   ──┤ 1 │ 0                        ├
ACTIVE      ──┌──────┐──────────────────────────
COMPLETE    ─────────┌──────────────────────────  sticky
EN          ──┌──────┐──────────────────────────  hardware clears
dma_complete──────────┌─────────────────────────  level, until ICLR
                      │
            ICLR write┴───────────────────────┐
COMPLETE    ──────────────────────────────────┘

[N-8.1] REMAINING and COMPLETE change on the same edge — the D-1 inconsistency is structurally impossible now that there is no crossing between them.

### 8.3 Erratum D-2: the write collision

BEFORE (Rev 2.2):
              │ T0      │ T1                │
REMAINING   ──┤ 1 │ 0                       ├
hw_clear_en ──────┌─┐──────────────────────────
sw_write_CR ──────┌─┐──────────────────────────  firmware writes CR, EN=1
EN          ──┌──────────────────────────────┐   ◀── stayed set!
ACTIVE      ──┌──────────────────────────────┐   ◀── active, REMAINING=0
COMPLETE    ─────────┌──────────────────────────
                     ▲
                     └── firmware waits on COMPLETE forever on the
                         next transfer: the channel never re-arms

AFTER (Rev 3.0):
hw_clear_en ──────┌─┐──────────────────────────
sw_write_CR ──────┌─┐──────────────────────────
EN          ──┌──────┐─────────────────────────  ◀── hardware wins [N-7.17]
              (MODE/SINC/DINC from the sw write DO take effect)

### 8.4 Register access latency, before and after

Rev 2.2, with CDC:
apb write ──┐
            ├─ pclk capture ─ 2ff sync ─ hclk apply
            │  1 pclk         2 hclk     1 hclk     = 3-4 hclk extra
            
Rev 3.0, no CDC:
apb write ──┐
            ├─ pclk capture ─ hclk sample (shared edge)
            │  1 pclk         0 extra

[N-8.2] Every DMA register access is 3–4 hclk cycles faster, and the register file has one fewer failure mode. This is a side benefit; the reason for the change is correctness of the model, not speed.

## 9 Clock, reset and power

[N-9.1] Two clocks, no CDC. The channel logic, arbiter, engine and AHB master are in hclk. The APB register interface is in pclk. Values cross on shared edges (GARUDA-AHB2APB-SPEC-001 §7.2).

[N-9.2] Reset is hreset_n for the hclk side and preset_n for the APB side. All channels reset disabled with CNT = 0, so no transfer can begin before firmware configures one.

[N-9.3] Power: each channel's registers and state machine are gated by that channel's EN. A disabled channel receives no clock. With all channels idle the block is inert apart from the arbiter's combinational logic.

[N-9.4] The per-channel gate is the cleanest clock-gating opportunity in the chip: EN is an explicit, single-bit, already-registered idle condition, unlike the core's WFI condition which has to be derived from pipeline state.

(Open, AUD-12 item 2: rtl/dma contains no clock gate, so [N-9.3] and [N-9.4] describe a gate that was not built. Ruling pending (Track A DMA plan D5; recommended: this revision has no per-channel gate). vPlan F41.)

## 10 Assertions

// --- no CDC primitives remain (R-3): structural, checked at review
// --- and behaviourally: the register file responds within one hclk of the pclk edge
a_no_cdc_latency: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  (apb_write && apb_sel) |=> $changed(reg_hclk_view) || $stable(apb_wdata));

// --- REMAINING and COMPLETE are always consistent (R-6, erratum D-1)
a_remaining_complete_consistent: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  ch_complete[n] |-> (ch_remaining[n] == 16'd0));
a_active_implies_remaining: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  ch_active[n] |-> (ch_remaining[n] != 16'd0));

// --- hardware wins EN on a collision (R-7, erratum D-2)
a_hw_wins_en: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (hw_clear_en[n] && sw_write_cr[n]) |=> !ch_en[n]);

// --- but the rest of the software write still lands
a_sw_write_lands: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (hw_clear_en[n] && sw_write_cr[n]) |=> (ch_mode[n] == $past(sw_wdata[2:1])));

// --- a beat is never split between channels (N-7.11)
a_beat_atomic: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (xfer_state == READ) |-> ##[1:$] (xfer_state == WRITE) && $stable(cur_ch));

// --- exactly one channel active in the engine at a time
a_one_channel: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (xfer_state != IDLE) |-> $onehot(ch_granted));

// --- one beat per request assertion (N-7.10). need_low[n] is a checker flop:
// --- set by a successful beat on n while dma_req_i[n] is high, cleared while
// --- dma_req_i[n] is low. (Rev 3.0 wrote "$rose(ack) |-> !ack throughout
// --- req[*1:$]", which is false in its own first cycle because ack has just
// --- risen; corrected 2026-10-12.)
a_one_beat_per_req: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (need_low[n] && ch_mode[n] != M2M) |-> !ch_eligible[n]);

// --- error aborts only the affected channel (R-8)
a_error_isolated: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  $rose(ch_error[n]) |=> !ch_en[n] && $stable(ch_en_others));

// --- ERRPHASE is set correctly
a_errphase: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (hresp && xfer_state == READ) |=> (ch_errphase[cur_ch] == 3'd1));

// --- SAR/DAR/REMAINING freeze at the failure point (N-7.15)
a_freeze_on_error: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  ch_error[n] |=> $stable(ch_sar[n]) && $stable(ch_dar[n]) && $stable(ch_remaining[n]));

// --- no ISRAM or Boot ROM access ever (R-9)
a_no_isram: assert property (
  @(posedge hclk_i) disable iff (!hreset_n_i)
  (dma_htrans != IDLE) |-> (dma_haddr[31:28] != 4'h0 && dma_haddr[31:28] != 4'h1));

// --- MODE=3 is rejected (N-6.1)
a_mode3_rejected: assert property (
  @(posedge pclk_i) disable iff (!preset_n_i)
  (apb_write && apb_wdata[2:1] == 2'b11) |=> $stable(ch_mode[n]));

// --- all channels disabled at reset
a_reset_disabled: assert property (
  @(posedge hclk_i) !hreset_n_i |=> (ch_en == 6'd0));

## 11 Verification plan

| Req | Test | Oracle | Coverage goal | Status |
| --- | --- | --- | --- | --- |
| R-1 | t_dma_all_channels | each channel transfers independently | all 6 | passing, re-run after CDC removal |
| R-2 | t_dma_modes | P2M, M2P, M2M each correct | 3 modes × 3 sizes | passing, re-run |
| R-3 | t_dma_no_cdc | register access completes with no synchroniser latency | — | new |
| R-3 | — | dma_cdc_*.v absent from the file list | — | review gate |
| R-4 | t_dma_priority | highest-priority requesting channel wins | all 15 channel pairs | passing, re-run |
| R-5 | t_dma_interrupts | complete and error reach the right CLIC IDs | all 12 IDs | extend |
| R-6 | t_dma_stat_consistent | COMPLETE never with non-zero REMAINING | — | D-1 regression guard |
| R-7 | t_dma_write_collision | software CR write in the completion cycle | collision at each of the last 3 beats | D-2 regression guard |
| R-8 | t_dma_error_isolated | one channel errors, others unaffected | error on each channel | extend |
| R-8 | t_dma_errphase | read vs write error distinguished | both phases | new |
| R-9 | t_dma_no_isram | ISRAM and Boot ROM addresses give a clean ERROR | both regions | new |
| §7.3 | t_dma_req_ack | one beat per request, across the pclk/hclk ratio | — | extend |
| §7.6 | t_dma_hw_sw_priority | the [N-7.19] rule for every hardware-owned bit | all 5 bits | new |
| — | t_dma_random | constrained-random config vs. a reference model | all modes, sizes, channels | extend |

[N-11.1] The existing regression passes for R-1, R-2, R-4 and R-8. Rev 3.0's verification burden is mostly re-running those after the CDC removal, plus the two regression guards. Removing logic is lower risk than adding it, but the request/acknowledge path changes timing, so §7.3 needs re-verification rather than assumption.

[N-11.2] t_dma_write_collision must hit the collision cycle exactly. A random test will almost never produce it, which is why D-2 survived to be found late. The test needs a directed, cycle-accurate trigger.

## 12 Design decisions

| Decision | ADR |
| --- | --- |
| No CDC on the config port; three CDC modules deleted | ADR-0002 |
| Channel assignment generated once; no shared channels | ADR-0008 |
| No boot role; no ISRAM path | ADR-0015, 0005 |
| Channel 4 = SPI slave (companion link), restored by ADR-0020 Rev 2; updated 2026-10-12 (was "spare") | ADR-0020 |
| Block 9, CLIC IDs 1–6 and 8–13 (INT-2) | ADR-0011, 0009 |

## 13 Not implemented

| Feature | Why not |
| --- | --- |
| CDC synchronisers | §1.3. The boundary is synchronous; they added latency and implied an asynchrony that does not exist. |
| Burst transfers | A peripheral request is per-byte or per-word. A burst would need a FIFO in the DMA to justify it, and at 7% bus utilisation there is nothing to gain. |
| Scatter-gather / descriptor chaining | Needs descriptor fetches from memory and a much larger state machine. No use case: every transfer here is one contiguous buffer. |
| 2D / strided transfers | No use case. |
| Channel linking (one completion starts the next) | Firmware can do this in the completion handler at the cost of a few microseconds, which no transfer here is sensitive to. |
| Round-robin or weighted arbitration | §7.12: starvation is unreachable. |
| Automatic retry on bus error | §7.16: a DMA bus error is a firmware bug, and retrying a bad address forever is worse than stopping. |
| M2M rate limiting | §7.6: no hardware protection. Firmware discipline, stated explicitly. |
| A shared channel serving two peripherals | §1.3: not implementable — one req/ack pair per channel. |

## 14 Open items

- DMA-9 to DMA-14 (2026-10-12, Docs/bugs/dma.md): restart after a software stop reloads REMAINING from CNT; a bad DAR is refused after the source read; a beat still in flight lands in a re-armed transfer; ACK_CYCLES is unguarded; an unaligned halfword or word SAR/DAR puts a misaligned transfer on the bus (vPlan F17); a CR write during a beat (such as the stop CR = 0) changes that beat's size (vPlan F45). Rulings pending (Track A DMA plan). Until then this specification does not describe the restart, re-arm, unaligned and write-during-a-beat behaviour.

- AUD-12 item 2 (2026-10-12): the per-channel clock gate of [N-9.3] does not exist in rtl/dma; ruling pending (DMA plan D5).

- OPEN-D1 — closed 2026-09-26. The APB side now runs on pclk, with the registers still on hclk as ADR-0002 Rev 2 asks. PRDATA is registered on pclk at the setup-to-access edge, so it is stable for the whole access phase: the 4 ns hclk-to-pclk hop still exists but is now local to the block, a few gates from the source flop, and what crosses to the bridge is pclk-to-pclk with a full period. tb_timers gained a permanent monitor that counts any PRDATA movement during an access phase; it reports 3 movements against the pre-migration RTL and 0 after, so the check discriminates rather than merely passing.

## 15 Errata

Existing RTL — fixed and verified:

| ID | Symptom | Root cause | Fix | Regression test |
| --- | --- | --- | --- | --- |
| D-1 | Firmware could read COMPLETE = 1 with REMAINING non-zero, and act on a transfer that had not finished. | The two fields crossed the pclk/hclk boundary through different mechanisms — a pulse synchroniser and a gray-coded counter — with different latencies. | Rev 3.0 removes the crossing entirely, so both fields update on the same edge. The mechanism is gone, not just the symptom. | t_dma_stat_consistent |
| D-2 | A channel that completed in the same cycle as a software CR write was left enabled with REMAINING = 0, permanently ACTIVE and never completing again. Firmware waiting on COMPLETE hung. | The software write took the whole register including a stale EN = 1, overwriting the hardware clear. | Per-bit priority: hardware wins EN; the rest of the write lands. Generalised to the [N-7.19] rule. | t_dma_write_collision |

Defects fixed in this revision:

| ID | Symptom | Root cause | Fix |
| --- | --- | --- | --- |
| D-3 | CDC logic implemented against a clock the bridge spec said needed none. | Two documents reasoning independently about one boundary. | §1.3; three modules deleted. |
| D-4 | Channel assignment contradicted two other documents; one channel was shared between two peripherals; one source was a block that does not exist. | Assignment maintained by hand in three places. | Generated from GARUDA-SYS-001. |
| D-5 | A DMA→ISRAM path existed solely to support a boot copy that is no longer performed. | The path was added to work around the interconnect restriction rather than fixing it. | Path removed; ADR-0015. |
| D-6 | Block numbered 9 here, CLIC at 16 and I²C at 12 in §18.2, contradicting the TRM. | Block numbers assigned per-document. | ADR-0011. |