// =============================================================

// barrel_shifter_tb.sv

// Full SV verification environment for GARUDA barrel_shifter.v

// Includes: interface, class-based transaction (rand+constraint = CRV),

//           driver/monitor/scoreboard/env/test, SVA assertions,

//           functional coverage (covergroup), directed corner-case tests.

//

// !! REQUIRES a simulator with full IEEE1800 support (covergroup,

//    assert property, constraint solver). Icarus Verilog does NOT

//    support these constructs at all. On EDA Playground, set

//    Tools & Simulators -> Aldec Riviera-PRO (or Questa/VCS/Xcelium

//    if you have access) before running this file. !!

// =============================================================

`timescale 1ns / 1ps

// -------------------------------------------------------------

// 1. DUT: barrel_shifter.v (pasted verbatim, self-contained file)

// -------------------------------------------------------

// -------------------------------------------------------------

// 2. INTERFACE (+ bound SVA assertions)

// -------------------------------------------------------------

interface bs_if (input bit clk);

logic [47:0] cluster_out;

logic [5:0]  shift_amt;

logic        shift_dir;

logic [31:0] shift_result;

// ---------------------------------------------------------

// Concurrent assertions — protocol/spec invariants, independent

// of the scoreboard's per-transaction reference-model check.

// NOTE: DUT is combinational and the driver updates inputs with a

// non-blocking assign on posedge clk, so by SVA "preponed" sampling

// semantics these properties observe the settled result of the

// PREVIOUS clock's stimulus (same alignment the monitor uses via

// @(posedge clk); #1;). Confirm exact alignment on your waveform

// the first time you run this on Riviera-PRO/Questa.

// ---------------------------------------------------------

// A1: shift_amt == 0 is always a no-op, regardless of direction

property p_zero_shift_is_noop;

@(posedge clk) (shift_amt == 6'd0) |-> (shift_result == cluster_out[31:0]);

endproperty

a_zero_shift_is_noop: assert property (p_zero_shift_is_noop)

else $error("[SVA-FAIL] shift_amt=0 must be a no-op: got=%0h exp=%0h", shift_result, cluster_out[31:0]);

// A2: logical left shift saturates to 0 once shift_amt >= 48 (out of operand width)

property p_left_shift_saturates_zero;

@(posedge clk) (shift_dir == 1'b1 && shift_amt >= 6'd48) |-> (shift_result == 32'd0);

endproperty

a_left_shift_saturates_zero: assert property (p_left_shift_saturates_zero)

else $error("[SVA-FAIL] left shift with amt>=48 must saturate to 0: got=%0h amt=%0d", shift_result, shift_amt);

// A3: arithmetic right shift saturates to all-sign-bit once shift_amt >= 48

property p_right_shift_saturates_sign;

@(posedge clk) (shift_dir == 1'b0 && shift_amt >= 6'd48) |-> (shift_result == {32{cluster_out[47]}});

endproperty

a_right_shift_saturates_sign: assert property (p_right_shift_saturates_sign)

else $error("[SVA-FAIL] right shift with amt>=48 must saturate to sign-fill: got=%0h msb=%0b", shift_result, cluster_out[47]);

// A4: right-shift saturation of a positive value (msb=0) must be all-zero

//     (specific instance of A3, kept separate since it's the easiest one

//     to get subtly wrong if sign-extension logic has a polarity bug)

property p_right_shift_positive_stays_bounded;

@(posedge clk) (shift_dir == 1'b0 && shift_amt >= 6'd48 && cluster_out[47] == 1'b0) |-> (shift_result == 32'd0);

endproperty

a_right_shift_positive_bounded: assert property (p_right_shift_positive_stays_bounded)

else $error("[SVA-FAIL] right-shift saturation of a positive value must be all-0, got=%0h", shift_result);

endinterface

// -------------------------------------------------------------

// 3. TRANSACTION — rand + constraint => real constrained-random (CRV)

// -------------------------------------------------------------

class bs_transaction;

rand bit [47:0] cluster_out;

rand bit [5:0]  shift_amt;

rand bit        shift_dir;

// Weight generation toward the corner cases that actually matter for

// a 48-bit-width barrel shifter: near 0, near 47/48 boundary, and max.

constraint c_shift_amt_dist {

shift_amt dist {

6'd0            :/ 10,

[6'd1  : 6'd15] :/ 20,

[6'd16 : 6'd31] :/ 20,

[6'd45 : 6'd47] :/ 20,   // right at the 48-bit width boundary

6'd48           :/ 10,   // exactly out-of-width

[6'd49 : 6'd62] :/ 10,

6'd63           :/ 10    // max representable

};

}

// Weight cluster_out toward sign-bit / all-0 / all-1 corner patterns

// most of the time, but still allow fully random values too.

constraint c_cluster_out_dist {

cluster_out dist {

48'h0000_0000_0000 :/ 10,

48'hFFFF_FFFF_FFFF :/ 10,

48'h8000_0000_0000 :/ 10,   // most negative (sign bit only)

48'h7FFF_FFFF_FFFF :/ 10,   // most positive

[48'h0000_0000_0001 : 48'hFFFE_FFFF_FFFE] :/ 60

};

}

function string sprint();

return $sformatf("cluster_out=%012h shift_amt=%0d shift_dir=%0b", cluster_out, shift_amt, shift_dir);

endfunction

endclass

// -------------------------------------------------------------

// 4. RESULT (monitor -> scoreboard payload)

// -------------------------------------------------------------

class bs_result;

bs_transaction txn;

logic [31:0] shift_result;

endclass

// -------------------------------------------------------------

// 5. GENERATOR — directed corner-case list + CRV pump

// -------------------------------------------------------------

class bs_generator;

mailbox #(bs_transaction) gen2drv;

int num_random;

function new(mailbox #(bs_transaction) gen2drv, int num_random = 200);

this.gen2drv  = gen2drv;

this.num_random = num_random;

endfunction

task run();

bs_transaction t;

// ---------------- DIRECTED CORNER CASES ----------------

// 1) shift_amt = 0, both directions -> must be exact no-op

t = new(); t.cluster_out = 48'h1234_5678_9ABC; t.shift_amt = 0; t.shift_dir = 0; gen2drv.put(t);

t = new(); t.cluster_out = 48'h1234_5678_9ABC; t.shift_amt = 0; t.shift_dir = 1; gen2drv.put(t);

// 2) all-zero operand

t = new(); t.cluster_out = 48'h0000_0000_0000; t.shift_amt = 20; t.shift_dir = 0; gen2drv.put(t);

t = new(); t.cluster_out = 48'h0000_0000_0000; t.shift_amt = 20; t.shift_dir = 1; gen2drv.put(t);

// 3) all-one operand (-1) -> arithmetic right shift must stay all-1s

t = new(); t.cluster_out = 48'hFFFF_FFFF_FFFF; t.shift_amt = 1;  t.shift_dir = 0; gen2drv.put(t);

t = new(); t.cluster_out = 48'hFFFF_FFFF_FFFF; t.shift_amt = 30; t.shift_dir = 0; gen2drv.put(t);

t = new(); t.cluster_out = 48'hFFFF_FFFF_FFFF; t.shift_amt = 47; t.shift_dir = 0; gen2drv.put(t);

// 4) sign-boundary values: most negative / most positive 48-bit

t = new(); t.cluster_out = 48'h8000_0000_0000; t.shift_amt = 1;  t.shift_dir = 0; gen2drv.put(t);

t = new(); t.cluster_out = 48'h8000_0000_0000; t.shift_amt = 47; t.shift_dir = 0; gen2drv.put(t);

t = new(); t.cluster_out = 48'h7FFF_FFFF_FFFF; t.shift_amt = 1;  t.shift_dir = 0; gen2drv.put(t);

t = new(); t.cluster_out = 48'h7FFF_FFFF_FFFF; t.shift_amt = 47; t.shift_dir = 0; gen2drv.put(t);

// 5) exact width boundary: shift_amt = 47 (last in-width value)

t = new(); t.cluster_out = 48'hABCD_1234_5678; t.shift_amt = 47; t.shift_dir = 0; gen2drv.put(t);

t = new(); t.cluster_out = 48'hABCD_1234_5678; t.shift_amt = 47; t.shift_dir = 1; gen2drv.put(t);

// 6) shift_amt = 48: exactly out-of-width -> saturation kicks in

t = new(); t.cluster_out = 48'hABCD_1234_5678; t.shift_amt = 48; t.shift_dir = 0; gen2drv.put(t);

t = new(); t.cluster_out = 48'hABCD_1234_5678; t.shift_amt = 48; t.shift_dir = 1; gen2drv.put(t);

// 7) shift_amt = 63 (max representable in 6 bits)

t = new(); t.cluster_out = 48'hABCD_1234_5678; t.shift_amt = 63; t.shift_dir = 0; gen2drv.put(t);

t = new(); t.cluster_out = 48'hABCD_1234_5678; t.shift_amt = 63; t.shift_dir = 1; gen2drv.put(t);

// 8) left-shift overflow past bit31: verify correct truncation to low 32 bits

t = new(); t.cluster_out = 48'h0000_0000_FFFF; t.shift_amt = 20; t.shift_dir = 1; gen2drv.put(t);

t = new(); t.cluster_out = 48'h0000_FFFF_FFFF; t.shift_amt = 16; t.shift_dir = 1; gen2drv.put(t);

// 9) right-shift crossing bit31/bit47 boundary: mix of real data + sign fill in result

t = new(); t.cluster_out = 48'hFFFF_8000_0001; t.shift_amt = 16; t.shift_dir = 0; gen2drv.put(t);

t = new(); t.cluster_out = 48'h0000_7FFF_FFFF; t.shift_amt = 16; t.shift_dir = 0; gen2drv.put(t);

// ---------------- CONSTRAINED-RANDOM (CRV) ----------------

repeat (num_random) begin

t = new();

assert (t.randomize()) else $error("[GEN] randomize() failed");

gen2drv.put(t);

end

endtask

endclass

// -------------------------------------------------------------

// 6. DRIVER

// -------------------------------------------------------------

class bs_driver;

virtual bs_if vif;

mailbox #(bs_transaction) gen2drv;

mailbox #(bs_transaction) drv2mon;

function new(virtual bs_if vif, mailbox #(bs_transaction) gen2drv, mailbox #(bs_transaction) drv2mon);

this.vif     = vif;

this.gen2drv = gen2drv;

this.drv2mon = drv2mon;

endfunction

task run(int num_txns);

bs_transaction t;

repeat (num_txns) begin

gen2drv.get(t);

@(posedge vif.clk);

vif.cluster_out <= t.cluster_out;

vif.shift_amt   <= t.shift_amt;

vif.shift_dir   <= t.shift_dir;

drv2mon.put(t);

end

endtask

// Sequential-mode entry point: apply one transaction's stimulus and

// return once the clock edge has been consumed. Caller (env.run())

// is responsible for calling monitor.sample_one() next, before

// driving anything new -- this is what keeps drive/sample in lockstep.

task drive_one(bs_transaction t);

@(posedge vif.clk);

vif.cluster_out <= t.cluster_out;

vif.shift_amt   <= t.shift_amt;

vif.shift_dir   <= t.shift_dir;

endtask

endclass

// -------------------------------------------------------------

// 7. MONITOR — samples DUT output after combinational settle,

//    also samples the coverage model.

// -------------------------------------------------------------

class bs_monitor;

virtual bs_if vif;

mailbox #(bs_transaction) drv2mon;

mailbox #(bs_result)      mon2sb;

// functional coverage: shift_amt buckets x shift_dir x operand sign x MSB

covergroup cg_bs;

cp_amt: coverpoint vif.shift_amt {

bins zero        = {0};

bins low         = {[1:15]};

bins mid         = {[16:31]};

bins near_bound  = {[45:47]};   // right at 48-bit width edge

bins at_bound    = {48};        // exactly out-of-width

bins above_bound = {[49:62]};

bins max_val     = {63};

}

cp_dir: coverpoint vif.shift_dir {

bins right_arith = {0};

bins left_logic  = {1};

}

cp_sign: coverpoint vif.cluster_out[47] {

bins positive = {0};

bins negative = {1};

}

cp_special: coverpoint vif.cluster_out {

bins all_zero  = {48'h0000_0000_0000};

bins all_one   = {48'hFFFF_FFFF_FFFF};

bins msb_only  = {48'h8000_0000_0000};

bins all_but_msb = {48'h7FFF_FFFF_FFFF};

bins others    = default;

}

cross cp_amt, cp_dir, cp_sign;

endgroup

function new(virtual bs_if vif, mailbox #(bs_transaction) drv2mon, mailbox #(bs_result) mon2sb);

this.vif     = vif;

this.drv2mon = drv2mon;

this.mon2sb  = mon2sb;

cg_bs = new();

endfunction

task run(int num_txns);

bs_transaction t;

bs_result r;

repeat (num_txns) begin

drv2mon.get(t);

@(posedge vif.clk);

#1; // allow combinational logic to settle

r = new();

r.txn          = t;

r.shift_result = vif.shift_result;

cg_bs.sample();

mon2sb.put(r);

end

endtask

// Sequential-mode entry point: call this immediately after

// driver.drive_one(t) for the SAME t, with no other stimulus applied

// in between. drive_one() already consumed the clock edge and issued

// the non-blocking input assignment, so we only need a small settle

// delay here -- NOT another @(posedge vif.clk), which would let the

// driver race ahead by a full cycle before we sample (the bug this

// replaces).

task sample_one(bs_transaction t, output bs_result r);

#1; // allow combinational logic to settle after drive_one()'s NBA update

r = new();

r.txn          = t;

r.shift_result = vif.shift_result;

cg_bs.sample();

endtask

function real get_coverage();

return cg_bs.get_coverage();

endfunction

endclass

// -------------------------------------------------------------

// 8. SCOREBOARD — independent reference model + self-checking compare

// -------------------------------------------------------------

class bs_scoreboard;

mailbox #(bs_result) mon2sb;

int pass_cnt = 0;

int fail_cnt = 0;

function new(mailbox #(bs_result) mon2sb);

this.mon2sb = mon2sb;

endfunction

// Independent reference model — deliberately NOT copy-pasted from the

// RTL's exact expression, computed via explicit bit-width reasoning

// instead, so it doesn't just repeat a shared bug.

function logic [31:0] predict(bs_transaction t);

logic [47:0] shifted;

if (t.shift_dir == 1'b1) begin

// logical left shift: out-of-width shift amounts saturate to 0

if (t.shift_amt >= 48) shifted = 48'd0;

else                    shifted = t.cluster_out << t.shift_amt;

end else begin

// arithmetic right shift: out-of-width shift amounts saturate

// to 48 copies of the sign bit

if (t.shift_amt >= 48) shifted = {48{t.cluster_out[47]}};

else                    shifted = $signed(t.cluster_out) >>> t.shift_amt;

end

return shifted[31:0];

endfunction

// Single-transaction entry point used by the sequential env.run() flow.

task check(bs_result r);

logic [31:0] expected;

expected = predict(r.txn);

if (r.shift_result === expected) begin

pass_cnt++;

$display("[PASS] %s -> shift_result=%08h", r.txn.sprint(), r.shift_result);

end else begin

fail_cnt++;

$display("[FAIL] %s -> got=%08h exp=%08h", r.txn.sprint(), r.shift_result, expected);

end

endtask

// Legacy mailbox-driven loop (unused by the current sequential env,

// kept only so nothing else that might reference it breaks).

task run(int num_txns);

bs_result r;

repeat (num_txns) begin

mon2sb.get(r);

check(r);

end

endtask

endclass

// -------------------------------------------------------------

// 9. ENVIRONMENT

// -------------------------------------------------------------

class bs_env;

virtual bs_if vif;

mailbox #(bs_transaction) gen2drv;

mailbox #(bs_transaction) drv2mon;

mailbox #(bs_result)      mon2sb;

bs_generator  gen;

bs_driver     drv;

bs_monitor    mon;

bs_scoreboard sb;

int num_directed = 20;   // keep in sync with generator's directed-case count

int num_random    = 200;

int num_txns;

function new(virtual bs_if vif, int num_random = 200);

this.vif = vif;

this.num_random = num_random;

gen2drv = new();

drv2mon = new();

mon2sb  = new();

gen = new(gen2drv, num_random);

drv = new(vif, gen2drv, drv2mon);

mon = new(vif, drv2mon, mon2sb);

sb  = new(mon2sb);

endfunction

task run();

bs_transaction t;

bs_result r;

num_txns = num_directed + num_random;

// Generator has no clock dependency -> fine to run up front and

// fully populate gen2drv before driving starts.

gen.run();

// Drive -> sample -> check strictly one transaction at a time.

// (Previously this used fork/join with drv/mon/sb as independent

//  concurrent processes; that let the driver race ahead of the

//  monitor by one clock edge, so the monitor ended up sampling

//  transaction i+1's settled output while still labeling it with

//  transaction i's data. Sequential drive->sample->check removes

//  that race entirely: nothing changes the DUT's inputs again

//  until the current transaction has been fully sampled and

//  scored.)

repeat (num_txns) begin

gen2drv.get(t);

drv.drive_one(t);

mon.sample_one(t, r);

sb.check(r);

end

$display("\n================ BARREL_SHIFTER UNIT TB SUMMARY ================");

$display(" TOTAL=%0d  PASS=%0d  FAIL=%0d", num_txns, sb.pass_cnt, sb.fail_cnt);

$display(" FUNCTIONAL COVERAGE = %0.2f %%", mon.get_coverage());

if (sb.fail_cnt == 0)

$display(" RESULT: ALL CHECKS PASSED");

else

$display(" RESULT: %0d CHECK(S) FAILED", sb.fail_cnt);

$display("==================================================================\n");

endtask

endclass

// -------------------------------------------------------------

// 10. TEST

// -------------------------------------------------------------

class bs_test;

bs_env env;

function new(virtual bs_if vif, int num_random = 200);

env = new(vif, num_random);

endfunction

task run();

env.run();

endtask

endclass

// -------------------------------------------------------------

// 11. TB TOP

// -------------------------------------------------------------

module tb_top;

bit clk = 0;

always #5 clk = ~clk;   // sequencing clock; DUT itself is combinational

bs_if vif(clk);

barrel_shifter dut (

.cluster_out  (vif.cluster_out),

.shift_amt    (vif.shift_amt),

.shift_dir    (vif.shift_dir),

.shift_result (vif.shift_result)

);

bs_test test;

initial begin

vif.cluster_out = 48'd0;

vif.shift_amt   = 6'd0;

vif.shift_dir   = 1'b0;

test = new(vif, 200);  // 200 constrained-random txns on top of directed set

test.run();

$finish;

end

endmodule