// =============================================================

// dsu_decoder_tb.sv

// Full SV verification environment for GARUDA dsu_decoder.v

// Architecture matches barrel_shifter_tb.sv (sequential drive->

// sample->check env -- no fork/join, no mailbox -- avoiding the

// driver/monitor race found and fixed during barrel_shifter

// verification).

//

// Requires a simulator with full IEEE1800 support (covergroup,

// assert property, constraint solver) -- e.g. Cadence Xcelium,

// Aldec Riviera-PRO, Questa, VCS. NOT Icarus Verilog.

// =============================================================

`timescale 1ns / 1ps

// -------------------------------------------------------------

// 0. DEFINES (inlined from dsu_defs.vh)

// -------------------------------------------------------------

`ifndef DSU_DEFS_VH

`define DSU_DEFS_VH

`define ACC_FX  2'b00

`define ACC_FY  2'b01

`define ACC_MAG 2'b10

`define MAC_CTRL_W      7

`define MAC_CTRL_EN     6

`define MAC_CTRL_ADDSUB 5

`define MAC_CTRL_CLEAR  4

`define MAC_CTRL_LOAD   3

`define MAC_CTRL_ABS    2

`define MAC_CTRL_DOT    1

`define MAC_CTRL_FLUSH  0

`define FUNCT5_MAC_SEL  5'd0

`define FUNCT5_MACSUB   5'd1

`define FUNCT5_MACABS   5'd2

`define FUNCT5_MACDOT   5'd3

`define FUNCT5_MACLOAD  5'd4

`define FUNCT5_MACCLEAR 5'd5

`define FUNCT5_MACSAT   5'd6

`define FUNCT5_MACRD_LO 5'd7

`define FUNCT5_MACRD_HI 5'd8

`define FUNCT5_MACSHIFT 5'd9

`endif

// -------------------------------------------------------------

// 1. DUT: dsu_decoder.v (pasted verbatim, self-contained file)

// -------------------------------------------------------------

module dsu_decoder (

input wire [31:0] instr,

input wire        dsu_en,

input wire        flush,

output wire [`MAC_CTRL_W-1:0] control,

output wire [1:0]             acc_sel,

output wire sat_op,

output wire       shift_op,

output wire [5:0] shift_amt,

output wire       shift_dir,

output wire [1:0] shift_acc_sel,

output wire rd_lo_op,

output wire rd_hi_op,

output wire [1:0] result_src,

output wire       writes_regfile,

output wire [4:0] rd_addr,

output wire illegal_instr,

output wire [4:0] funct5,

output wire [2:0] funct3,

output wire       is_custom0

);

wire [6:0]  opcode    = instr[6:0];

wire [4:0]  rd_f      = instr[11:7];

assign      funct3    = instr[14:12];

wire [4:0]  rs1_f     = instr[19:15];

wire [4:0]  rs2_f     = instr[24:20];

wire [1:0]  acc_sel_r = instr[26:25];

wire [4:0]  funct5_r  = instr[31:27];

wire [11:0] imm_i     = instr[31:20];

assign is_custom0 = (opcode == 7'b0001011);

wire is_rtype   = is_custom0 & (funct3 == 3'b000);

wire is_itype   = is_custom0 & (funct3 == 3'b001);

wire op_mac_sel  = is_rtype & (funct5_r == `FUNCT5_MAC_SEL);

wire op_macsub   = is_rtype & (funct5_r == `FUNCT5_MACSUB);

wire op_macabs   = is_rtype & (funct5_r == `FUNCT5_MACABS);

wire op_macdot   = is_rtype & (funct5_r == `FUNCT5_MACDOT);

wire op_macload  = is_rtype & (funct5_r == `FUNCT5_MACLOAD);

wire op_macclear = is_rtype & (funct5_r == `FUNCT5_MACCLEAR);

wire op_macsat   = is_rtype & (funct5_r == `FUNCT5_MACSAT);

wire op_rd_lo    = is_rtype & (funct5_r == `FUNCT5_MACRD_LO);

wire op_rd_hi    = is_rtype & (funct5_r == `FUNCT5_MACRD_HI);

wire op_macshift = is_itype;

wire [1:0] acc_sel_eff = is_itype ? imm_i[7:6] : acc_sel_r;

assign acc_sel       = acc_sel_eff;

assign shift_acc_sel = imm_i[7:6];

wire any_legal_op = op_mac_sel | op_macsub | op_macabs | op_macdot | op_macload | op_macclear | op_macsat | op_rd_lo | op_rd_hi | op_macshift;

wire bad_acc_sel  = (acc_sel_eff == 2'b11);

wire bad_funct3   = is_custom0 & ~(is_rtype | is_itype);

assign illegal_instr = is_custom0 & dsu_en & (~any_legal_op | bad_acc_sel | bad_funct3);

wire compute_op = op_mac_sel | op_macsub | op_macabs | op_macdot | op_macload | op_macclear;

wire mac_en     = compute_op & dsu_en & ~illegal_instr;

wire mac_addsub = op_macsub;

wire mac_clear  = op_macclear;

wire mac_load   = op_macload;

wire mac_abs    = op_macabs;

wire mac_dot    = op_macdot;

wire mac_flush  = flush;

assign control = {

mac_en,

mac_addsub,

mac_clear,

mac_load,

mac_abs,

mac_dot,

mac_flush

};

assign sat_op   = op_macsat & dsu_en & ~illegal_instr;

assign shift_op = op_macshift & dsu_en & ~illegal_instr;

assign rd_lo_op = op_rd_lo & dsu_en & ~illegal_instr;

assign rd_hi_op = op_rd_hi & dsu_en & ~illegal_instr;

assign shift_amt  = imm_i [5:0];

assign shift_dir  = imm_i [8];

assign result_src = op_rd_lo ? 2'b01 : op_rd_hi ? 2'b10 : op_macshift ? 2'b11 : 2'b00;

assign writes_regfile = (op_rd_lo | op_rd_hi | op_macshift) & dsu_en & ~illegal_instr;

assign rd_addr        = rd_f;

assign funct5 = funct5_r;

endmodule

// -------------------------------------------------------------

// 2. INTERFACE (+ bound SVA assertions)

// -------------------------------------------------------------

interface dsu_if (input bit clk);

logic [31:0] instr;

logic        dsu_en;

logic        flush;

logic [`MAC_CTRL_W-1:0] control;

logic [1:0]             acc_sel;

logic                   sat_op;

logic                   shift_op;

logic [5:0]             shift_amt;

logic                   shift_dir;

logic [1:0]             shift_acc_sel;

logic                   rd_lo_op;

logic                   rd_hi_op;

logic [1:0]             result_src;

logic                   writes_regfile;

logic [4:0]             rd_addr;

logic                   illegal_instr;

logic [4:0]             funct5;

logic [2:0]             funct3;

logic                   is_custom0;

// ---------------------------------------------------------

// Concurrent assertions -- spec invariants, independent of the

// scoreboard's reference-model check. Sampled one clk after the

// driver applies stimulus (same alignment the sequential env uses).

// ---------------------------------------------------------

// A1: any non-Custom-0 opcode can never be flagged illegal (illegal_instr

//     only means "recognized as Custom-0 but malformed", not "unknown opcode")

property p_non_custom0_never_illegal;

@(posedge clk) (is_custom0 == 1'b0) |-> (illegal_instr == 1'b0);

endproperty

a_non_custom0_never_illegal: assert property (p_non_custom0_never_illegal)

else $error("[SVA-FAIL] illegal_instr asserted for a non-Custom-0 opcode");

// A2: illegal_instr can never fire while the unit is disabled

property p_disabled_never_illegal;

@(posedge clk) (dsu_en == 1'b0) |-> (illegal_instr == 1'b0);

endproperty

a_disabled_never_illegal: assert property (p_disabled_never_illegal)

else $error("[SVA-FAIL] illegal_instr asserted while dsu_en=0");

// A3: flush passes through to control[0] unconditionally -- regardless

//     of instruction validity, dsu_en, or opcode

property p_flush_passthrough;

@(posedge clk) (control[0] == flush);

endproperty

a_flush_passthrough: assert property (p_flush_passthrough)

else $error("[SVA-FAIL] control[0] (mac_flush) != flush: control[0]=%0b flush=%0b", control[0], flush);

// A4: funct5/funct3/is_custom0/rd_addr are unconditional raw decodes of

//     instr, independent of legality/dsu_en -- verifies they're always live

property p_funct5_unconditional;

@(posedge clk) (funct5 == instr[31:27]);

endproperty

a_funct5_unconditional: assert property (p_funct5_unconditional)

else $error("[SVA-FAIL] funct5 != instr[31:27]");

property p_funct3_unconditional;

@(posedge clk) (funct3 == instr[14:12]);

endproperty

a_funct3_unconditional: assert property (p_funct3_unconditional)

else $error("[SVA-FAIL] funct3 != instr[14:12]");

property p_is_custom0_unconditional;

@(posedge clk) (is_custom0 == (instr[6:0] == 7'b0001011));

endproperty

a_is_custom0_unconditional: assert property (p_is_custom0_unconditional)

else $error("[SVA-FAIL] is_custom0 != (opcode==0x0B)");

property p_rd_addr_unconditional;

@(posedge clk) (rd_addr == instr[11:7]);

endproperty

a_rd_addr_unconditional: assert property (p_rd_addr_unconditional)

else $error("[SVA-FAIL] rd_addr != instr[11:7]");

// A5: writes_regfile implies at most one of {rd_lo_op, rd_hi_op, shift_op}

property p_writeback_source_onehot0;

@(posedge clk) writes_regfile |-> $onehot0({rd_lo_op, rd_hi_op, shift_op});

endproperty

a_writeback_source_onehot0: assert property (p_writeback_source_onehot0)

else $error("[SVA-FAIL] writes_regfile asserted with multiple writeback sources active");

// A6: mac_en (control[6]) implies at most one of the 5 MAC modifier

//     bits (addsub/clear/load/abs/dot = control[5:1]) is set -- MAC_SEL

//     legitimately sets zero of them, every other compute op sets one

property p_mac_modifiers_onehot0;

@(posedge clk) control[6] |-> $onehot0(control[5:1]);

endproperty

a_mac_modifiers_onehot0: assert property (p_mac_modifiers_onehot0)

else $error("[SVA-FAIL] mac_en asserted with multiple MAC modifier bits set: control=%07b", control);

// A7: illegal_instr gates off every functional enable except the

//     unconditional taps (funct5/funct3/is_custom0/rd_addr) and the

//     flush passthrough

property p_illegal_gates_everything;

@(posedge clk) illegal_instr |-> (control[6] == 1'b0 && sat_op == 1'b0 &&

shift_op == 1'b0 && rd_lo_op == 1'b0 &&

rd_hi_op == 1'b0 && writes_regfile == 1'b0);

endproperty

a_illegal_gates_everything: assert property (p_illegal_gates_everything)

else $error("[SVA-FAIL] a functional enable stayed active while illegal_instr=1");

// A8: acc_sel == 2'b11 while enabled and recognized as Custom-0 must

//     always be flagged illegal (this is one of the two illegal conditions)

property p_bad_accsel_is_illegal;

@(posedge clk) (is_custom0 && dsu_en && acc_sel == 2'b11) |-> illegal_instr;

endproperty

a_bad_accsel_is_illegal: assert property (p_bad_accsel_is_illegal)

else $error("[SVA-FAIL] acc_sel=11 with dsu_en & is_custom0 did not flag illegal_instr");

// A9: shift_amt/shift_dir/shift_acc_sel are unconditional raw slices of

//     instr[31:20] -- true even when the instruction is NOT a MACSHIFT

//     (documented decoder characteristic, not a bug: these wires are

//     only *meaningful* when shift_op=1, but they are always *live*)

property p_shift_fields_unconditional;

@(posedge clk) (shift_amt == instr[25:20] && shift_dir == instr[28] && shift_acc_sel == instr[27:26]);

endproperty

a_shift_fields_unconditional: assert property (p_shift_fields_unconditional)

else $error("[SVA-FAIL] shift_amt/shift_dir/shift_acc_sel are not raw instr[31:20] slices");

endinterface

// -------------------------------------------------------------

// 3. TRANSACTION -- rand + constraint => real CRV

// -------------------------------------------------------------

typedef enum {

OP_MAC_SEL, OP_MACSUB, OP_MACABS, OP_MACDOT, OP_MACLOAD,

OP_MACCLEAR, OP_MACSAT, OP_RD_LO, OP_RD_HI, OP_MACSHIFT,

OP_ILLEGAL_FUNCT3, OP_ILLEGAL_FUNCT5_MID, OP_ILLEGAL_FUNCT5_ALIAS9,

OP_ILLEGAL_FUNCT5_MAX, OP_ILLEGAL_ACCSEL_RTYPE, OP_ILLEGAL_ACCSEL_ITYPE,

OP_NON_CUSTOM0

} dsu_op_e;

class dsu_transaction;

rand dsu_op_e op;

rand bit [4:0] rd, rs1, rs2;

rand bit [1:0] acc_sel;        // R-type

rand bit [5:0] shift_amt;      // I-type

rand bit       shift_dir;

rand bit [1:0] shift_acc_sel;  // I-type

rand bit       dsu_en;

rand bit       flush;

rand bit [2:0] bad_funct3;

rand bit [4:0] bad_funct5;

rand bit [4:0] nc_funct5;      // funct5/funct3 field values to use when

rand bit [2:0] nc_funct3;      // op==OP_NON_CUSTOM0 -- these bits are just

// "don't care" for a non-Custom-0 opcode,

// but the DUT still decodes them

// unconditionally (per A4a/A4b), so they

// need to be varied to close the

// cp_funct5/cp_funct3 x cp_is_custom0

// cross coverage on the is_custom0=0 side.

constraint c_op_dist {

op dist {

OP_MAC_SEL               :/ 10,

OP_MACSUB                :/ 10,

OP_MACABS                :/ 10,

OP_MACDOT                :/ 10,

OP_MACLOAD               :/ 10,

OP_MACCLEAR              :/ 10,

OP_MACSAT                :/ 10,

OP_RD_LO                 :/ 10,

OP_RD_HI                 :/ 10,

OP_MACSHIFT               :/ 10,

OP_ILLEGAL_FUNCT3         :/ 6,

OP_ILLEGAL_FUNCT5_MID     :/ 6,

OP_ILLEGAL_FUNCT5_ALIAS9  :/ 6,

OP_ILLEGAL_FUNCT5_MAX     :/ 6,

OP_ILLEGAL_ACCSEL_RTYPE   :/ 6,

OP_ILLEGAL_ACCSEL_ITYPE   :/ 6,

OP_NON_CUSTOM0            :/ 4

};

}

constraint c_dsu_en_dist { dsu_en dist { 1'b1 :/ 80, 1'b0 :/ 20 }; }

constraint c_flush_dist  { flush  dist { 1'b0 :/ 70, 1'b1 :/ 30 }; }

constraint c_accsel_default {

// for ops that aren't specifically exercising a bad acc_sel, keep

// it legal so THAT particular fault doesn't mask the intended one

if (op != OP_ILLEGAL_ACCSEL_RTYPE) acc_sel != 2'b11;

if (op != OP_ILLEGAL_ACCSEL_ITYPE) shift_acc_sel != 2'b11;

}

constraint c_bad_funct3_range { bad_funct3 inside {3'b010, 3'b011, 3'b100, 3'b101, 3'b110, 3'b111}; }

constraint c_bad_funct5_mid   { bad_funct5 inside {[5'd10 : 5'd30]}; }

constraint c_shift_amt_dist {

shift_amt dist { 6'd0 :/ 15, [6'd1:6'd30] :/ 30, [6'd31:6'd62] :/ 40, 6'd63 :/ 15 };

}

function bit [31:0] pack();

bit [31:0] instr;

bit [6:0] opcode_custom0 = 7'b0001011;

bit [6:0] opcode_other   = 7'b0110011;

bit [4:0] f5;

bit [2:0] f3;

case (op)

OP_MAC_SEL  : f5 = `FUNCT5_MAC_SEL;

OP_MACSUB   : f5 = `FUNCT5_MACSUB;

OP_MACABS   : f5 = `FUNCT5_MACABS;

OP_MACDOT   : f5 = `FUNCT5_MACDOT;

OP_MACLOAD  : f5 = `FUNCT5_MACLOAD;

OP_MACCLEAR : f5 = `FUNCT5_MACCLEAR;

OP_MACSAT   : f5 = `FUNCT5_MACSAT;

OP_RD_LO    : f5 = `FUNCT5_MACRD_LO;

OP_RD_HI    : f5 = `FUNCT5_MACRD_HI;

OP_ILLEGAL_FUNCT5_MID   : f5 = bad_funct5;

OP_ILLEGAL_FUNCT5_ALIAS9: f5 = `FUNCT5_MACSHIFT;   // =9, legal for I-type, illegal for R-type

OP_ILLEGAL_FUNCT5_MAX   : f5 = 5'd31;

default: f5 = 5'd0;

endcase

f3 = (op == OP_ILLEGAL_FUNCT3) ? bad_funct3 :

(op == OP_MACSHIFT || op == OP_ILLEGAL_ACCSEL_ITYPE) ? 3'b001 : 3'b000;

case (op)

OP_MAC_SEL, OP_MACSUB, OP_MACABS, OP_MACDOT, OP_MACLOAD, OP_MACCLEAR,

OP_MACSAT, OP_RD_LO, OP_RD_HI,

OP_ILLEGAL_FUNCT5_MID, OP_ILLEGAL_FUNCT5_ALIAS9, OP_ILLEGAL_FUNCT5_MAX,

OP_ILLEGAL_ACCSEL_RTYPE, OP_ILLEGAL_FUNCT3: begin

instr = {f5, acc_sel, rs2, rs1, f3, rd, opcode_custom0};

end

OP_MACSHIFT, OP_ILLEGAL_ACCSEL_ITYPE: begin

automatic bit [11:0] imm_i = {3'b000, shift_dir, shift_acc_sel, shift_amt};

instr = {imm_i, rs1, 3'b001, rd, opcode_custom0};

end

OP_NON_CUSTOM0: begin

instr = {nc_funct5, acc_sel, rs2, rs1, nc_funct3, rd, opcode_other};

end

default: instr = 32'd0;

endcase

return instr;

endfunction

function string name();

return op.name();

endfunction

endclass

// -------------------------------------------------------------

// 4. RESULT (monitor -> scoreboard payload)

// -------------------------------------------------------------

class dsu_result;

dsu_transaction txn;

logic [`MAC_CTRL_W-1:0] control;

logic [1:0]  acc_sel;

logic        sat_op;

logic        shift_op;

logic [5:0]  shift_amt;

logic        shift_dir;

logic [1:0]  shift_acc_sel;

logic        rd_lo_op;

logic        rd_hi_op;

logic [1:0]  result_src;

logic        writes_regfile;

logic [4:0]  rd_addr;

logic        illegal_instr;

logic [4:0]  funct5;

logic [2:0]  funct3;

logic        is_custom0;

endclass

// -------------------------------------------------------------

// 5. GENERATOR -- critical directed corner cases + CRV

// -------------------------------------------------------------

class dsu_generator;

dsu_transaction q[$];

int num_random;

function new(int num_random = 200);

this.num_random = num_random;

endfunction

// Plain field-assignment builder -- deliberately avoids named/default

// function arguments (mixing positional+named calls with per-argument

// defaults is legal SystemVerilog, but this project has repeatedly hit

// simulator-support gaps on less-common syntax, so directed test

// construction sticks to the simplest, most portable form: create,

// then set every field explicitly).

function dsu_transaction mk();

dsu_transaction t = new();

t.rd = 5'd3; t.rs1 = 5'd5; t.rs2 = 5'd7;

t.acc_sel = 2'b01; t.shift_amt = 6'd12; t.shift_dir = 1'b1; t.shift_acc_sel = 2'b10;

t.dsu_en = 1'b1; t.flush = 1'b0; t.bad_funct3 = 3'b101; t.bad_funct5 = 5'd15;

t.nc_funct5 = 5'd0; t.nc_funct3 = 3'd0;

return t;

endfunction

function void build();

dsu_transaction t;

dsu_op_e legal_ops[10] = '{OP_MAC_SEL, OP_MACSUB, OP_MACABS, OP_MACDOT, OP_MACLOAD,

OP_MACCLEAR, OP_MACSAT, OP_RD_LO, OP_RD_HI, OP_MACSHIFT};

// 1) Each of the 10 legal ops, dsu_en=1, flush=0

foreach (legal_ops[i]) begin

t = mk(); t.op = legal_ops[i];

q.push_back(t);

end

// 2) Each of the 10 legal ops again, dsu_en=0 -> verify full gating,

//    EXCEPT the unconditional taps (funct5/funct3/is_custom0/rd_addr)

foreach (legal_ops[i]) begin

t = mk(); t.op = legal_ops[i]; t.dsu_en = 1'b0;

q.push_back(t);

end

// 3) Illegal: bad funct3 (not 000/001)

t = mk(); t.op = OP_ILLEGAL_FUNCT3; t.bad_funct3 = 3'b101; q.push_back(t);

t = mk(); t.op = OP_ILLEGAL_FUNCT3; t.bad_funct3 = 3'b010; q.push_back(t); // boundary: first invalid value above 001

// 4) Illegal: bad funct5 in R-type, middle of range

t = mk(); t.op = OP_ILLEGAL_FUNCT5_MID; t.bad_funct5 = 5'd15; q.push_back(t);

// 5) CRITICAL CORNER: funct5=9 in an R-type instruction. 9 is

//    FUNCT5_MACSHIFT's own encoding, but MACSHIFT is only reached

//    via is_itype (funct3=001), never via a funct5 compare in the

//    R-type branch -- so this must be flagged illegal, not decoded

//    as some kind of "R-type shift".

t = mk(); t.op = OP_ILLEGAL_FUNCT5_ALIAS9; q.push_back(t);

// 6) Illegal: bad funct5 at the max representable value (31)

t = mk(); t.op = OP_ILLEGAL_FUNCT5_MAX; q.push_back(t);

// 7) Boundary sanity: funct5=8 (MACRD_HI, last legal value) must

//    NOT be illegal -- guards against an off-by-one in the decoder

t = mk(); t.op = OP_RD_HI; q.push_back(t);

// 8) Illegal: acc_sel=11 on an otherwise-legal R-type op

t = mk(); t.op = OP_ILLEGAL_ACCSEL_RTYPE; t.acc_sel = 2'b11; q.push_back(t);

// 9) Boundary sanity: acc_sel=10 (ACC_MAG, last legal value) must be legal

t = mk(); t.op = OP_MAC_SEL; t.acc_sel = 2'b10; q.push_back(t);

// 10) Illegal: shift_acc_sel=11 on an otherwise-legal I-type op

t = mk(); t.op = OP_ILLEGAL_ACCSEL_ITYPE; t.shift_acc_sel = 2'b11; q.push_back(t);

// 11) Non-Custom-0 opcode entirely

t = mk(); t.op = OP_NON_CUSTOM0; q.push_back(t);

// 12) Flush passthrough while everything else is off (non-custom0, dsu_en=0)

t = mk(); t.op = OP_NON_CUSTOM0; t.dsu_en = 1'b0; t.flush = 1'b1; q.push_back(t);

// 13) Flush passthrough alongside an illegal instruction

t = mk(); t.op = OP_ILLEGAL_FUNCT5_MID; t.bad_funct5 = 5'd20; t.flush = 1'b1; q.push_back(t);

// 14) Flush passthrough alongside a legal, enabled compute op

//     (both mac_en and the flush bit must be set simultaneously)

t = mk(); t.op = OP_MAC_SEL; t.flush = 1'b1; q.push_back(t);

// 15) rd_addr boundary: rd=0 (x0) -- decoder does not special-case

//     x0, that filtering belongs to regfile/writeback, not here

t = mk(); t.op = OP_RD_LO; t.rd = 5'd0; q.push_back(t);

// 16) rd_addr boundary: rd=31 (x31, last register)

t = mk(); t.op = OP_RD_HI; t.rd = 5'd31; q.push_back(t);

// 17) shift_amt boundary: 0

t = mk(); t.op = OP_MACSHIFT; t.shift_amt = 6'd0; t.shift_dir = 1'b0; q.push_back(t);

// 18) shift_amt boundary: 63 (max representable)

t = mk(); t.op = OP_MACSHIFT; t.shift_amt = 6'd63; t.shift_dir = 1'b1; q.push_back(t);

// 19) CRITICAL CORNER: shift_amt/shift_dir/shift_acc_sel are raw,

//     unconditional slices of instr[31:20] -- verify they still

//     read out correctly even when the instruction is an R-type

//     (not a MACSHIFT at all). Confirmed by SVA a_shift_fields_unconditional

//     on every transaction; this entry specifically forces an R-type

//     encoding through the queue to exercise that check.

t = mk(); t.op = OP_MAC_SEL; t.rd = 5'd3; t.rs1 = 5'd5; t.rs2 = 5'd7; t.acc_sel = 2'b01;

q.push_back(t);

// 20) COVERAGE CLOSURE: funct5/funct3 are unconditional raw decodes

//     (A4a/A4b) -- every funct5/funct3 value is reachable with

//     is_custom0=0 too, but that side of the cp_funct5/cp_funct3 x

//     cp_is_custom0 x cp_dsu_en cross needs the field explicitly

//     swept, since it has no bearing on legality for a non-Custom-0

//     opcode and constrained-random alone won't reliably hit every

//     bin given OP_NON_CUSTOM0's low relative weight in c_op_dist.

begin

bit [4:0] funct5_sweep[12] = '{5'd0, 5'd1, 5'd2, 5'd3, 5'd4, 5'd5, 5'd6, 5'd7, 5'd8, 5'd9, 5'd15, 5'd31};

bit [2:0] funct3_sweep[8]  = '{3'd0, 3'd1, 3'd2, 3'd3, 3'd4, 3'd5, 3'd6, 3'd7};

foreach (funct5_sweep[i]) begin

t = mk(); t.op = OP_NON_CUSTOM0; t.nc_funct5 = funct5_sweep[i]; t.nc_funct3 = 3'd0; t.dsu_en = 1'b1;

q.push_back(t);

t = mk(); t.op = OP_NON_CUSTOM0; t.nc_funct5 = funct5_sweep[i]; t.nc_funct3 = 3'd0; t.dsu_en = 1'b0;

q.push_back(t);

end

foreach (funct3_sweep[i]) begin

t = mk(); t.op = OP_NON_CUSTOM0; t.nc_funct5 = 5'd0; t.nc_funct3 = funct3_sweep[i]; t.dsu_en = 1'b1;

q.push_back(t);

t = mk(); t.op = OP_NON_CUSTOM0; t.nc_funct5 = 5'd0; t.nc_funct3 = funct3_sweep[i]; t.dsu_en = 1'b0;

q.push_back(t);

end

end

// ---------------- CONSTRAINED-RANDOM (CRV) ----------------

repeat (num_random) begin

dsu_transaction t = new();

assert (t.randomize()) else $error("[GEN] randomize() failed");

q.push_back(t);

end

endfunction

endclass

// -------------------------------------------------------------

// 6. DRIVER

// -------------------------------------------------------------

class dsu_driver;

virtual dsu_if vif;

function new(virtual dsu_if vif);

this.vif = vif;

endfunction

task drive_one(dsu_transaction t);

@(posedge vif.clk);

vif.instr  <= t.pack();

vif.dsu_en <= t.dsu_en;

vif.flush  <= t.flush;

endtask

endclass

// -------------------------------------------------------------

// 7. MONITOR

// -------------------------------------------------------------

class dsu_monitor;

virtual dsu_if vif;

covergroup cg_dsu;

cp_funct3: coverpoint vif.funct3 {

bins rtype = {3'b000};

bins itype = {3'b001};

bins illegal_funct3[] = {[3'b010:3'b111]};

}

cp_funct5: coverpoint vif.funct5 {

bins mac_sel  = {0};

bins macsub   = {1};

bins macabs   = {2};

bins macdot   = {3};

bins macload  = {4};

bins macclear = {5};

bins macsat   = {6};

bins rd_lo    = {7};

bins rd_hi    = {8};

bins alias9   = {9};        // legal only as I-type, illegal as R-type

bins mid_illegal = {[10:30]};

bins max_illegal = {31};

}

cp_is_custom0: coverpoint vif.is_custom0;

cp_dsu_en:     coverpoint vif.dsu_en;

cp_flush:      coverpoint vif.flush;

cp_illegal:    coverpoint vif.illegal_instr;

cp_acc_sel:    coverpoint vif.acc_sel {

bins fx  = {2'b00};

bins fy  = {2'b01};

bins mag = {2'b10};

bins bad = {2'b11};

}

cp_shift_amt: coverpoint vif.shift_amt {

bins zero = {0};

bins mid  = {[1:62]};

bins max_val  = {63};

}

cp_result_src: coverpoint vif.result_src;

cross cp_funct5, cp_dsu_en, cp_is_custom0;

cross cp_funct3, cp_is_custom0;

cross cp_acc_sel, cp_illegal;

endgroup

function new(virtual dsu_if vif);

this.vif = vif;

cg_dsu = new();

endfunction

task sample_one(dsu_transaction t, output dsu_result r);

#1; // allow combinational logic to settle after drive_one()'s NBA update

r = new();

r.txn            = t;

r.control        = vif.control;

r.acc_sel        = vif.acc_sel;

r.sat_op         = vif.sat_op;

r.shift_op       = vif.shift_op;

r.shift_amt      = vif.shift_amt;

r.shift_dir      = vif.shift_dir;

r.shift_acc_sel  = vif.shift_acc_sel;

r.rd_lo_op       = vif.rd_lo_op;

r.rd_hi_op       = vif.rd_hi_op;

r.result_src     = vif.result_src;

r.writes_regfile = vif.writes_regfile;

r.rd_addr        = vif.rd_addr;

r.illegal_instr  = vif.illegal_instr;

r.funct5         = vif.funct5;

r.funct3         = vif.funct3;

r.is_custom0     = vif.is_custom0;

cg_dsu.sample();

endtask

function real get_coverage();

return cg_dsu.get_coverage();

endfunction

endclass

// -------------------------------------------------------------

// 8. SCOREBOARD -- independent reference model + self-checking compare

// -------------------------------------------------------------

class dsu_scoreboard;

int pass_cnt = 0;

int fail_cnt = 0;

function void predict(dsu_transaction t,

output logic [`MAC_CTRL_W-1:0] e_control,

output logic [1:0]  e_acc_sel,

output logic        e_sat_op,

output logic        e_shift_op,

output logic [5:0]  e_shift_amt,

output logic        e_shift_dir,

output logic [1:0]  e_shift_acc_sel,

output logic        e_rd_lo_op,

output logic        e_rd_hi_op,

output logic [1:0]  e_result_src,

output logic        e_writes_regfile,

output logic [4:0]  e_rd_addr,

output logic        e_illegal_instr,

output logic [4:0]  e_funct5,

output logic [2:0]  e_funct3,

output logic        e_is_custom0);

bit [31:0] instr = t.pack();

bit [6:0]  opcode    = instr[6:0];

bit [4:0]  rd_f      = instr[11:7];

bit [2:0]  funct3    = instr[14:12];

bit [1:0]  acc_sel_r = instr[26:25];

bit [4:0]  funct5_r  = instr[31:27];

bit [11:0] imm_i     = instr[31:20];

bit is_custom0, is_rtype, is_itype;

bit op_mac_sel, op_macsub, op_macabs, op_macdot, op_macload, op_macclear, op_macsat, op_rd_lo, op_rd_hi, op_macshift;

bit [1:0] acc_sel_eff;

bit any_legal_op, bad_acc_sel, bad_funct3;

bit compute_op, mac_en;

is_custom0 = (opcode == 7'b0001011);

is_rtype   = is_custom0 & (funct3 == 3'b000);

is_itype   = is_custom0 & (funct3 == 3'b001);

op_mac_sel  = is_rtype & (funct5_r == `FUNCT5_MAC_SEL);

op_macsub   = is_rtype & (funct5_r == `FUNCT5_MACSUB);

op_macabs   = is_rtype & (funct5_r == `FUNCT5_MACABS);

op_macdot   = is_rtype & (funct5_r == `FUNCT5_MACDOT);

op_macload  = is_rtype & (funct5_r == `FUNCT5_MACLOAD);

op_macclear = is_rtype & (funct5_r == `FUNCT5_MACCLEAR);

op_macsat   = is_rtype & (funct5_r == `FUNCT5_MACSAT);

op_rd_lo    = is_rtype & (funct5_r == `FUNCT5_MACRD_LO);

op_rd_hi    = is_rtype & (funct5_r == `FUNCT5_MACRD_HI);

op_macshift = is_itype;

acc_sel_eff = is_itype ? imm_i[7:6] : acc_sel_r;

any_legal_op = op_mac_sel | op_macsub | op_macabs | op_macdot | op_macload |

op_macclear | op_macsat | op_rd_lo | op_rd_hi | op_macshift;

bad_acc_sel = (acc_sel_eff == 2'b11);

bad_funct3  = is_custom0 & ~(is_rtype | is_itype);

e_illegal_instr = is_custom0 & t.dsu_en & (~any_legal_op | bad_acc_sel | bad_funct3);

compute_op = op_mac_sel | op_macsub | op_macabs | op_macdot | op_macload | op_macclear;

mac_en     = compute_op & t.dsu_en & ~e_illegal_instr;

e_control  = {mac_en, op_macsub, op_macclear, op_macload, op_macabs, op_macdot, t.flush};

e_acc_sel        = acc_sel_eff;

e_shift_acc_sel  = imm_i[7:6];

e_sat_op         = op_macsat   & t.dsu_en & ~e_illegal_instr;

e_shift_op       = op_macshift & t.dsu_en & ~e_illegal_instr;

e_rd_lo_op       = op_rd_lo    & t.dsu_en & ~e_illegal_instr;

e_rd_hi_op       = op_rd_hi    & t.dsu_en & ~e_illegal_instr;

e_shift_amt      = imm_i[5:0];

e_shift_dir      = imm_i[8];

e_result_src     = op_rd_lo ? 2'b01 : op_rd_hi ? 2'b10 : op_macshift ? 2'b11 : 2'b00;

e_writes_regfile = (op_rd_lo | op_rd_hi | op_macshift) & t.dsu_en & ~e_illegal_instr;

e_rd_addr        = rd_f;

e_funct5         = funct5_r;

e_funct3         = funct3;

e_is_custom0     = is_custom0;

endfunction

task check(dsu_result r);

logic [`MAC_CTRL_W-1:0] e_control;

logic [1:0] e_acc_sel, e_shift_acc_sel, e_result_src;

logic e_sat_op, e_shift_op, e_shift_dir, e_rd_lo_op, e_rd_hi_op, e_writes_regfile, e_illegal_instr, e_is_custom0;

logic [5:0] e_shift_amt;

logic [4:0] e_rd_addr, e_funct5;

logic [2:0] e_funct3;

bit ok;

predict(r.txn, e_control, e_acc_sel, e_sat_op, e_shift_op, e_shift_amt,

e_shift_dir, e_shift_acc_sel, e_rd_lo_op, e_rd_hi_op, e_result_src,

e_writes_regfile, e_rd_addr, e_illegal_instr, e_funct5, e_funct3, e_is_custom0);

ok = (r.control        === e_control)        &&

(r.acc_sel        === e_acc_sel)         &&

(r.sat_op         === e_sat_op)          &&

(r.shift_op       === e_shift_op)        &&

(r.shift_amt      === e_shift_amt)       &&

(r.shift_dir      === e_shift_dir)       &&

(r.shift_acc_sel  === e_shift_acc_sel)   &&

(r.rd_lo_op       === e_rd_lo_op)        &&

(r.rd_hi_op       === e_rd_hi_op)        &&

(r.result_src     === e_result_src)      &&

(r.writes_regfile === e_writes_regfile)  &&

(r.rd_addr        === e_rd_addr)         &&

(r.illegal_instr  === e_illegal_instr)   &&

(r.funct5         === e_funct5)          &&

(r.funct3         === e_funct3)          &&

(r.is_custom0     === e_is_custom0);

if (ok) begin

pass_cnt++;

$display("[PASS] op=%-24s instr=%08h dsu_en=%0b flush=%0b -> control=%07b illegal=%0b wr_rf=%0b res_src=%0b",

r.txn.name(), r.txn.pack(), r.txn.dsu_en, r.txn.flush, r.control, r.illegal_instr, r.writes_regfile, r.result_src);

end else begin

fail_cnt++;

$display("[FAIL] op=%-24s instr=%08h dsu_en=%0b flush=%0b", r.txn.name(), r.txn.pack(), r.txn.dsu_en, r.txn.flush);

$display("       control : got=%07b exp=%07b", r.control, e_control);

$display("       acc_sel : got=%02b exp=%02b", r.acc_sel, e_acc_sel);

$display("       sat_op  : got=%0b  exp=%0b",  r.sat_op, e_sat_op);

$display("       shift_op: got=%0b  exp=%0b",  r.shift_op, e_shift_op);

$display("       shift_amt/dir/acc: got=%0d/%0b/%02b exp=%0d/%0b/%02b",

r.shift_amt, r.shift_dir, r.shift_acc_sel, e_shift_amt, e_shift_dir, e_shift_acc_sel);

$display("       rd_lo/rd_hi: got=%0b/%0b exp=%0b/%0b", r.rd_lo_op, r.rd_hi_op, e_rd_lo_op, e_rd_hi_op);

$display("       res_src : got=%02b exp=%02b", r.result_src, e_result_src);

$display("       wr_rf   : got=%0b  exp=%0b",  r.writes_regfile, e_writes_regfile);

$display("       rd_addr : got=%0d  exp=%0d",  r.rd_addr, e_rd_addr);

$display("       illegal : got=%0b  exp=%0b",  r.illegal_instr, e_illegal_instr);

$display("       funct5/3/custom0: got=%0d/%0d/%0b exp=%0d/%0d/%0b",

r.funct5, r.funct3, r.is_custom0, e_funct5, e_funct3, e_is_custom0);

end

endtask

endclass

// -------------------------------------------------------------

// 9. ENVIRONMENT (sequential drive -> sample -> check, no fork/join)

// -------------------------------------------------------------

class dsu_env;

virtual dsu_if vif;

dsu_generator  gen;

dsu_driver     drv;

dsu_monitor    mon;

dsu_scoreboard sb;

int num_txns;

function new(virtual dsu_if vif, int num_random = 200);

this.vif = vif;

gen = new(num_random);

drv = new(vif);

mon = new(vif);

sb  = new();

endfunction

task run();

dsu_result r;

gen.build();

num_txns = gen.q.size();

foreach (gen.q[i]) begin

drv.drive_one(gen.q[i]);

mon.sample_one(gen.q[i], r);

sb.check(r);

end

$display("\n================ DSU_DECODER UNIT TB SUMMARY ================");

$display(" TOTAL=%0d  PASS=%0d  FAIL=%0d", num_txns, sb.pass_cnt, sb.fail_cnt);

$display(" FUNCTIONAL COVERAGE = %0.2f %%", mon.get_coverage());

if (sb.fail_cnt == 0)

$display(" RESULT: ALL CHECKS PASSED");

else

$display(" RESULT: %0d CHECK(S) FAILED", sb.fail_cnt);

$display("===============================================================\n");

endtask

endclass

// -------------------------------------------------------------

// 10. TEST

// -------------------------------------------------------------

class dsu_test;

dsu_env env;

function new(virtual dsu_if vif, int num_random = 200);

env = new(vif, num_random);

endfunction

task run();

env.run();

endtask

endclass

// -------------------------------------------------------------

// 11. TB TOP

// -------------------------------------------------------------

module tb_top;

bit clk = 0;

always #5 clk = ~clk;   // sequencing clock; DUT itself is combinational

dsu_if vif(clk);

dsu_decoder dut (

.instr          (vif.instr),

.dsu_en         (vif.dsu_en),

.flush          (vif.flush),

.control        (vif.control),

.acc_sel        (vif.acc_sel),

.sat_op         (vif.sat_op),

.shift_op       (vif.shift_op),

.shift_amt      (vif.shift_amt),

.shift_dir      (vif.shift_dir),

.shift_acc_sel  (vif.shift_acc_sel),

.rd_lo_op       (vif.rd_lo_op),

.rd_hi_op       (vif.rd_hi_op),

.result_src     (vif.result_src),

.writes_regfile (vif.writes_regfile),

.rd_addr        (vif.rd_addr),

.illegal_instr  (vif.illegal_instr),

.funct5         (vif.funct5),

.funct3         (vif.funct3),

.is_custom0     (vif.is_custom0)

);

dsu_test test;

initial begin

vif.instr  = 32'd0;

vif.dsu_en = 1'b0;

vif.flush  = 1'b0;

test = new(vif, 200);

test.run();

$finish;

end

endmodule