# GARUDA DSU (DSP Support Unit) — Design Specification

## 0.1 Identity

| Field | Value |
| --- | --- |
| Document ID | GARUDA-DSU-SPEC-001 |
| Revision | 3.0 |
| Date | 2026-09-18 |
| Status | Released for implementation |
| Block | 2 (dsu), inside block 1 |
| Owner | Team AeroSoC |
| Supersedes | GARUDA_DSU_Design_Spec_v2_0 |

## 0.2 Revision history

| Rev | Change | Driver |
| --- | --- | --- |
| 1.0–2.0 | Three MACs, 48-bit accumulators, nine Custom-0 instructions. RTL implemented and verified | — |
| 3.0 | Numeric contract stated for the first time (§6): Q1.15 operands, Q18.30 accumulators. The EKF acceleration claim is scoped to what the format can actually hold. Overflow guard width resolved. Clock gating on dsu_busy. | ADR-0017 |

## 0.3 Normative references

- RISC-V Unprivileged ISA v2.2 — the custom-0 opcode space (0x0B).

- GARUDA-SYS-001 Rev 4.0 — dsu section.

- GARUDA-ADR-001 Rev 1.0.

- GARUDA-CORE-SPEC-001 Rev 3.0 — EX-stage integration, dsu_busy as hold source H4.

- GARUDA-DEBUG-SPEC-001 Rev 2.0 — the accumulator taps.

## 1 Purpose and scope

### 1.1 What this is

Three multiply-accumulate units with 48-bit accumulators, decoded from the Custom-0 opcode space in the core’s EX stage. It exists to make the APF collision-avoidance inner loop and the bounded matrix products of the EKF cheap.

### 1.2 Status

Implemented and verified as part of the core’s regression.

### 1.3 The gap Rev 3.0 closes

No previous revision stated a fixed-point format. That had two consequences:

- Firmware could not be written correctly against the spec. A multiply-accumulate unit with no stated binary point leaves the driver author to infer the scaling from the RTL, and any mismatch is a silent numerical error rather than a crash.

- The EKF claim was unfalsifiable. Every revision said the DSU accelerates the EKF. With no format stated, there was no way to check whether it can. It partly cannot — see §7.5, which says so explicitly rather than leaving the claim standing.

## 2 Requirements

| ID | Requirement | Source |
| --- | --- | --- |
| R-1 | Three independent 16×16 multiply-accumulate units with 48-bit accumulators. | System |
| R-2 | Decoded in EX from Custom-0; not a bus peripheral. | ADR, latency |
| R-3 | A stated, normative fixed-point format. | ADR-0017 |
| R-4 | Saturating accumulation with a sticky overflow flag per accumulator. | Numerical safety |
| R-5 | Same-accumulator RAW hazards interlocked in hardware. | Correctness |
| R-6 | Accumulators readable by the debugger without stopping the core. | Bring-up |
| R-7 | acc_sel = 2’b11 shall be illegal everywhere. | ADR |

## 3 Block diagram

┌───────────────────────────────────────────────────────────────┐
  │                      dsu  (block 2, inside EX)                │
  │                                                               │
  │  instruction word ──▶ ┌──────────────┐                        │
  │  (raw, from EX)       │ dsu_decode   │                        │
  │  rs1, rs2 ───────────▶│ custom-0     │                        │
  │                       └──────┬───────┘                        │
  │                              │ acc_sel, op                    │
  │            ┌─────────────────┼─────────────────┐              │
  │            ▼                 ▼                 ▼              │
  │     ┌────────────┐    ┌────────────┐    ┌────────────┐        │
  │     │ mac_unit 0 │    │ mac_unit 1 │    │ mac_unit 2 │        │
  │     │            │    │            │    │            │        │
  │     │ 16×16 mul  │    │ 16×16 mul  │    │ 16×16 mul  │        │
  │     │   Q1.15    │    │            │    │            │        │
  │     │     ▼      │    │            │    │            │        │
  │     │  Q2.30     │    │            │    │            │        │
  │     │     ▼      │    │            │    │            │        │
  │     │ ┌────────┐ │    │ ┌────────┐ │    │ ┌────────┐ │        │
  │     │ │ACC_FX  │ │    │ │ACC_FY  │ │    │ │ACC_MAG │ │        │
  │     │ │48b     │ │    │ │48b     │ │    │ │48b     │ │        │
  │     │ │Q18.30  │ │    │ │Q18.30  │ │    │ │Q18.30  │ │        │
  │     │ └────────┘ │    │ └────────┘ │    │ └────────┘ │        │
  │     │  ovf ──────┼────┼─ ovf ──────┼────┼─ ovf ──────┼──┐     │
  │     └────────────┘    └────────────┘    └────────────┘  │     │
  │            │                 │                 │        │     │
  │            └────────┬────────┴────────┬────────┘        │     │
  │                     ▼                 ▼                 ▼     │
  │              ┌─────────────┐   ┌──────────────┐               │
  │              │ result mux  │   │ dsu_busy     │               │
  │              │ → EX WB     │   │ RAW interlock│               │
  │              └─────────────┘   └──────┬───────┘               │
  │                                       │                       │
  │              taps ────────────────────┼───────────────────────┼──▶ Debug
  └───────────────────────────────────────┼───────────────────────┘
                                          ▼
                                    pipe_ctrl H4

## 4 Sub-block inventory

| Name | Type | RTL file | Role |
| --- | --- | --- | --- |
| dsu_decode | comb | dsu_decode.v | Custom-0 decode, acc_sel, operation. |
| mac_unit ×3 | seq | mac_unit.v | 16×16 multiplier, 49-bit accumulate, saturation, overflow. |
| kogge_stone_49 | comb | kogge_stone_49.v | 49-bit adder for the accumulate path. |
| dsu_interlock | comb | dsu_interlock.v | Same-accumulator RAW detect → dsu_busy. |
| dsu_taps | comb | dsu_top.v | Read-only accumulator and overflow outputs for Debug. |

## 5 Interfaces

| Port | Dir | Width | Domain | Reset | Description |
| --- | --- | --- | --- | --- | --- |
| hclk_i | in | 1 | hclk (gated) | — | Gated with the core (GARUDA-CORE-SPEC-001 §7.7) and additionally by dsu_idle. |
| core_rst_n_i | in | 1 | hclk | — |  |
| instr_i | in | 32 | hclk | — | Raw instruction word from EX. |
| rs1_i, rs2_i | in | 32 | hclk | — | Register operands. |
| dsu_valid_i | in | 1 | hclk | — | A Custom-0 instruction is in EX. |
| dsu_result_o | out | 32 | hclk | 0 | To the EX writeback multiplexer. |
| dsu_busy_o | out | 1 | hclk | 0 | To pipe_ctrl as hold source H4. |
| dsu_illegal_o | out | 1 | hclk | 0 | Illegal Custom-0 encoding → illegal-instruction exception. |
| dsu_acc_o | out | 48×3 | hclk | 0 | Live taps to Debug. |
| dsu_ovf_o | out | 3 | hclk | 0 | Sticky overflow taps. |

[N-5.1] The DSU is in the core’s EX timing cone, in parallel with the 33×33 multiplier. Both contribute to the 250 MHz question (GARUDA-CORE-SPEC-001 §7.9).

## 6 Numeric contract — normative

[N-6.1] This section is the authority on DSU number formats. Firmware, the RTL and the testbench all follow it. It did not exist before Rev 3.0.

### 6.1 Formats

| Stage | Format | Width | Integer bits | Fractional bits | Range |
| --- | --- | --- | --- | --- | --- |
| Operand | Q1.15 | 16 | 1 (sign) | 15 | [−1.0, +0.999969) |
| Product | Q2.30 | 32 | 2 | 30 | [−2.0, +2.0) |
| Accumulator | Q18.30 | 48 | 18 | 30 | [−131072, +131072) |

[N-6.2] Operand extraction: MAC uses the low 16 bits of rs1_i and rs2_i, interpreted as signed Q1.15. The upper 16 bits are ignored, not checked. Firmware packs two Q1.15 values per 32-bit word where useful.

[N-6.3] The product’s 30 fractional bits align exactly with the accumulator’s 30 fractional bits, so accumulation is a plain 48-bit add with no shift. This is why Q18.30 was chosen rather than a format requiring realignment on every accumulate — a shifter in the accumulate path would land in the EX critical cone.

[N-6.4] Headroom: 18 integer bits against a product magnitude below 2.0 allows 2¹⁷ = 131,072 accumulations before the integer field can overflow, worst case. Practical loops here accumulate 10 to 50 terms.

### 6.2 What the format can and cannot represent

[N-6.5] Q1.15 operands are restricted to (−1, +1). Any value outside that range must be scaled by firmware before it reaches the DSU. The unit does not check, and an out-of-range operand is silently reinterpreted — a wrapped Q1.15 value, not a saturated one.

[N-6.6] This is the load-bearing limitation and the reason §7.5 exists. A quantity with a dynamic range wider than about 2¹⁵ relative steps cannot be represented in Q1.15 at fixed scaling, and an EKF covariance matrix has exactly that problem: diagonal terms span many orders of magnitude as the filter converges.

### 6.3 Saturation

[N-6.7] On accumulate, if the 49-bit intermediate sum exceeds the 48-bit signed range, the accumulator saturates to 0x7FFF_FFFF_FFFF (positive) or 0x8000_0000_0000 (negative), and the sticky overflow flag for that accumulator sets.

[N-6.8] Saturation rather than wrap: a wrapped accumulator changes sign, so an APF repulsion force would reverse direction and push the vehicle toward the obstacle. Saturation gives a wrong magnitude in the right direction, which degrades rather than inverts.

[N-6.9] The overflow flag is sticky and set-beats-clear: a MACCLEAR in the same cycle as an overflow leaves the flag set. Firmware must check it after a computation, not assume its own clear won.

## 7 Functional description

### 7.1 Instruction set

All in the Custom-0 opcode space (0x0B), distinguished by funct3 and funct7.

| Instruction | Operation | Result | Cycles |
| --- | --- | --- | --- |
| MAC_SEL acc | Select the target accumulator | — | 1 |
| MAC rs1, rs2 | acc += rs1[15:0] × rs2[15:0] | — | 1 |
| MACSUB rs1, rs2 | acc -= rs1[15:0] × rs2[15:0] | — | 1 |
| MACDOT rs1, rs2 | Two products into two accumulators | — | 1 |
| MACCLEAR | acc = 0; overflow flag unchanged | — | 1 |
| MACLOAD rs1 | acc = sign_extend(rs1) as Q18.30 | — | 1 |
| MACREAD_LO rd | rd = acc[31:0] | rd | 1 |
| MACREAD_HI rd | rd = sign_extend(acc[47:32]) | rd | 1 |
| MACSHIFT rs1 | acc >>>= rs1[4:0], arithmetic | — | 1 |
| MACSAT rd | rd = acc saturated to Q1.15 in rd[15:0] | rd | 1 |
| MACABS | acc = ¦acc¦ | — | 1 |

[N-7.1] Every instruction is single-cycle throughput. The only stall is the same-accumulator RAW interlock (§7.3).

[N-7.2] MACDOT is the EKF and APF primitive: it issues two 16×16 products into two different accumulators in one instruction, which is what makes a 2-element dot product one cycle rather than two.

[N-7.3] MACSAT converts an accumulator back to Q1.15 with saturation, which is the normal way a result re-enters the RV32IM datapath. MACREAD_LO/_HI give the raw Q18.30 value for firmware that needs the full precision or is writing diagnostics.

### 7.2 acc_sel encoding

| acc_sel | Accumulator |
| --- | --- |
| 00 | ACC_FX |
| 01 | ACC_FY |
| 10 | ACC_MAG |
| 11 | illegal |

[N-7.4] acc_sel = 2’b11 raises an illegal-instruction exception in every instruction that carries the field (R-7). It does not alias to ACC_MAG, and it does not silently do nothing.

[N-7.5] Rationale: an illegal encoding that aliases to a real accumulator turns an assembler bug or a corrupted instruction word into a silent wrong answer in the flight computation. Raising the exception makes it a precise fault with mtval = the instruction.

### 7.3 The RAW interlock

[N-7.6] An accumulator write has a one-cycle result latency. An instruction reading the same accumulator in the next cycle — MACREAD_LO immediately after MAC, for instance — would read the stale value.

[N-7.7] dsu_interlock detects this and asserts dsu_busy_o, which pipe_ctrl treats as hold source H4 (GARUDA-CORE-SPEC-001 §7.6). One cycle of stall.

[N-7.8] The interlock is per accumulator. A MAC on ACC_FX followed by a MACREAD_LO on ACC_FY does not stall. This is what makes three accumulators worth having: firmware interleaves work on different accumulators and the interlock never fires.

[N-7.9] dsu_busy_o is an ordinary hold source, not a special case in the pipeline control logic. The flush-beats-hold rule of [N-7.21] in the core spec applies to it exactly as to load-use: a branch or trap concurrent with dsu_busy discards the DSU operation in EX. That matters because the accumulator must not be updated by an instruction the flush killed.

### 7.4 APF — what this unit is really for

[N-7.10] The artificial potential field inner loop computes, for each neighbour, a repulsion vector inversely proportional to distance, and sums the vectors:

for each neighbour i:
      dx = x_self - x_i          # Q1.15, normalised to the field extent
      dy = y_self - y_i
      MAC_SEL ACC_MAG;  MAC dx, dx;  MAC dy, dy     # |d|² accumulates
      MAC_SEL ACC_FX;   MAC dx, w_i                 # weighted x
      MAC_SEL ACC_FY;   MAC dy, w_i                 # weighted y

[N-7.11] Roughly 300 cycles for 10 neighbours, against 250,000 cycles per loop iteration. Positions normalised to the field extent are naturally within (−1, +1), so Q1.15 is the right format for this computation and there is no scaling difficulty.

[N-7.12] This is the DSU’s clear, unqualified win, and it is the workload it was designed around.

### 7.5 EKF — the honest scope

[N-7.13] The DSU accelerates the parts of the EKF whose operands are bounded and well-scaled, and does not accelerate the rest. Stating which is which:

| EKF stage | DSU-suitable | Why |
| --- | --- | --- |
| State prediction x = Fx | yes | F entries are near-unity, x is normalised. Bounded. |
| Measurement residual y = z − Hx | yes | Both terms normalised to sensor full-scale. |
| H P products, small blocks | partly | Depends on the block’s scaling; usable where P has been renormalised. |
| Covariance propagation P = FPFᵀ + Q | no | P’s diagonal spans many orders of magnitude as the filter converges. Q1.15 at fixed scaling cannot hold it. |
| Kalman gain K = PHᵀ(HPHᵀ+R)⁻¹ | no | Requires division and a matrix inverse; no DSU operation and no hardware divider. |
| P update (I−KH)P | no | Same dynamic-range problem as propagation. |

[N-7.14] So the covariance path is RV32IM firmware, using the core’s 33×33 multiplier and the software divide handler. The DSU serves the prediction and residual paths.

[N-7.15] This document does not claim a speedup figure for the EKF. The figure is measurable — profile with mcycle around each stage, with and without the DSU path — and it is the single most useful number for the project’s technical write-up. Until it is measured, no figure is asserted. Asserting one and being asked to defend it is a worse outcome than saying it is pending.

[N-7.16] A v2 option that would extend the DSU’s reach here is block floating-point: a per-matrix exponent held in firmware, with MACSHIFT used to renormalise between blocks. MACSHIFT exists and is sufficient for this; it requires no hardware change, only a firmware scheme. Recorded as a possibility, not a commitment.

### 7.6 Debug taps

[N-7.17] All three accumulators and all three overflow flags are continuously visible to the Debug Module, with no core involvement (R-6).

[N-7.18] This is the most valuable observability in the chip. With SBA-only debug there is no GPR inspection (GARUDA-DEBUG-SPEC-001 [N-7.9]), so during bring-up of the APF or EKF the accumulators are the only direct window into the arithmetic. A wrong repulsion force is visible here immediately, without instrumenting firmware.

[N-7.19] The 48-bit values are read as two 32-bit halves with a coherent-snapshot mechanism (GARUDA-DEBUG-SPEC-001 §6.8).

### 7.7 Clock gating

[N-7.20] The DSU is inside the core’s WFI clock gate, and additionally gated by dsu_idle — no Custom-0 instruction in EX and no interlock pending.

[N-7.21] dsu_idle is derived in dsu_decode from dsu_valid_i and the interlock state, both of which are already registered. It is not a second opinion about pipeline state, so it does not fall foul of GARUDA-CORE-SPEC-001 [N-7.29].

[N-7.22] The accumulators must retain their values while gated, so they are not reset by the gate and the gate is enable-based, never a reset. An accumulator cleared by entering an idle cycle would corrupt a computation split across a stall.

## 8 Timing

### 8.1 Back-to-back MAC on different accumulators — no stall

│ T0 │ T1 │ T2 │ T3 │
EX        ──┤MAC │MAC │MAC │MAC ├
acc_sel   ──┤ FX │ FY │MAG │ FX ├
dsu_busy  ──────────────────────────  never asserts  [N-7.8]

ACC_FX    ──┤ A      │ A+p0        │ A+p0+p3 ├
ACC_FY    ──┤ B          │ B+p1              ├
ACC_MAG   ──┤ C              │ C+p2          ├

### 8.2 Same-accumulator RAW — one-cycle interlock

│ T0 │ T1 │ T2 │ T3 │
EX        ──┤MAC │ rd │ rd │ i+2├   rd = MACREAD_LO, same acc
            │ FX │ FX │ FX │    │
dsu_busy  ──────┌─────┐──────────   asserted one cycle  [N-7.7]
                 │
ACC_FX    ──┤ A │ A+p            ├
dsu_result──────────────┤ A+p    ├   correct value read

### 8.3 Flush during dsu_busy — the accumulator must not update

│ T0 │ T1 │ T2 │
dsu_busy  ──┌─────────┐──────
trap_req  ──────┌───────────────

EX        ──┤MAC │ -- │ -- ├   MAC discarded by the flush
ACC_FX    ──┤ A       │ A  ├   ◀── unchanged: the killed instruction
                                    must not have accumulated  [N-7.9]

### 8.4 Saturation and the sticky flag

ACC_MAG   ──┤ 0x7FFF_FFFF_F000 │ 0x7FFF_FFFF_FFFF │ 0x7FFF_FFFF_FFFF ├
                                 ▲                  ▲
                                 │                  └── stays saturated
                                 └── clamped, not wrapped  [N-6.8]
ovf[2]    ─────────────────────────┌───────────────────────────────────
                                   └── sticky; a same-cycle MACCLEAR
                                       does not clear it  [N-6.9]

## 9 Clock, reset and power

[N-9.1] One clock domain, hclk, gated as in §7.7. No CDC.

[N-9.2] core_rst_n_i clears all three accumulators and all three overflow flags.

[N-9.3] Power: three 16×16 multipliers and three 48-bit adders are a meaningful share of the core’s dynamic power when active, and they are active for a few hundred cycles per millisecond. dsu_idle gating is therefore worth more here than almost anywhere else in the chip, in proportion to the logic it covers.

## 10 Assertions

Existing assertions retained. New and changed for Rev 3.0:

// --- product/accumulator alignment: accumulate is a plain add, no shift (N-6.3)
a_no_realign: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  (mac_en[n] && !ovf_n) |=> (acc[n] == $past(acc[n]) + $past(product_sext)));

// --- saturation, not wrap (N-6.8)
a_saturate_pos: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  (mac_en[n] && sum49[48:47] == 2'b01) |=> (acc[n] == 48'h7FFF_FFFF_FFFF));
a_saturate_neg: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  (mac_en[n] && sum49[48:47] == 2'b10) |=> (acc[n] == 48'h8000_0000_0000));
a_never_wraps: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  (acc[n][47] != $past(acc[n][47])) |-> !$past(sign_change_legal));

// --- overflow is sticky and set-beats-clear (N-6.9)
a_ovf_sticky: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i) $fell(ovf[n]) |-> $past(ovf_clear_explicit));
a_ovf_set_beats_clear: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  (ovf_set[n] && macclear[n]) |=> ovf[n]);

// --- overflow detection uses the full 49-bit intermediate (OPEN-9)
a_ovf_from_49: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  ovf_set[n] == (sum49[48] ^ sum49[47]));

// --- acc_sel = 11 is always illegal (R-7, N-7.4)
a_accsel3_illegal: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  (dsu_valid_i && instr_acc_sel == 2'b11) |-> dsu_illegal_o);
a_accsel3_no_effect: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  (dsu_valid_i && instr_acc_sel == 2'b11) |=> $stable(acc[0]) && $stable(acc[1]) && $stable(acc[2]));

// --- interlock is per accumulator (N-7.8)
a_interlock_per_acc: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  (mac_en[0] ##1 (dsu_valid_i && instr_acc_sel == 2'b01)) |-> !dsu_busy_o);
a_interlock_fires: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  (mac_en[n] ##1 (dsu_read && instr_acc_sel == n)) |-> dsu_busy_o);

// --- a flushed DSU instruction must not accumulate (N-7.9)
a_flush_no_accumulate: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  (dsu_valid_i && ex_flush) |=> $stable(acc[instr_acc_sel]));

// --- MACSAT saturates correctly to Q1.15
a_macsat: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  macsat_en |-> (dsu_result_o[15:0] == q15_saturate(acc[instr_acc_sel])));

// --- taps are always live, never gated (R-6)
a_taps_live: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i) dsu_acc_o == acc);

// --- accumulators retain value while clock-gated (N-7.22)
a_gate_retains: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  !dsu_clk_en |=> $stable(acc[0]) && $stable(acc[1]) && $stable(acc[2]));

// --- single-cycle throughput except on interlock (N-7.1)
a_single_cycle: assert property (
  @(posedge hclk_i) disable iff (!core_rst_n_i)
  (dsu_valid_i && !dsu_busy_o) |=> dsu_complete);

## 11 Verification plan

| Req | Test | Oracle | Coverage goal | Status |
| --- | --- | --- | --- | --- |
| R-1 | t_dsu_mac | each accumulator accumulates correctly | all 3 | passing |
| R-2 | t_dsu_decode | all 9 instructions decode and execute | 9/9 | passing |
| R-3 | t_dsu_qformat | Q1.15 × Q1.15 → Q2.30 → Q18.30 against a Python golden model | ≥10⁴ random operand pairs | new |
| R-3 | t_dsu_qformat_edge | ±1.0−ε, ±smallest, zero, and all sign combinations | all 9 combinations | new |
| R-4 | t_dsu_saturate | positive and negative saturation; no wrap | both directions | extend |
| R-4 | t_dsu_ovf_sticky | sticky; set beats a same-cycle clear | — | extend |
| R-5 | t_dsu_interlock | fires on same-acc, not on cross-acc | 9 acc pairs | passing |
| R-5 | t_dsu_flush | a flushed DSU op does not accumulate | flush at each pipeline position | new |
| R-6 | t_dsu_taps | taps readable while accumulating | — | new |
| R-7 | t_dsu_accsel3 | illegal exception, no accumulator change | all instructions carrying the field | extend |
| §7.4 | t_dsu_apf | full 10-neighbour APF loop vs. a float reference, error bound stated | — | new |
| §7.5 | t_dsu_ekf_scope | the prediction and residual paths match a float reference within the stated bound; covariance is not attempted | — | new |
| §7.7 | t_dsu_gate | accumulators retain values across gated cycles | — | new |

[N-11.1] t_dsu_qformat against a Python golden model is the test Rev 3.0 exists to make possible. Without a stated format there was nothing to compare against, so the arithmetic was verified only for self-consistency. A golden model turns “the RTL does what the RTL does” into “the RTL computes the specified function.”

[N-11.2] t_dsu_apf must state a numerical error bound against the float reference, not merely “close.” That bound is what firmware relies on when it decides whether Q1.15 is adequate for a given field extent, and it is a number the project’s write-up will want.

[N-11.3] OPEN-9 resolution path: mac_unit.v:140 computes overflow from a 49-bit result_ext via kogge_stone_49, which is what a_ovf_from_49 requires. Rev 2.0 listed the widening as unapplied. Read u_csa2’s width and confirm — if the compressor is 48 bits, the guard bit reads a truncated carry and the widening is still needed; if it is 49, the erratum is closed and Rev 2.0’s text was stale.

## 12 Design decisions

| Decision | ADR |
| --- | --- |
| Q1.15 operands, Q2.30 products, Q18.30 accumulators; EKF scope stated | ADR-0017 |
| In-pipeline EX extension, not a bus peripheral | System |
| acc_sel = 11 illegal everywhere | ADR-0017 |
| Saturating, not wrapping | ADR-0017 |
| Live debug taps, no core involvement | ADR-0012 |

## 13 Not implemented

| Feature | Why not |
| --- | --- |
| 32-bit operands | Would need a 32×32 multiplier per MAC, tripling the area in the EX critical cone at 4 ns. Q1.15 covers APF fully and the EKF partly (§7.5). |
| Floating point | Would change the core’s datapath, add exception semantics, and require F-extension CSRs. Out of scope for a 1.45 mm die. |
| Hardware division or reciprocal | No divider anywhere in the chip (GARUDA-CORE-SPEC-001 §7.2). The Kalman gain’s inverse is firmware. |
| Automatic renormalisation / block floating-point in hardware | §7.16: MACSHIFT makes a firmware scheme possible, which is the cheaper place to put it. |
| More than three accumulators | Three matches APF’s {FX, FY, MAG} exactly, and the per-accumulator interlock means three is enough to avoid stalls in practice. |
| Square root | APF needs ¦d¦², not ¦d¦. Comparisons against a squared threshold avoid the root entirely. |
| A 4th acc_sel accumulator | acc_sel = 11 is deliberately illegal (§7.2) — it is a bug detector, and spending it on a fourth accumulator would remove that. |
| Wrapping mode as an option | §6.8: a wrapped APF force reverses direction. There is no use case for wrap here. |

## 14 Open items

OPEN-9: confirm u_csa2’s width in mac_unit.v ([N-11.3]). A 15-minute RTL read. Either the erratum is already closed and Rev 2.0’s text was stale, or the compressor needs widening to 49 bits.

OPEN-10 (shared with the core spec): measure the DSU’s actual contribution to the APF and EKF prediction paths with mcycle. §7.15 asserts no figure until this exists.

## 15 Errata

| ID | Symptom | Root cause | Status | Test |
| --- | --- | --- | --- | --- |
| OVF-1 | Overflow flag could miss an overflow, reading a truncated carry from the accumulate path. | The guard bit was taken from a 48-bit compressor output rather than the 49-bit sum. | Verify — see OPEN-9. RTL appears to use a 49-bit result_ext with kogge_stone_49; Rev 2.0 said the widening was unapplied. One of the two is stale. | t_dsu_ovf_sticky, a_ovf_from_49 |

Defects fixed in this revision:

| ID | Symptom | Root cause | Fix |
| --- | --- | --- | --- |
| DS-1 | No fixed-point format was ever stated, so firmware had to infer scaling from RTL and the arithmetic could only be verified for self-consistency. | The numeric contract was treated as an implementation detail rather than an interface. | §6, and t_dsu_qformat against a golden model. |
| DS-2 | The EKF acceleration claim was unfalsifiable and partly false: Q1.15 cannot hold covariance dynamic range. | The claim predated any stated format. | §7.5 scopes it stage by stage; no speedup figure asserted until measured. |