# GARUDA SoC ��� Block I: Processor Core

Table of Contents

# GARUDA SoC — Block I: Processor Core

## Block-Level Design Specification

| Document ID | AERO-GARUDA-DS-001 |
| --- | --- |
| Block ID | U_CORE |
| Revision | 1.0 |
| Status | Released for RTL |
| Process Node | 28 nm |
| Target Frequency | 200 MHz core / 100 MHz peripherals |
| Parent Document | GARUDA SoC TRM v2.0 |
| Author | Team AeroSoC |
| Date | June 2026 |

## Table of Contents

- Introduction

- Core as a Black Box (Top-Level Module)

- Core Microarchitecture (Block Diagram)

- Top-Level Pin Contract

- Pipeline Architecture

- Instruction Fetch and Prefetch Buffer

- Decode, Control, and Operands

- Execute Stage

- DSU Issue Interface

- Memory Stage and Data Port

- Hazards, Forwarding, and Stalls

- Branch and Jump Handling

- CSRs and Privilege

- Trap and Interrupt Architecture

- Debug Interface

- AHB-Lite Master Interface

- Memory Map

- Verification and Sign-Off

## 1. Introduction

### 1.1 Purpose

This document specifies the GARUDA processor core, Block I of the GARUDA SoC. The core is a custom RV32IM implementation built from scratch — it is not derived from Ibex or any existing core. Its defining responsibility is to integrate the Domain-Specific Unit (DSU, Block II) directly into its execute stage and to fetch and access memory over the SoC’s two-tier AHB-Lite / APB fabric.

This is a block-level design specification. It defines the external pin contract, the pipeline microarchitecture, the DSU issue interface, the dual AHB-Lite master interface, the M-mode privilege and CLIC-based trap model, hazard and forwarding behavior, and the verification hooks. The RTL team builds against this document.

### 1.2 Scope

This document covers the core top-module interface to memory, interrupts, and debug; the five-stage pipeline and all datapath/control blocks; the Custom-0 issue path to the DSU and its stall/flush/writeback contract; the instruction-fetch front-end including the prefetch buffer; M-mode CSRs, synchronous exceptions, and the CLIC interrupt interface; hazard detection, forwarding, and branch handling; and reset, clocking, and the dual AHB-Lite master protocol.

This document does not cover: the DSU internals (Block II — see AERO-GARUDA-DS-002); the bus interconnect, bridge, memories, DMA, or peripherals (Blocks III–XXIV); physical design; or firmware.

### 1.3 Conventions

Signal names are lowercase-with-underscores; buses carry their width, e.g. haddr_i[31:0]. Active-low signals carry an _n suffix. Module port directions use the suffix _i (input) and _o (output) at the core boundary only; internal signals are unsuffixed. Vectors are MSB-on-the-left, bit [0] is the LSB. Two’s-complement signed arithmetic is the default. Hexadecimal is written 0x1234_5678.

Requirement language: shall / must denotes a normative requirement; should denotes a recommendation; may denotes an option.

### 1.4 Reference Documents

- GARUDA SoC Technical Reference Manual v2.0, Team AeroSoC, 2026.

- GARUDA SoC DSU Block-Level Design Specification, AERO-GARUDA-DS-002, Rev 1.0.

- The RISC-V Instruction Set Manual, Volume I: Unprivileged ISA, 20191213.

- The RISC-V Instruction Set Manual, Volume II: Privileged Architecture, 20211203.

- RISC-V CLIC Specification (draft), used for the interrupt-controller interface.

- ARM AMBA 5 AHB Protocol Specification (IHI 0033), AHB-Lite subset.

## 2. Core as a Black Box (Top-Level Module)

The core is a single synthesizable unit (U_CORE). It presents two independent AHB-Lite master ports (Harvard: an instruction port and a data port), a CLIC interrupt interface, a RISC-V Debug-Module interface, the machine-timer inputs, and the global clock/reset. The DSU is instantiated inside the core in the execute stage and is therefore not visible at the core boundary — the only external evidence of the DSU is the three debug accumulator taps, which the core forwards to the Debug Module.

The complete signal-by-signal pin contract is given in Section 4. What does not exist at the boundary is listed in Section 4.4.

### 2.1 Clock and Reset

clk_i is the single 200 MHz core clock. The core is strictly single-clock-domain; all clock-domain crossing to the 100 MHz peripheral side happens in the AHB-to-APB bridge, outside the core.

rst_n_i is active-low with synchronous deassertion. On assertion: the program counter loads the reset vector (0x1000_0000, Boot ROM); all pipeline registers clear to the NOP bubble state; the register file is not cleared (x0 reads as zero by construction); and all M-mode CSRs take their specified reset values.

## 3. Core Microarchitecture (Block Diagram)

The figure below is the authoritative structural view of the core: the five pipeline stages (IF, ID, EX, MEM, WB), the pipeline registers between them, the DSU embedded in EX, the dual AHB-Lite masters, and the non-pipelined CONTROL / CSR / TRAP / CLIC block that owns architectural CSR state and drives trap entry/exit.

### 3.1 Figure Legend

The block diagram uses the following line and box conventions:

| Style | Meaning |
| --- | --- |
| Solid black line | Data bus / datapath |
| Solid gray line, slotted box | Pipeline register (if_id, id_ex, ex_mem, mem_wb) |
| Blue dashed line | Forwarding, hazard, and stall control (incl. dsu_busy, hold lines) |
| Red dashed line | Redirect and flush (branch / JALR / trap) |
| Gold / orange dashed line | CSR and DSU control & status (csr_addr/wdata/op, csr_rdata, illegal_instr, dsu_overflow, csr_clear_overflow) |
| Gray dashed line | Debug / accumulator taps (dbg_acc_0/1/2) |

### 3.2 Scope of the Figure vs. This Document (Normative)

The block diagram shows datapath structure and major control flow. It is deliberately simplified for readability and does not draw every control qualifier or handshake pulse. The following fine-grained control semantics are specified normatively in the indicated sections, which take precedence over any visual simplification in the figure:

- Operand-mux (MUX_A / MUX_B) select conditions — §11 (Hazards & Forwarding) and §8 (Execute Stage).

- Branch / jump target formulas and redirect timing — §12 (Branch & Jump Handling).

- EX result-mux priority and the mem_to_reg load override — §8 (Execute Stage).

- Exception priority and per-cause mtval — §14 (Trap & Interrupt Architecture).

- Interrupt-take gating and the CLIC acknowledge handshake — §14.

- Stall / flush composition and hold-vs-flush priority — §11.

- Full core↔DSU port contract — §9 (DSU Issue Interface).

The figure is authoritative for structure; this document is authoritative for behavior. Where the two appear to differ, the text governs.

## 4. Top-Level Pin Contract

### 4.1 Pin Contract Table

Infrastructure

| Signal | Dir | Width | Function |
| --- | --- | --- | --- |
| clk_i | in | 1 | 200 MHz core clock. |
| rst_n_i | in | 1 | Active-low, synchronous-deassert reset. |

Instruction AHB-Lite master port (I-port)

| Signal | Dir | Width | Function |
| --- | --- | --- | --- |
| i_haddr_o | out | 32 | Instruction fetch address. |
| i_htrans_o | out | 2 | Transfer type (IDLE / NONSEQ / SEQ). |
| i_hsize_o | out | 3 | Always 3'b010 (word). |
| i_hburst_o | out | 3 | SINGLE or INCR (prefetch). |
| i_hprot_o | out | 4 | 0b0010 (privileged, opcode fetch). |
| i_hwrite_o | out | 1 | Tied 1'b0; the I-port never writes. |
| i_hwdata_o | out | 32 | Tied 0. |
| i_hrdata_i | in | 32 | Fetched instruction word. |
| i_hready_i | in | 1 | Slave ready / wait-state. |
| i_hresp_i | in | 1 | OKAY(0) / ERROR(1); ERROR raises instruction access fault. |

Data AHB-Lite master port (D-port)

| Signal | Dir | Width | Function |
| --- | --- | --- | --- |
| d_haddr_o | out | 32 | Load/store address. |
| d_htrans_o | out | 2 | Transfer type. |
| d_hsize_o | out | 3 | Byte/half/word from funct3. |
| d_hburst_o | out | 3 | Always SINGLE 3'b000. |
| d_hprot_o | out | 4 | 0b0011 (privileged, data). |
| d_hwrite_o | out | 1 | 1 = store, 0 = load. |
| d_hwdata_o | out | 32 | Store data, lane-aligned. |
| d_hrdata_i | in | 32 | Load data. |
| d_hready_i | in | 1 | Slave ready / wait-state. |
| d_hresp_i | in | 1 | OKAY / ERROR; ERROR raises load/store access fault. |

CLIC interrupt interface

| Signal | Dir | Width | Function |
| --- | --- | --- | --- |
| clic_irq_i | in | 1 | Interrupt request pending (level). |
| clic_irq_id_i | in | 12 | Interrupt ID of the highest-pending source. |
| clic_irq_lvl_i | in | 8 | Interrupt level of that source. |
| clic_irq_shv_i | in | 1 | 1 = selective hardware vectoring for this source. |
| clic_irq_ack_o | out | 1 | Core has taken the interrupt (1-cycle pulse). |
| clic_irq_id_ack_o | out | 12 | ID being acknowledged. |
| clic_mintthresh_o | out | 8 | Current interrupt threshold (from CSR). |

Memory-mapped timer (FreeRTOS tick)

| Signal | Dir | Width | Function |
| --- | --- | --- | --- |
| mtime_i | in | 64 | Machine time, from system timer block. |
| mtimecmp_i | in | 64 | Time-compare value (mirrored for the local timer-IRQ compare). |

Debug-Module interface (RISC-V DM 0.13)

| Signal | Dir | Width | Function |
| --- | --- | --- | --- |
| dm_halt_req_i | in | 1 | Halt request from Debug Module. |
| dm_resume_req_i | in | 1 | Resume request. |
| dm_halted_o | out | 1 | Core is halted in Debug Mode. |
| dbg_acc_0_o | out | 48 | DSU ACC FX tap (forwarded from DSU). |
| dbg_acc_1_o | out | 48 | DSU ACC FY tap. |
| dbg_acc_2_o | out | 48 | DSU ACC MAG tap. |

### 4.2 Reset and Idle Behavior

Out of reset both masters drive HTRANS = IDLE. The I-port begins fetching from the reset vector once rst_n_i deasserts and i_hready_i is high. See §16 for the full AHB-Lite protocol.

### 4.3 Clock-Domain Notes

The core has exactly one clock domain. No core output is permitted to depend on a peripheral-domain clock. All synchronization to 100 MHz is external (AHB-to-APB bridge).

### 4.4 What Does Not Exist at the Core Boundary

The following are explicitly out of scope and must not be added without revising this specification:

- No in-core MAC, multiplier-accumulator, or saturating accumulator. All MAC math lives in the DSU.

- No internal instruction or data memory arrays. All memory is external over AHB-Lite.

- No cache or tag logic. The prefetch buffer is a FIFO, not a cache.

- No bus arbitration. Inter-master arbitration is the interconnect’s responsibility (SoC Block VI).

- No clock division or PLL.

## 5. Pipeline Architecture

### 5.1 Overview

The core is a classic five-stage in-order pipeline: IF, ID, EX, MEM, WB. One instruction is issued per cycle in the absence of hazards. The DSU occupies the EX stage and can request a one-cycle hold via dsu_busy. There is no out-of-order completion and no register renaming.

| Stage | Function |
| --- | --- |
| IF | Fetch from prefetch buffer (fed by I-port AHB master); PC generation. |
| ID | Decode, register-file read, immediate generation, hazard detection, static branch predict, JAL target. |
| EX | ALU, single-cycle multiply, DSU issue, branch/JALR resolution, address calculation, CSR read-modify-write. |
| MEM | Data memory access over the D-port AHB master; load alignment. |
| WB | Register-file writeback (ALU / load / multiply / DSU result). |

### 5.2 Pipeline Registers

Four pipeline registers separate the stages: if_id, id_ex, ex_mem, mem_wb. Each supports three controls:

- stall (hold): retains current contents (used by if_id and the PC during a load-use or DSU hold).

- flush (bubble): forces the register’s control fields to the NOP-equivalent (all write-enables low) on the next edge (used on branch mispredict and trap entry).

- normal clocked update otherwise.

On rst_n_i all four registers reset to the bubble state. The canonical NOP carried in a flushed bubble is 0x0000_0013 (addi x0, x0, 0).

Pipeline-register field contents (as drawn on the block diagram):

| Register | Carried fields |
| --- | --- |
| if_id | instr, pc, fault, valid |
| id_ex | ctrl, rs1_data, rs2_data, imm, pc, rs1_idx, rs2_idx, rd, funct3, instr |
| ex_mem | ex_result, rs2_data, ctrl, rd, funct3 |
| mem_wb | wb_data, ctrl, rd |

### 5.3 Datapath Width and Register Model

All datapaths are 32-bit. The architectural register file is 32×32-bit (x0–x31), x0 hardwired to zero. The file has two synchronous read ports (rs1, rs2) and one write port (WB). A third read port is not implemented — the DSU owns accumulator state internally and is never read back through the register file.

## 6. Instruction Fetch and Prefetch Buffer

### 6.1 Purpose

The fetch front-end decouples instruction supply from AHB-Lite fetch latency. It comprises the program counter, the prefetch buffer (a FIFO that runs ahead of the IF stage), and the I-port AHB-Lite master.

### 6.2 Program Counter and Fetch-Address Generation

Two address pointers exist:

- pc — the architectural program counter, the address of the instruction entering ID. It advances when the IF/ID register accepts a new instruction.

- fetch_pc — the speculative fetch pointer driving the I-port. It runs ahead of pc by up to the buffer depth, incrementing by 4 each time the I-port accepts a fetch (i_hready_i & i_htrans_o != IDLE) and the buffer is not full.

On a control-flow redirect (branch taken, JAL/JALR, trap, mispredict correction) both pointers are reloaded with the redirect target and the buffer is flushed (§6.5).

### 6.3 Prefetch Buffer Structure

The buffer is a depth-4 circular FIFO of 32-bit instruction words, each tagged with the PC it was fetched from (the tag is needed for trap mepc and for the branch target adder in ID).

- Write side: a word is written when the I-port returns valid read data (i_hready_i high in the data phase of an outstanding fetch) and the buffer is not full. The write enable is on the memory side.

- Read side: the IF stage pops one word per cycle when the buffer is non-empty and the pipeline is not stalled.

- Occupancy: maintained by wr_ptr, rd_ptr, and a wrap/occupancy counter giving full and empty. full backpressures fetch_pc advance; empty starves IF (inserts a fetch bubble, not a NOP into the architectural stream).

### 6.4 I-Port AHB-Lite Operation

The I-port issues word reads (i_hsize_o = 3'b010). Sequential fetches use HTRANS = SEQ with HBURST = INCR so the interconnect/SRAM can pipeline; the first fetch after a redirect is NONSEQ. i_hwrite_o is tied 0.

An i_hresp_i = ERROR on a fetch tags the corresponding buffer entry as faulting; the fault is raised as an instruction access fault synchronous exception when (and only when) that entry reaches ID, so a speculatively-fetched faulting word that is later flushed never traps. The fault bit travels in the if_id register and is consumed by the exception logic in the CONTROL block (§14.1).

### 6.5 Flush

On redirect, the whole buffer is invalidated in one cycle by resetting wr_ptr, rd_ptr, and the occupancy counter; any in-flight I-port fetch is allowed to complete on the bus but its returned data is dropped (not written). This clears the entire buffer, not a single slot.

## 7. Decode, Control, and Operands

### 7.1 Instruction Decode

The ID stage extracts opcode[6:0], rd[11:7], funct3[14:12], rs1[19:15], rs2[24:20], funct7[31:25], and the five immediate forms (I/S/B/U/J). It generates the full control bundle for the downstream stages: reg_write, mem_read, mem_write, mem_to_reg, alu_src, alu_op, branch, jal, jalr, mul_en, dsu_en, csr_en, csr_op, and is_system.

### 7.2 Supported Instruction Classes

| Class | Opcode | Notes |
| --- | --- | --- |
| LUI / AUIPC | 0110111 / 0010111 | U-type. |
| JAL / JALR | 1101111 / 1100111 | Link = PC+4. |
| BRANCH | 1100011 | BEQ/BNE/BLT/BGE/BLTU/BGEU. |
| LOAD | 0000011 | LB/LH/LW/LBU/LHU. |
| STORE | 0100011 | SB/SH/SW. |
| OP-IMM | 0010011 | ADDI … SRAI. |
| OP | 0110011 | RV32I ALU + M-ext MUL family. |
| SYSTEM | 1110011 | CSRRW/S/C[I], ECALL, EBREAK, MRET, WFI. |
| Custom-0 (DSU) | 0001011 | Issued to the DSU; see §9. |

M-extension MUL/MULH/MULHSU/MULHU are implemented in hardware (§8.2). DIV/DIVU/REM/REMU are decoded but not executed: they raise an illegal-instruction exception so the software Newton-Raphson handler runs.

### 7.3 Immediate Generation

A single combinational immediate generator selects the I/S/B/U/J form from opcode and sign-extends to 32 bits. B- and J-immediates are pre-shifted (LSB zero) and sign-extended; U-immediate is {instr[31:12], 12'b0}.

### 7.4 Register File

Synchronous-write, combinational-read. Read-during-write returns the new value via a WB→ID bypass, removing a structural hazard on back-to-back dependent ops. x0 writes are discarded; x0 reads return 0.

## 8. Execute Stage

### 8.1 ALU

A 32-bit ALU performing ADD, SUB, AND, OR, XOR, SLL, SRL, SRA, SLT, SLTU. Operand A (via MUX_A) is the forwarded rs1 or pc (for AUIPC and branch/jump target computation, which reuse the ALU adder). Operand B (via MUX_B) is the forwarded rs2 or the sign-extended immediate, selected by alu_src. The forwarding selects on MUX_A/MUX_B are driven by the Hazard/Forwarding unit (§11.1).

### 8.2 Single-Cycle Multiplier

A combinational 32×32 signed/unsigned multiplier produces a 64-bit product within the EX cycle. funct3 selects the returned half: MUL returns product[31:0]; MULH/MULHSU/MULHU return product[63:32] with the appropriate operand signedness. The multiplier has no internal pipeline registers and is the dominant EX-stage critical-path candidate.

Timing note. Closure at 200 MHz on the 28 nm PDK is expected but not asserted by this specification. The critical path is the full EX cone — the multiplier in series with the EX result mux (§8.3) — not the multiplier alone. If the path fails, the documented fallback is the TRM’s 100 MHz fallback clock with no RTL change. No retiming is implied.

### 8.3 EX Result Selection

The value forwarded to MEM/WB as the EX result is selected, in strict priority order:

- csr_rdata, when csr_en (CSRRW/CSRRS/CSRRC and immediate variants read the “old” CSR value into rd per §13.3 — this is the highest-priority input so the read-modify-write is never shadowed by a coincident DSU or multiply result on the same cycle);

- DSU result, when dsu_rd_valid (DSU read/shift instructions);

- multiply result, when mul_en;

- ALU result otherwise.

Loads override this later in WB selection via mem_to_reg (§10.3). On the block diagram this priority is the EX-stage MUX fed by CSR_RW, ALU, DSU, and MULT 32×32; the CSR input is qualified by csr_en and the DSU input by dsu_rd_valid.

Erratum §6.3-E1 (resolved). An earlier draft’s mux priority (§6.3) enumerated only DSU > MUL > ALU and omitted the CSR read-modify-write path required by §11.3 for CSRRW/CSRRS/CSRRC to return the old CSR value to rd. This is resolved per Option A of the erratum: csr_rdata is added as the highest-priority EX-result-mux input, gated by csr_en, ahead of the DSU and multiply results. This resolution is normative as of Rev 1.0 and the erratum is closed; architect sign-off recorded.

### 8.4 CSR Read-Modify-Write Datapath

CSRRW/CSRRS/CSRRC and their immediate variants execute in EX. The CSR_RW block in EX issues the access to the CONTROL/CSR block, which owns the architectural CSR state (CSRs are not pipelined). The write source into CSR_RW is rs1_data (register variants) or the 5-bit zero-extended immediate (immediate variants). The exchange is:

- CSR_RW → CONTROL: csr_addr, csr_wdata, csr_op.

- CONTROL → CSR_RW: csr_rdata, returned into the EX result mux as the highest-priority input (§8.3).

The custom dsu_ovf CSR (§13.2) is the only path that pulses csr_clear_overflow into the DSU and reads back dsu_overflow.

## 9. DSU Issue Interface

### 9.1 Integration Model

The DSU (dsu_top) is instantiated in the EX stage. The core decodes the Custom-0 opcode (0001011), presents operands and the raw instruction word to the DSU, honors the DSU’s stall request, and routes the DSU’s register result and exception back into the pipeline. The core does not interpret funct5, acc_sel, or the DSU immediate — the DSU decodes those internally.

### 9.2 Boundary Contract (Normative)

This boundary matches the released, compiling dsu_top.v RTL. It drives the full instruction word, not separated fields.

| DSU port | Dir | Width | Driven by / consumed by the core |
| --- | --- | --- | --- |
| clk | in | 1 | clk_i. |
| rst_n | in | 1 | rst_n_i. |
| flush | in | 1 | EX-stage flush (branch mispredict or trap squashing the in-flight DSU op). Must be the EX flush, not the IF/ID flush. |
| dsu_en | in | 1 | (opcode == 0x0B) of the instruction in EX. |
| instr | in | 32 | The EX-stage instruction word (carries funct5/acc_sel/imm/funct3/rd). |
| rs1 | in | 32 | Forwarded rs1 operand. |
| rs2 | in | 32 | Forwarded rs2 operand. |
| csr_clear_overflow | in | 1 | Pulse from the CSR unit on a write to the dsu_ovf CSR. |
| dsu_busy | out | 1 | Stall request; asserted for one cycle on a same-accumulator dependent readback (§9.4). Drives the pipeline hold. |
| dsu_rd_data | out | 32 | Writeback value for MACREAD LO/HI and MACSHIFT. |
| dsu_rd_valid | out | 1 | Selects dsu_rd_data into the EX result mux and gates WB. Self-gated low during a dsu_busy hold. |
| dsu_rd_addr | out | 5 | Destination GPR for the DSU result. |
| dsu_overflow | out | 1 | Sticky saturation flag; exposed read-only in the CSR map. |
| illegal_instr | out | 1 | Reserved/illegal DSU encoding; OR-ed into the core’s illegal-instruction exception. |
| dbg_acc_0/1/2 | out | 48 | Accumulator taps; forwarded to dbg_acc_*_o. |

Documentation note. The DSU design spec (AERO-GARUDA-DS-002, Fig. II.1) draws separated inputs funct5[4:0], acc_sel[1:0], imm[4:0]. The released dsu_top.v instead takes the full instr[31:0] and emits dsu_rd_addr. This specification follows the RTL, which is frozen. The DSU design-spec figure should be updated to match; the core does not change.

### 9.3 Custom-0 Instruction Encoding (Reference)

The DSU decodes Custom-0 internally; the core does not. This table is provided so integrators and codegen can reason about the seam. Encoding is taken from the frozen dsu_decoder.v / dsu_defs.vh.

R-type form (funct3 = 000): funct5 = instr[31:27], acc_sel = instr[26:25], rs2 = instr[24:20], rs1 = instr[19:15], rd = instr[11:7]. I-type form (funct3 = 001, MACSHIFT): acc_sel = imm[7:6], shift_dir = imm[8], shift_amt = imm[5:0].

| Instruction | Form | funct5 | Writes GPR? | Notes |
| --- | --- | --- | --- | --- |
| MAC_SEL | R | 0 | No | rs1×rs2, accumulate into acc_sel. |
| MACSUB | R | 1 | No | rs1×rs2, subtract from acc_sel. |
| MACABS | R | 2 | No | MAC of absolute values. |
| MACDOT | R | 3 | No | Dual 16×16 dot-product accumulate. |
| MACLOAD | R | 4 | No | Load rs1 (sign-extended) into acc_sel. |
| MACCLEAR | R | 5 | No | Clear acc_sel. |
| MACSAT | R | 6 | No | Saturate acc_sel to int32 range (readback-class for hazards). |
| MACRD_LO | R | 7 | Yes | Lower 32 bits of acc_sel → rd. |
| MACRD_HI | R | 8 | Yes | Upper 16 bits of acc_sel → rd. |
| MACSHIFT | I | 9 | Yes | Arithmetic shift of acc_sel → rd. |

acc_sel legal values are 00 (FX), 01 (FY), 10 (MAG). acc_sel = 11 is illegal and raises illegal_instr, as does any other reserved encoding or a bad funct3.

### 9.4 Stall and Writeback Timing (Normative)

The core treats dsu_busy as an opaque one-cycle pipeline hold: while dsu_busy is high, the PC and IF/ID register hold, and a bubble is inserted into MEM. The core makes no assumption about which instructions assert it — it obeys the signal.

When the DSU asserts dsu_busy. The DSU raises dsu_busy for exactly one cycle under a single condition: the immediately-preceding issued DSU op was a MACRD_LO, MACRD_HI, or MACSAT; and the current DSU op also reads an accumulator (MACRD_LO, MACRD_HI, MACSAT, or MACSHIFT); and both reference the same acc_sel. Compute ops (MAC_SEL, MACSUB, MACABS, MACDOT, MACLOAD, MACCLEAR) never assert dsu_busy. The hold history is cleared by flush and by reset.

Ownership (Normative). The hold-history state (whether the immediately-preceding DSU op was a readback on the current acc_sel) is internal to dsu_top.v and produces dsu_busy as its only externally visible effect. The core shall not maintain a duplicate or shadow copy of this history — it samples dsu_busy combinationally each cycle per the integration constraint above and reacts to it. Any core-side replication of DSU readback history is a specification violation even where it would be functionally redundant with the DSU’s own logic.

Integration constraint (Normative). dsu_busy is a registered DSU output. The core’s hold logic shall sample dsu_busy in the same EX cycle as the dependent instruction and assert the PC / IF-ID hold that cycle. The core shall not register dsu_busy a second time, or the inserted bubble lands one cycle late.

Writeback. For 1-cycle DSU read/shift instructions, the DSU asserts dsu_rd_valid in the same EX cycle, and the result is written to dsu_rd_addr in WB through the normal writeback path. Because dsu_rd_valid = writes_regfile & ~dsu_busy inside the DSU, it is forced low during a held cycle; the core’s ex_mem capture of dsu_rd_data shall therefore be qualified by dsu_rd_valid, not by dsu_en. dsu_en compute instructions that do not write a register assert neither dsu_rd_valid nor reg_write.

Software ordering constraint (Normative, firmware/codegen). The DSU’s two-cycle accumulate folds a compute op’s products into the accumulator on the next write-enabling edge, and the stall FSM does not interlock a compute op against a following same-accumulator readback. Software must not issue a MACRD_LO/MACRD_HI/MACSAT/MACSHIFT that reads an accumulator in the cycle immediately following a compute op (MAC_SEL/MACSUB/MACABS/MACDOT) that writes the same accumulator; at least one independent instruction (or a compute op to the same accumulator) must separate them so the accumulate result is settled before it is read. This is a firmware/codegen contract, not a hardware interlock, and is verified by test C30.

### 9.5 Operand Routing and Forwarding

DSU operands take the same EX forwarding paths as ALU operands (§11.1): rs1/rs2 presented to the DSU are the post-forwarding values. EX→EX and MEM→EX forwarding into a DSU source is therefore identical to forwarding into the ALU.

### 9.6 Exception and Flush Coupling (Normative)

illegal_instr from the DSU is treated exactly as a core-decoded illegal instruction: it traps with mcause = 2, mepc = the offending PC, and squashes the instruction.

On any EX flush (mispredict or trap), the core shall assert the DSU’s flush so an in-flight 2-cycle MAC leaves accumulator state unchanged (the DSU clears its sum_reg/carry_reg and gates the accumulate write on ~flush). The flush presented to the DSU shall be the EX-stage flush, not the IF/ID flush — feeding the wrong flush domain is a known integration trap. This satisfies the DSU sign-off item “trap during cycle 2 leaves the accumulator unchanged.”

## 10. Memory Stage and Data Port

### 10.1 Load/Store Unit

The MEM stage drives the D-port AHB-Lite master. d_hsize_o is derived from funct3: byte (000), halfword (001), word (010). d_hburst_o is always SINGLE. Store data is lane-aligned per AMBA: a byte/halfword is replicated onto the correct lane of d_hwdata_o.

### 10.2 Alignment and Faults

The misalignment check completes before the D-port address phase commits (late EX / early MEM); on a misalignment the bus transaction is not issued.

| Condition | Cause (mcause) |
| --- | --- |
| Halfword access to an odd address, or word access to a non-4-aligned address (load) | 4 — load address misaligned |
| Same, store | 6 — store/AMO address misaligned |
| d_hresp_i = ERROR on a load | 5 — load access fault |
| d_hresp_i = ERROR on a store | 7 — store access fault |

### 10.3 Load Result Formatting

Load data from d_hrdata_i is extracted from the addressed lane and then sign- or zero-extended per funct3 (LB/LH sign-extend; LBU/LHU zero-extend; LW passes through). The formatted value is selected into WB by mem_to_reg, overriding the EX result for load instructions.

### 10.4 Wait States

d_hready_i low extends the access: the MEM stage holds and propagates a stall upstream (PC, IF/ID, ID/EX hold; a bubble enters WB) until d_hready_i is high. Bank-parallel DSRAM access keeps the common case at zero wait states; same-bank CPU/DMA contention costs one cycle, resolved by the interconnect, transparent to the core.

## 11. Hazards, Forwarding, and Stalls

### 11.1 Forwarding

Two forwarding paths feed the EX operand muxes:

- EX→EX: the EX/MEM-stage result forwarded to the immediately following instruction.

- MEM→EX: the MEM/WB-stage result forwarded to an instruction two slots behind.

Each operand mux (MUX_A, MUX_B) selects, in priority order: the most recent producer (EX/MEM result), then the MEM/WB result, then register-file data (no hazard). The selects are driven by the Hazard/Forwarding unit. Forwarding write-enable qualification prevents forwarding from an instruction that does not write its destination (reg_write low, or rd = x0).

MUX_A data inputs: rs1_data, EX/MEM forward, MEM/WB forward, pc (AUIPC/target). MUX_B data inputs: rs2_data, EX/MEM forward, MEM/WB forward, imm.

### 11.2 Load-Use Hazard

A load result is not available until the end of MEM. The hazard unit detects the load-use case by comparing the ID-stage rs1_idx/rs2_idx against the EX-stage rd when mem_read is set for the instruction in EX. On a hit it takes two simultaneous actions: it stalls PC and IF/ID, and it flushes ID/EX (inserts one bubble), so the value can be forwarded MEM→EX in the following cycle. This is the only data-hazard stall in the core.

### 11.3 DSU Hold

dsu_busy causes a one-cycle hold as specified in §9.4 (a same-accumulator dependent readback). Unlike load-use, a DSU hold retains the id_ex contents (the held DSU op must not be squashed) — it asserts hold lines but does not flush id_ex.

### 11.4 Stall / Flush Composition (Normative)

Hold sources combine onto the PC / IF-ID / ID-EX hold lines through a priority-OR: any asserted hold source holds the corresponding registers. The flush behavior, however, differs by source, so the combination is governed by the following rule when more than one event is active in the same cycle:

Hold-vs-flush priority table.

| Event (this cycle) | PC | IF/ID | ID/EX | Notes |
| --- | --- | --- | --- | --- |
| D-port wait state | hold | hold | hold | bubble enters WB; highest-priority hold. |
| DSU hold (dsu_busy) | hold | hold | hold (no flush) | held DSU op retained in id_ex. |
| Load-use | hold | hold | flush | one bubble into id_ex. |
| Branch mispredict (EX) | redirect | flush | flush | 2 bubbles; redirect from EX. |
| Trap / MRET (CONTROL) | redirect | flush | flush | 2 bubbles; redirect target from CONTROL. |

Rule. A redirect/flush from EX or CONTROL (mispredict, trap, MRET) takes precedence over any hold: when a flush and a hold target the same register in the same cycle, the flush wins and the held op is squashed (it is being discarded anyway). A DSU hold and a load-use stall cannot coexist for the same instruction (an instruction is either a DSU op or a load consumer, not both); if a DSU hold overlaps a downstream load-use bubble, the load-use flush of id_ex still wins for id_ex while the PC/IF-ID hold is the OR of both. A D-port wait state outranks a DSU hold for the PC/IF-ID/ID-EX hold lines because the memory access must complete before any younger instruction advances.

### 11.5 Hazard Source Summary

| Source | Penalty | Mechanism |
| --- | --- | --- |
| Load-use | 1 bubble | stall PC/IF-ID, flush ID/EX. |
| DSU same-acc readback | 1 bubble | dsu_busy hold (§9.4); no ID/EX flush. |
| D-port wait state | N cycles | stall until d_hready_i. |
| I-port empty buffer | N cycles | fetch bubble, no architectural NOP. |
| Branch mispredict | 2 bubbles | flush IF and ID (§12). |
| Trap / MRET | 2 bubbles | flush IF and ID, redirect PC. |

## 12. Branch and Jump Handling

### 12.1 Prediction Policy

Static prediction: backward branches (negative B-immediate) are predicted taken, forward branches predicted not-taken. The prediction is made in ID, where the B-immediate sign is available, and a predicted-taken branch redirects fetch from ID with a one-bubble penalty. For the APF inner loop (a backward branch iterating over neighbors) the prediction is correct on every iteration except the last.

### 12.2 Target Computation and Redirect Origin

| Instruction | Target | Resolved in | Redirect origin | Penalty |
| --- | --- | --- | --- | --- |
| Conditional branch | PC + sext(Bimm) (dedicated ID adder) | EX (direction) | ID (predicted) / EX (correction) | 1 (predicted) / 2 (mispredict) |
| JAL | PC + sext(Jimm) | ID | ID | 1 bubble; link = PC+4 → rd |
| JALR | (rs1_fwd + sext(Iimm)) & ~1 | EX (needs register) | EX | 2 bubbles; link = PC+4 → rd |

JALR depends on a register, so it is resolved in EX with forwarded rs1; this is the path the prior draft omitted entirely. On the block diagram the ID redirect (predict_taken / JAL) and the EX redirect (JALR / branch-mispredict) are distinct arrows into the redirect mux, reflecting their different penalties.

### 12.3 Resolution and Correction

Conditional branches resolve in EX using the ALU comparison. The EX-resolved direction is compared against the ID prediction:

- Prediction correct: no action.

- Predicted not-taken, actually taken: flush IF and ID, redirect to the branch target.

- Predicted taken, actually not-taken: flush IF and ID, redirect to PC+4 of the branch.

Either misprediction costs two bubbles. JALR always costs two bubbles. All redirects flush the prefetch buffer (§6.5).

## 13. CSRs and Privilege

### 13.1 Privilege Model

M-mode only. There is no U- or S-mode. misa reports RV32IM with the X (non-standard) bit set for the Custom-0 DSU extension. WFI is implemented per §14.5.

### 13.2 Implemented CSRs

| CSR | Address | Function |
| --- | --- | --- |
| misa | 0x301 | ISA = RV32IM + X. Read-only. |
| mvendorid | 0xF11 | 0 (non-commercial). Read-only. |
| marchid | 0xF12 | Implementation arch ID. Read-only. |
| mimpid | 0xF13 | Revision. Read-only. |
| mhartid | 0xF14 | 0 (single hart). Read-only. |
| mstatus | 0x300 | MIE, MPIE, MPP (always 11). Global interrupt enable. |
| mtvec | 0x305 | Trap-vector base + mode[1:0]. Mode 3 = CLIC. |
| mtvt | 0x307 | CLIC trap-vector-table base (hardware vectoring). |
| mie | 0x304 | Interrupt-enable bits (legacy view; CLIC enables are per-source in the CLIC block). |
| mip | 0x344 | Interrupt-pending (read-only view). |
| mepc | 0x341 | Exception PC. |
| mcause | 0x342 | Trap cause; bit 31 = interrupt. |
| mtval | 0x343 | Faulting address or instruction. |
| mscratch | 0x340 | Scratch for trap handler. |
| mintstatus | 0xFB1 | Current and previous interrupt level (CLIC). |
| mintthresh | 0x347 | Interrupt-level threshold; drives clic_mintthresh_o. |
| mnxti | 0x345 | CLIC next-interrupt handler acceleration. |
| mcycle/h | 0xB00/0xB80 | Cycle counter. |
| minstret/h | 0xB02/0xB82 | Retired-instruction counter. |
| dsu_ovf | 0xBC0 | Custom: read DSU sticky overflow; write-1 pulses csr_clear_overflow. |

### 13.3 CSR Access

CSRRW/CSRRS/CSRRC and their immediate variants execute in EX, read-modify-write atomically within the stage (datapath in §8.4). Writes to read-only CSRs and accesses to unimplemented CSRs raise illegal-instruction. The custom dsu_ovf CSR is the only path that drives csr_clear_overflow to the DSU and reads dsu_overflow.

## 14. Trap and Interrupt Architecture

The CONTROL / CSR / TRAP / CLIC block is not pipelined. It owns architectural CSR state, performs trap entry/exit, and drives the trap-target redirect into the redirect mux.

### 14.1 Synchronous Exceptions

The core detects the following synchronous exceptions, listed in decreasing priority:

| Cause | mcause | Source |
| --- | --- | --- |
| Instruction access fault | 1 | I-port ERROR on the fetched word reaching ID (fault bit in if_id). |
| Illegal instruction | 2 | Bad opcode/funct, DIV/REM, DSU illegal_instr, CSR violation. |
| Breakpoint | 3 | EBREAK. |
| Load address misaligned | 4 | Misaligned LH/LW. |
| Load access fault | 5 | D-port ERROR on a load. |
| Store address misaligned | 6 | Misaligned SH/SW. |
| Store access fault | 7 | D-port ERROR on a store. |
| Environment call (M-mode) | 11 | ECALL. |

Exceptions are precise: the offending instruction and all younger instructions are squashed; older instructions complete.

### 14.2 Per-Cause mtval (Normative)

| Cause | mtval written |
| --- | --- |
| 1 — instruction access fault | Faulting fetch PC. |
| 2 — illegal instruction | The offending 32-bit instruction word. |
| 3 — breakpoint | Address of the EBREAK instruction. |
| 4 — load address misaligned | The misaligned load address. |
| 5 — load access fault | The faulting load address. |
| 6 — store address misaligned | The misaligned store address. |
| 7 — store access fault | The faulting store address. |
| 11 — ECALL | 0. |
| Interrupts | 0. |

DIV/REM note. DIV/DIVU/REM/REMU (§7.2, decoded but not executed in hardware) share cause 2 with all other illegal-instruction sources; mtval is the offending 32-bit instruction word, not a special-cased value, so the software Newton-Raphson handler decodes funct3/rs1/rs2 directly from the trapped instruction rather than from a separate dispatch table.

### 14.3 CLIC Interrupt Interface and Take Condition (Normative)

The core implements CLIC-mode trap handling (mtvec.mode = 3). The external CLIC presents the highest-priority pending source on clic_irq_i/_id_i/_lvl_i/_shv_i.

Take condition. The core takes the presented interrupt when all of the following hold:

- clic_irq_i is high, and

- mstatus.MIE is set, and

- clic_irq_lvl_i > mintthresh (strictly greater than the threshold), and

- clic_irq_lvl_i > the current handling level in mintstatus (strictly greater than the level of any active handler).

Both level comparisons are strictly-greater-than. Threshold and active-level use the same > relation; a source at exactly the threshold or at the active level does not preempt.

On taking the interrupt the core:

- pulses clic_irq_ack_o for one cycle with clic_irq_id_ack_o = the taken ID;

- writes mepc, sets mcause[31] = 1 with the interrupt ID, updates mintstatus;

- if clic_irq_shv_i = 1, hardware-vectors through the mtvt table; otherwise jumps to mtvec.base;

- flushes IF and ID and redirects fetch.

Acknowledge handshake. clic_irq_ack_o / clic_irq_id_ack_o are the core’s only outputs back to the CLIC and form a one-cycle pulse handshake; the CLIC uses them to clear the pending/edge state of the taken source.

Preemptive nesting is supported through mintstatus/mintthresh: a higher-level source may interrupt an active handler. Nesting depth is bounded by the level count (8).

### 14.4 Trap Entry and Exit Sequence

On any trap (exception or interrupt): mepc ← PC of the trapped/next instruction; mcause ← cause; mtval ← faulting address/instruction per §14.2; mstatus.MPIE ← MIE; mstatus.MIE ← 0; MPP ← 11. MRET restores MIE from MPIE, restores the interrupt level, and redirects to mepc. Both entry and exit cost two bubbles (flush IF/ID). The trap/MRET target is driven from the CONTROL block into the redirect mux.

### 14.5 WFI Behavior (Normative)

WFI is implemented as a pipeline-draining stall:

- On WFI decode, the core drains the pipeline to a precise boundary: all in-flight instructions older than the WFI complete (including any MEM access); no younger instruction is fetched.

- The core then holds with the PC pointing at the instruction after the WFI.

- The wake condition is any pending, enabled interrupt — specifically clic_irq_i high with the source enabled at the CLIC. The wake condition is evaluated independently of mstatus.MIE: WFI wakes even if MIE = 0. If MIE = 1 and the take condition (§14.3) is met, the core takes the interrupt on wake; if MIE = 0, the core resumes execution at the instruction after WFI without taking the trap.

- On wake, normal fetch resumes from the held PC.

Vectoring on wake (Normative). When the core takes an interrupt on WFI wake, entry proceeds exactly as in §14.3 steps 1–4, including the clic_irq_shv_i check: SHV sources hardware-vector through the mtvt table, non-SHV sources jump to mtvec.base. WFI wake is not a distinct trap-entry path — it is ordinary interrupt entry whose only WFI-specific precondition is that the wake check ignores mstatus.MIE (step 3 above), while taking the interrupt still requires MIE = 1 and the full §14.3 take condition. The core’s WFI FSM and CLIC entry logic shall invoke the same redirect path rather than implementing independent entry sequences.

### 14.6 Timer Interrupt

The FreeRTOS tick uses the machine timer: when mtime_i ≥ mtimecmp_i the local machine-timer-interrupt pending bit sets and is delivered through the CLIC like any other source. The timer block itself is external; the core only consumes the compare result.

## 15. Debug Interface

### 15.1 Debug Mode

The core supports RISC-V Debug 0.13 via an external Debug Module (DM) over JTAG. The core exposes dm_halt_req_i, dm_resume_req_i, and dm_halted_o. On halt request the core drains the pipeline to a precise boundary, enters Debug Mode (fetch redirected to the DM’s program buffer / dpc held), and asserts dm_halted_o. Resume restores dpc and re-enters normal fetch. Four hardware breakpoints are supported via the trigger CSRs; a breakpoint match raises a debug entry rather than an EBREAK exception when Debug Mode is armed.

### 15.2 Accumulator Observability

The three DSU accumulator taps (dbg_acc_0/1/2_o, 48-bit each) are wired straight from the DSU to the core boundary so the DM can read accumulator state in a halted core without injecting MACREAD instructions, satisfying the DSU’s built-in observability requirement.

## 16. AHB-Lite Master Interface

### 16.1 Two Independent Masters

The core presents two AHB-Lite master ports (I and D). Each is a compliant AHB-Lite master; neither performs arbitration. The interconnect (a separate SoC block) is responsible for multiplexing the I-port, D-port, and DMA onto the shared slaves and for generating per-slave HSEL and the read-data mux.

### 16.2 Transfer Protocol

- Address phase drives HADDR, HTRANS, HWRITE, HSIZE, HBURST, HPROT.

- Data phase samples HRDATA (read) or drives HWDATA (write), qualified by HREADY.

- HREADY low inserts wait states; the master holds address/control stable.

- HRESP = ERROR is the two-cycle AMBA error response; the master accepts both cycles, then raises the corresponding access-fault exception (instruction or load/store).

- The I-port may issue INCR bursts for sequential prefetch; the D-port issues only SINGLE.

### 16.3 Reset and Idle

Out of reset both masters drive HTRANS = IDLE. The I-port begins fetching from the reset vector once rst_n_i deasserts and i_hready_i is high.

## 17. Memory Map

The core targets the TRM memory map. Out-of-range accesses rely on the interconnect’s default slave to return ERROR, which the core converts to an access fault.

| Region | Base | Size | Port |
| --- | --- | --- | --- |
| Instruction SRAM | 0x0000_0000 | 64 KB | I-port |
| Boot ROM | 0x1000_0000 | 4 KB | I-port (reset vector) |
| Data SRAM | 0x2000_0000 | 64 KB | D-port (4 banks) |
| Peripherals | 0x4000_0000 | — | D-port via APB bridge |

The reset vector is 0x1000_0000 (Boot ROM). Instruction fetch normally runs in the ISRAM region after the bootloader copies firmware from external SPI flash.

## 18. Verification and Sign-Off

### 18.1 Built-in Observability

- dbg_acc_0/1/2_o — continuous DSU accumulator view.

- minstret/mcycle — retirement and cycle counts for IPC and trace checks.

- dm_halted_o and the trigger CSRs — breakpoint and single-step bring-up.

### 18.2 Required Directed Tests

| ID | Scenario |
| --- | --- |
| C01 | RV32I base ISA compliance (riscv-arch-test, RV32I suite). |
| C02 | M-extension MUL/MULH/MULHSU/MULHU against reference products. |
| C03 | DIV/REM raise illegal-instruction and reach the software handler. |
| C04 | Load-use hazard: 1-bubble stall and correct MEM→EX forwarding. |
| C05 | EX→EX and MEM→EX forwarding on dependent ALU chains. |
| C06 | Static prediction: backward-taken loop, forward-not-taken if/else. |
| C07 | Branch misprediction in both directions; 2-bubble flush correct. |
| C08 | JAL link and target; redirect from ID, 1 bubble. |
| C09 | JALR (rs1+imm)&~1 target and link; 2 bubbles; forwarded rs1. |
| C10 | LB/LH/LW/LBU/LHU sign/zero extension; SB/SH/SW lane alignment. |
| C11 | Misaligned LH/LW and SH/SW raise causes 4/6; no bus transaction issued. |
| C12 | D-port and I-port ERROR raise access faults 5/7/1 precisely. |
| C13 | Speculatively-fetched faulting word that is flushed does not trap. |
| C14 | Prefetch buffer fill/drain, full backpressure, empty fetch bubble. |
| C15 | Whole-buffer flush on redirect; no stale instruction issued. |
| C16 | DSU issue: dsu_en on Custom-0; rs1/rs2 forwarding into MAC_SEL. |
| C17 | Same-accumulator dependent-readback hazard: a MACRD/MACSAT followed by a MACRD/MACSAT/MACSHIFT on the same acc_sel asserts dsu_busy for one cycle, inserts exactly one bubble, and suppresses dsu_rd_valid during the held cycle. Verify no stall for a different acc_sel or for compute ops. |
| C18 | DSU read/shift writeback to dsu_rd_addr via WB. |
| C19 | DSU illegal_instr traps with cause 2 and correct mepc. |
| C20 | Trap during cycle 2 of a 2-cycle MAC leaves accumulator unchanged. |
| C21 | All M-mode CSR read/write, illegal-CSR traps, MRET restore. |
| C22 | ECALL/EBREAK traps; mepc/mcause/mtval correct per §14.2. |
| C23 | CLIC interrupt take, ack handshake, vectoring (SHV and non-SHV). |
| C24 | CLIC preemptive nesting honours mintthresh/mintstatus; strict-> compare at threshold and active level. |
| C25 | Machine-timer interrupt drives the FreeRTOS tick at the programmed period. |
| C26 | WFI drains and wakes on a pending enabled interrupt; wakes with MIE = 0 without taking the trap. |
| C27 | Debug halt/resume/single-step; accumulator taps readable while halted. |
| C28 | Reset: PC = 0x1000_0000, pipeline in bubble state, CSRs at reset values. |
| C29 | FreeRTOS boot + context switch on the SoC testbench. |
| C30 | Full APF inner-loop trace co-simulated against the DSU golden model; verifies the compute→same-acc-readback software ordering constraint (§9.4) is honoured. |

### 18.3 Sign-Off Checklist

- ☐ RV32IM (less hardware divide) passes riscv-arch-test.

- ☐ DIV/REM trap path verified into a working Newton-Raphson handler.

- ☐ All forwarding and the single load-use bubble verified; no missed-forward cases.

- ☐ Branch prediction, misprediction recovery, JAL and JALR all correct.

- ☐ Prefetch buffer: corrected handshake, fetch-ahead, full/empty, whole-buffer flush.

- ☐ DSU issue boundary matches dsu_top.v; dsu_busy same-acc readback hold and writeback verified.

- ☐ DSU illegal_instr and trap-during-cycle-2 behaviour verified.

- ☐ dsu_busy sampled (not re-registered); ex_mem capture qualified by dsu_rd_valid; EX flush (not IF/ID) fed to DSU.

- ☐ Compute→same-acc-readback software ordering constraint documented and honoured by codegen.

- ☐ All synchronous exceptions precise; causes and mtval correct per §14.2.

- ☐ CLIC take/ack/vector/nesting verified; strict-> thresholds; timer tick correct.

- ☐ WFI drain/wake verified including the MIE = 0 wake case.

- ☐ Both AHB-Lite masters compliant; ERROR two-cycle response handled.

- ☐ Debug halt/resume/step and accumulator observability verified.

- ☐ Synthesis closure at 200 MHz on the 28 nm PDK, or documented fallback to 100 MHz.

- ☐ FreeRTOS boots and runs the 1 kHz control loop on the SoC testbench.

End of document — AERO-GARUDA-DS-001 Rev 1.0.