GARUDA SoC — DSU / MAC Cluster

# Unit-Level Verification Report

Module Under Test: barrel_shifter.v

| Document | Unit Verification Report — barrel_shifter |
| --- | --- |
| Project | GARUDA RV32IM SoC — Team AeroSoC |
| Subsystem | DSU (Custom-0 MAC cluster) — MACSHIFT instruction datapath |
| Verification Phase | Phase 1 — Unit-Level Verification (per 1Tops RISC-V SoC Verification Strategy) |
| Simulator | Cadence Xcelium (xrun / xmsim) |
| Methodology | Directed tests + Constrained-Random Verification (CRV) + SVA + Functional Coverage |
| Overall Status | PASS — all checks passed, RTL bug fixed and closed |

# 1. Module Overview

barrel_shifter is a purely combinational module in the GARUDA DSU (Custom-0 MAC cluster) that implements the MACSHIFT instruction's datapath. It takes the 48-bit MAC accumulator output, shifts it by a software-specified amount and direction, and returns the low 32 bits for regfile writeback.

- shift_dir = 0: arithmetic right shift (sign-extending) — used to rescale a signed accumulator value.

- shift_dir = 1: logical left shift (zero-filling) — used to rescale upward / discard low fractional bits.

- shift_amt is a 6-bit field (0–63) and is not masked against the 48-bit operand width, so shift amounts at and beyond the operand width (≥48) are a first-class corner case, not an invalid input.

# 2. Signal Description

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| cluster_out | 48 bit | Input | Full 48-bit MAC accumulator value (signed, two's complement) from the DSU MAC cluster, to be shifted for the MACSHIFT instruction's result. |
| shift_amt | 6 bit | Input | Shift amount, 0-63. Encoded directly from instruction immediate bits imm_i[5:0]. Not masked against the 48-bit operand width by the module. |
| shift_dir | 1 bit | Input | Shift direction / mode select. 0 = arithmetic right shift (sign-extending). 1 = logical left shift (zero-filling). |
| shift_result | 32 bit | Output | Low 32 bits of the 48-bit shifted value, written back to the regfile for the MACSHIFT instruction. |

# 3. Verification Environment

A self-checking SystemVerilog class-based environment was built: transaction → generator → driver → monitor → scoreboard → env → test, matching the unit-level architecture defined in the project's verification strategy document.

- Driver applies stimulus synchronously to a free-running testbench clock (DUT itself is combinational).

- Monitor samples DUT outputs after combinational settle and feeds a functional covergroup.

- Scoreboard reference model is written independently from the RTL expression (via explicit bit-width reasoning) rather than mirroring it, so it does not silently repeat an RTL bug.

- 4 concurrent SVA properties are bound directly to the interface, checking spec invariants independently of the scoreboard.

- Stimulus = 20 directed corner-case transactions + 200 constrained-random transactions (dist-weighted toward shift_amt ∈ {0, 45-47, 48, 63} and cluster_out ∈ {all-0, all-1, msb-only, all-but-msb}).

# 4. Directed Test Cases

| ID | Test Case | Stimulus | Result |
| --- | --- | --- | --- |
| DTC-01/02 | Zero-shift no-op (both directions) | shift_amt=0, dir=0 and dir=1, cluster_out=0x123456789ABC | PASS |
| DTC-03/04 | All-zero operand | cluster_out=0x0, shift_amt=20, dir=0 and dir=1 | PASS |
| DTC-05/06/07 | All-one operand (-1), right shift | cluster_out=0xFFFFFFFFFFFF, shift_amt=1/30/47, dir=0 | PASS |
| DTC-08/09 | Sign-boundary: most negative (0x800000000000) | shift_amt=1/47, dir=0 | PASS |
| DTC-10/11 | Sign-boundary: most positive (0x7FFFFFFFFFFF) | shift_amt=1/47, dir=0 | PASS |
| DTC-12/13 | Width boundary, shift_amt=47 (last in-width value) | cluster_out=0xABCD12345678, dir=0 and dir=1 | PASS |
| DTC-14/15 | Out-of-width boundary, shift_amt=48 | cluster_out=0xABCD12345678, dir=0 and dir=1 | PASS |
| DTC-16/17 | Max representable shift, shift_amt=63 | cluster_out=0xABCD12345678, dir=0 and dir=1 | PASS |
| DTC-18/19 | Left-shift truncation to low 32 bits | cluster_out=0x0000_0000_FFFF / 0x0000_FFFF_FFFF, dir=1 | PASS |
| DTC-20/21 | Right-shift, sign-fill crossing into bit31 window | cluster_out=0xFFFF80000001 / 0x00007FFFFFFF, shift_amt=16, dir=0 | PASS |

# 5. Constrained-Random Verification (CRV)

200 constrained-random transactions were generated using a distribution-weighted rand + constraint model, biasing shift_amt toward the 45–63 boundary region and cluster_out toward sign-bit corner patterns, while still covering the full input space. CRV surfaced additional failing vectors beyond the directed set (e.g. shift_amt=17, 23, 26, 29, 45, 46, 55, 56 with a negative operand), which is what exposed the width and count of the underlying bug — the directed set alone under-represented how often the fault is visible in the 32-bit output window.

# 6. Functional & Assertion Coverage

## 6.1 Functional Coverage Model

- cp_amt: shift_amt bucketed as zero / low[1:15] / mid[16:31] / near_bound[45:47] / at_bound{48} / above_bound[49:62] / max{63}

- cp_dir: shift_dir (arithmetic-right / logical-left)

- cp_sign: cluster_out[47] (positive / negative)

- cp_special: cluster_out ∈ {all-zero, all-one, msb-only, all-but-msb, others}

- cross cp_amt × cp_dir × cp_sign

## 6.2 SVA Assertions

| Assertion | Checks | Status |
| --- | --- | --- |
| a_zero_shift_is_noop | shift_amt = 0 must return cluster_out[31:0] unchanged, regardless of shift_dir. | PASS |
| a_left_shift_saturates_zero | Logical left shift with shift_amt >= 48 must saturate to 32'h0. | PASS |
| a_right_shift_saturates_sign | Arithmetic right shift with shift_amt >= 48 must saturate to 32 copies of cluster_out[47]. | PASS |
| a_right_shift_positive_bounded | Right-shift saturation of a positive operand (msb=0) must be all-zero. | PASS |

# 7. Results Summary

| Metric | Count | % of Total |
| --- | --- | --- |
| Total transactions applied (20 directed + 200 CRV) | 220 | 100% |
| PASS (scoreboard, vs. independent reference model) | 220 | 100% |
| FAIL (scoreboard, vs. independent reference model) | 0 | 0% |
| SVA property failures (a_right_shift_saturates_sign) | 0 | - |

Note: this is the regression run after the fix in §8.5 was applied to barrel_shifter.v (root-cause: IEEE 1800 conditional-operator signed/unsigned coercion, identified in an earlier run of this same environment). All 220 transactions (20 directed + 200 CRV) pass against the independent scoreboard reference model, all 4 SVA properties held for the full run with zero fires, and functional coverage closed at 100%.

# 8. Root-Cause Analysis — Bug BS-001 (RESOLVED)

## 8.1 Symptom

Arithmetic right shifts (shift_dir=0) of a negative operand (cluster_out[47]=1) return an incorrectly zero-filled result instead of a sign-extended one, whenever the shift amount is large enough (≥17) for the corrupted fill bits to reach into the low 32-bit output window.

## 8.2 Offending Code

wire signed [47:0] acc_signed = cluster_out;

wire [47:0] shifted = shift_dir ? (cluster_out << shift_amt) : (acc_signed >>> shift_amt);

## 8.3 Root Cause

Per IEEE 1800, when the two branches of a conditional (?:) operator differ in signedness, the entire expression — including how the signed branch's own value is computed — is evaluated as unsigned. Here, the left branch (cluster_out << shift_amt) is unsigned and the right branch (acc_signed >>> shift_amt) is signed. This mismatch silently forces the arithmetic right-shift branch to be evaluated as an unsigned (logical) shift whenever shift_dir=0, discarding sign-extension.

## 8.4 Confirming Evidence

Example: cluster_out=0xFFFFFFFFFFFF (–1), shift_amt=47, shift_dir=0.

- Expected (arithmetic): sign-extension keeps –1 as –1 regardless of shift amount → shift_result = 0xFFFFFFFF.

- Actual (RTL): behaves as a logical shift → only 1 bit survives at the bottom → shift_result = 0x00000001, matching the logged got=00000001.

The corrupted (zero-filled instead of sign-filled) region occupies only the top-most shift_amt bits of the 48-bit result. Those bits only overlap the 32-bit output window once shift_amt ≥ 48–31 = 17, which is exactly the threshold observed across all 38 failures — consistent with a single, fully-explained root cause rather than multiple independent defects.

## 8.5 Proposed Fix

Compute the signed and unsigned shift results as two separate continuous assignments, then mux between the two pre-computed values. This keeps each branch's own signedness intact during its own computation; only the final 2:1 selection combines them, which does not carry the same signedness-coercion rule.

wire [47:0] left_shifted  = cluster_out << shift_amt;   // logical left

wire [47:0] right_shifted = acc_signed  >>> shift_amt;  // arithmetic right

wire [47:0] shifted = shift_dir ? left_shifted : right_shifted;

## 8.6 Severity / Priority

Severity: High — silently corrupted MACSHIFT results for any negative accumulator value shifted right by ≥17, with no error indication to software. Priority: P1 — was blocking Phase 1 unit signoff for this module.

## 8.7 Fix Verification

The fix in §8.5 was applied to barrel_shifter.v and the full 220-transaction regression (20 directed + 200 CRV) plus all 4 SVA properties were re-run. Result: 220/220 PASS, 0 SVA fires, 100% functional coverage. Bug BS-001 is closed.

# 9. Signoff Status

- 100% pass on directed + constrained-random regression: MET (220/220 pass, 100%).

- Functional coverage goals met: MET (100% coverage across cp_amt × cp_dir × cp_sign cross, all bins hit).

- Bug burn-down: 1 RTL defect identified (BS-001), root-caused, fixed, and verified closed via full regression.

Recommendation: barrel_shifter.v meets Phase 1 Unit-Level Verification signoff criteria per the project verification strategy (100% directed + CRV pass, functional coverage closed, zero open P0/P1 defects). Approved to proceed to Phase 2 (Core Sanity TB) integration for the DSU MACSHIFT datapath.