GARUDA SoC — DSU (Custom-0 MAC cluster)

# Unit-Level Verification Report

Module Under Test: csa_3to2.v

| Document | Unit Verification Report — csa_3to2 |
| --- | --- |
| Project | GARUDA RV32IM SoC — Team AeroSoC |
| Subsystem | DSU (Custom-0 MAC cluster) — 3:2 carry-save compressor, used twice per cycle inside mac_unit.v's accumulate path |
| Verification Phase | Phase 1 — Unit-Level Verification (per 1Tops RISC-V SoC Verification Strategy) |
| Simulator | Cadence Xcelium (xrun / xmsim) |
| Methodology | Exhaustive proof (small width) + Directed + CRV + SVA + Functional Coverage (full width) |
| Overall Status | PASS — all functional checks pass; architecture confirmed correct; 1 coverage-model bug found and fixed |

# 1. Module Overview

csa_3to2 is a parameterized (WIDTH=48) 3:2 carry-save compressor. It is purely combinational and used twice per clock cycle inside mac_unit.v's MAC accumulate path: once to compress the two signed 16x16 multiply results plus a subtract-correction constant, and once to compress the running accumulator with that first stage's registered sum/carry. The combined result is resolved once per cycle by a 49-bit Kogge-Stone adder (kogge_stone_49.v).

- sum[i] = a[i] ^ b[i] ^ c[i] (bitwise XOR).

- carry = majority(a,b,c) << 1, truncated to WIDTH bits -- carry[0] is always 0, and the majority bit generated at the top position (WIDTH-1) is unconditionally discarded.

# 2. Signal Description

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| a, b, c | WIDTH (48) | Input | The three operands to compress. In this design, instantiated twice per mac_unit.v cycle: csa1 folds the two 16x16 signed product terms plus a subtract-correction constant; csa2 folds the running accumulator with csa1's registered sum/carry. |
| sum | WIDTH (48) | Output | Bitwise XOR of the three operands: sum[i] = a[i]^b[i]^c[i]. |
| carry | WIDTH (48) | Output | Bitwise majority of the three operands, shifted left by 1 and truncated to WIDTH bits: carry = ((a&b)¦(b&c)¦(a&c)) << 1. carry[0] is always 0; the majority bit generated at position WIDTH-1 is unconditionally discarded. |

# 3. Architecture Review — Is the Internal Shift Correct?

During this exercise, an alternative implementation was proposed that removes the <<1 from csa_3to2 and defers the shift to a single point at the end of a multi-stage reduction tree (the standard Wallace/Dadda-tree convention). That convention is legitimate in general, but it is NOT what this integration requires.

Tracing mac_unit.v's final merge step:

wire [48:0] s_ext = {csa2_sum[47], csa2_sum};      // sign-extend, no shift

wire [48:0] c_ext = {csa2_carry[47], csa2_carry};  // sign-extend, no shift

kogge_stone_49 u_ks (.a(s_ext), .b(c_ext), .cin(1'b0), .sum(result_ext));

kogge_stone_49 performs a plain a+b+cin with no shift logic of its own. This only produces the correct accumulator value if carry is already bit-aligned with sum by the time it arrives here -- i.e. pre-shifted inside csa_3to2, exactly as the original module implements it. Removing the internal shift, as proposed, would misalign every accumulation in the chip, since nothing downstream compensates.

Conclusion: the original csa_3to2.v (with the internal <<1) is architecturally correct for this design as integrated. The proposed “fix” would introduce a real, systemic accumulator bug and must not be applied.

# 4. Verification Environment

Two-phase strategy, standard practice for parameterized, bit-independent combinational logic:

- Phase A -- exhaustive proof at WIDTH=4: all 4096 possible (a,b,c) combinations applied directly, no randomization needed since the space is small enough to cover completely.

- Phase B -- directed corners + permutation-invariance + CRV at the real instantiated width (WIDTH=48), using the same sequential drive-sample-check architecture validated in prior GARUDA unit TBs (no fork/join, avoiding the driver/monitor race found and fixed during barrel_shifter verification).

- Independent reference model: majority computed via bit-popcount (a[i]+b[i]+c[i] >= 2), deliberately not the RTL's own (a&b)|(b&c)|(a&c) expression, so the scoreboard cannot silently repeat the same conceptual bug as the RTL.

- Arithmetic-identity check: scoreboard verifies sum+carry reconstructs the true a+b+c (widened arithmetic), and separately classifies the documented top-bit-truncation cases (exact expected shortfall of 2^WIDTH) so they are never confused with a genuine mismatch.

- Permutation invariance: same 3 values run through all 6 orderings of the a/b/c ports -- catches operand-wire-swap integration bugs.

# 5. Results Summary

| Metric | Count | % of Total |
| --- | --- | --- |
| Phase A -- exhaustive @ WIDTH=4 (all possible combinations) | 4096 / 4096 | 100% |
| Phase B total transactions (47 directed + 300 CRV) @ WIDTH=48 | 347 | 100% |
| Phase B PASS (scoreboard, vs. independent reference model) | 347 | 100% |
| Phase B FAIL | 0 | 0% |
| ...of which had the documented top-bit truncation (verified exact) | 157 | 45.2% |
| SVA property failures (4 properties) | 0 | - |

The exhaustive phase is a complete, 100% proof that the bitwise formula is correct for every possible input at WIDTH=4 -- not a sample, an exhaustive certainty. Combined with the WIDTH=48 phase's 347/347 pass (including 157 cases where the top-bit truncation was verified to be exactly the documented, expected discrepancy), no RTL defect was found in csa_3to2.v.

# 6. SVA Assertions

| Assertion | Checks | Status |
| --- | --- | --- |
| a_sum_formula | sum always equals a^b^c. | PASS |
| a_carry_formula | carry always equals majority(a,b,c) << 1 (structural mirror of the RTL). | PASS |
| a_carry_lsb_always_zero | carry[0] is always 0, by construction of the left-shift-by-1. | PASS |
| a_no_x_propagation | If a/b/c are fully known (no X), sum/carry must also be fully known -- catches any silently-propagating unknown state. | PASS |

# 7. Open Item — Coverage-Model Bug Found and Fixed

The first regression reported 100% functional coverage, but two xmelab warnings flagged a real problem in the testbench itself:

*W,OBINVE: Bin value '2' of bin 'cg_csa.cp_msb_popcount.msb_2of3' is not within the possible
           values that coverpoint can store and will be ignored.

*W,OBINVE: Bin value '3' of bin 'cg_csa.cp_msb_popcount.msb_3of3' ... will be ignored.

Root cause: cp_msb_popcount coverpointed the inline expression (vif.a[47]+vif.b[47]+vif.c[47]) directly. With three 1-bit self-determined operands and no wider context, SystemVerilog can size that addition at 1 bit, silently wrapping mod 2 -- making bin values 2 and 3 structurally unreachable. The tool correctly excluded them from the coverage model rather than actually covering them, so the reported 100% did not include 2 of the 4 intended popcount bins.

Fix applied: the popcount is now computed into an explicit bit [1:0] msb_popcount class member (with each 1-bit operand zero-extended before the add) before the covergroup samples it, removing the width ambiguity entirely.

msb_popcount = {1'b0, vif.a[WIDTH-1]} + {1'b0, vif.b[WIDTH-1]} + {1'b0, vif.c[WIDTH-1]};

This is a testbench-authoring defect, not a DUT or scoreboard defect -- the functional PASS/FAIL results in §5 are unaffected. Re-run required to confirm the corrected coverage model reaches genuine 100% including the msb_2of3 and msb_3of3 bins.

# 8. Signoff Status

- 100% pass on directed + CRV regression: MET (347/347 pass, 100%).

- Exhaustive small-width proof: MET (4096/4096, 100% of the possible input space).

- All SVA properties pass: MET (4/4, zero fires).

- Architecture / integration correctness: CONFIRMED (§3) -- internal <<1 is required by mac_unit.v's integration and must be retained.

- Functional coverage goals met: OPEN -- coverage-model bug identified and fixed in §7; re-run pending to confirm true 100% closure.

- Bug burn-down: 0 RTL defects found in csa_3to2.v; 1 testbench coverage-model defect found and fixed.

Recommendation: csa_3to2.v is functionally correct and architecturally validated against its actual integration in mac_unit.v -- do not apply the previously-proposed shift-removal change. Re-run the corrected testbench to confirm genuine 100% functional coverage (including the msb_2of3/msb_3of3 bins), then close out Phase 1 signoff for this module.