GARUDA SoC — DSU (Custom-0 MAC cluster)

# Unit-Level Verification Report

Module Under Test: mac_unit.v

| Document | Unit Verification Report — mac_unit |
| --- | --- |
| Project | GARUDA RV32IM SoC — Team AeroSoC |
| Subsystem | DSU (Custom-0 MAC cluster) — core compute datapath: 16x16 signed multiply, 48-bit accumulator, 2-cycle carry-save pipeline |
| Verification Phase | Phase 1 — Unit-Level Verification (per 1Tops RISC-V SoC Verification Strategy) |
| Simulator | Cadence Xcelium (xrun / xmsim) |
| Methodology | Directed + Constrained-Random + SVA + Functional Coverage, checked against a behavioral reference model validated offline (Python prototype) before being ported to SV |
| Overall Status | PASS — 1084/1084 cycles, 100% functional coverage, 1 RTL defect found and fixed |

# 1. Module Overview

mac_unit is the core compute datapath of the GARUDA DSU: a 16x16 signed multiply-accumulate unit with a 48-bit accumulator and a 2-cycle carry-save (csa_3to2 + kogge_stone_49) pipeline. It supports MAC_SEL, MACSUB, MACABS, MACDOT (and combinations), MACLOAD, MACCLEAR, and an external saturation writeback path. Its control inputs (en/add_sub/clear/load/abs_en/dot_en/flush) are the same field set produced by dsu_decoder's control bus.

- A compute instruction's product is computed combinationally in the cycle it is issued, registered into sum_reg/carry_reg at that cycle's edge, then folded into the accumulator (acc) at the FOLLOWING enabled edge -- a genuine 2-cycle latency from issue to the value landing in mac_out.

- load/clear/sat_writeback_en bypass this pipeline and take effect in a single cycle, but still require write_en=(en|sat_writeback_en)&~flush to actually commit -- a modifier bit alone, without a real enable, is a no-op.

- flush clears the in-flight sum_reg/carry_reg pipeline register but does NOT clear acc itself -- the architectural accumulator holds across a flush.

# 2. Signal Description

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| clk / rst_n | 1/1 bit | Input | Clock; asynchronous active-low reset. |
| rs1, rs2 | 32 bit | Input | Operands. Each holds two 16-bit lanes; operand_router selects which lanes feed the two 16x16 multipliers depending on abs_en/dot_en. |
| en, add_sub, clear, load, abs_en, dot_en, flush | 1 bit ea. | Input | Control bus -- identical field set to dsu_decoder's control bus (mac_en/mac_addsub/mac_clear/mac_load/mac_abs/mac_dot/mac_flush). |
| sat_writeback, sat_writeback_en | 48/1 bit | Input | External saturation-corrected value writeback path, bypasses the multiply/accumulate pipeline entirely. |
| mac_out | 48 bit | Output | Registered accumulator value. 2-cycle pipeline latency from instruction issue to the product landing in mac_out. |
| overflow | 1 bit | Output | Signed 48-bit accumulator overflow flag. Subject of Bug MU-001 below. |

# 3. Verification Environment

mac_unit is sequential and pipelined, so correctness is defined by cycle-accurate sequences checked against a BEHAVIORAL reference model -- one that independently computes the true arithmetic result (products, dot-products, sign handling, accumulation) rather than mirroring the RTL's csa_3to2/kogge_stone_49 implementation choices, with the same 2-stage pipeline latency. The reference model's timing and arithmetic assumptions were validated offline via a Python prototype run against the full realistic pipeline (500+ random 60-cycle sequences through operand_router -> mult_16x16 -> csa1 -> csa2 -> kogge_stone_49) before being ported to this SV environment -- that offline exercise is what originally surfaced the central finding in §7, not a guess encoded directly into SV without proof.

- Every named sequence is isolated by a real async-reset pulse applied to the DUT before it runs, matching the fix established during dsu_stall verification.

- 4 concurrent SVA properties bound directly to the interface, checking spec invariants independently of the scoreboard.

- Coverage-closure sequences deterministically force overflow=1 for every legitimately-reachable op kind via sat_writeback (which accepts the full 48-bit range directly, unlike MACLOAD which is limited to 32-bit range and cannot reach the 48-bit overflow boundary).

# 4. Directed, Closure & CRV Coverage

| Category | Coverage Intent | Result |
| --- | --- | --- |
| 1 | Basic MAC_SEL accumulate x3 -- observe the 2-cycle pipeline latency directly | PASS |
| 2 | MACSUB after MAC_SEL (subtract a product from a running accumulation) | PASS |
| 3 | MACDOT (dual-lane dot product) and MACDOT+SUB | PASS |
| 4 | MACABS including the 0x8000 saturating-abs special case, and MACABS+SUB | PASS |
| 5 | MACABS+DOT: confirms dot_en is correctly suppressed by operand_router when abs_en=1 | PASS |
| 6 | MACLOAD: immediate 1-cycle effect, confirms rs2 is ignored | PASS |
| 7 | MACCLEAR: immediate 1-cycle effect | PASS |
| 8 | sat_writeback basic path | PASS |
| 9 | FINDING candidate: sat_writeback_en=1 with en=0 and non-zero rs1/rs2 -- does a saturation writeback silently pollute the in-flight pipeline register? (no corruption found) | PASS |
| 10 | en=0 with every modifier bit set simultaneously -- must be a total no-op (reachable per dsu_decoder's actual contract, not just robustness padding) | PASS |
| 11 | Flush mid-pipeline: sum_reg/carry_reg clear, but mac_out correctly HOLDS (does not clear) | PASS |
| 12 | Async reset mid-operation | PASS |
| 13 | Bubble-interspersed accumulation (pipeline pauses cleanly across en=0 gaps) | PASS |
| 14 | Overflow baseline, non-truncating case (acc near +max, small positive add) | PASS |
| 15 | FINDING1 (central finding): MACLOAD then MACSUB constructed to trigger csa2's internal truncation -- see §7 | PASS (post-fix) |
| 16 | Coverage closure: overflow=1 forced via sat_writeback-to-boundary for each of the 7 legitimately-reachable op kinds (mac_sel, macsub, macdot, macdot_sub, macabs, macabs_sub, macabs_dot) | 7 sequences / PASS |
| 17 | Constrained-random multi-cycle sequences, dist-weighted toward 16-bit lane boundary values (0x0000/0x7FFF/0x8000/0xFFFF) | 25 sequences x 40 cycles / PASS |

# 5. SVA Assertions

| Assertion | Checks | Status |
| --- | --- | --- |
| a_no_x_propagation | Known inputs (post-reset) never produce unknown mac_out/overflow. | PASS |
| a_reset_clears_acc | mac_out reads 0 the cycle immediately after reset release. | PASS |
| a_no_update_without_write_en | mac_out never changes when en=0, sat_writeback_en=0, flush=0 -- a modifier bit alone can never take effect. | PASS |
| a_flush_holds_acc | mac_out never changes on the cycle following a flush (only the in-flight pipeline register clears). | PASS |

# 6. Functional Coverage Model

- cp_op_kind: 11 named bins, one per legal control-bus combination this environment's generator produces. No default/others bin -- option.auto_bin_max=0, since that combination is exhaustive over every stimulus path and a default bin would be permanently unreachable, not just rarely hit.

- cp_overflow, cp_flush, cp_rst_n, cp_rs1_lo_boundary (0x0000/0x7FFF/0x8000/0xFFFF/other): single coverpoints.

- cross cp_overflow × cp_op_kind, with 4 documented ignore_bins excluding overflow=1 combined with macload/macclear/sat_wb/bubble -- not stimulus gaps but hardware guarantees: is_accum=~(load|clear|sat_writeback_en) forces overflow's own contribution to 0 for those three ops, and write_en=0 during a bubble independently forces it to 0 regardless.

Reaching 100% required both exclusions above (closing bins that can never be hit by any stimulus) AND the 7 deterministic COV_overflow1_* sequences in §4 (closing bins that are reachable but require a precise +/-2^47 boundary alignment CRV cannot be trusted to find within a bounded random budget).

# 7. Bug MU-001 — Overflow Flag Unreliable in Both Directions (RESOLVED)

## 7.1 Symptom

mac_out (the accumulated value) was correct in every case tested. overflow was unreliable whenever csa2's internal carry-save truncation condition fired (>=2 of the top bits of acc/sum_reg/carry_reg aligned) -- and unreliable in BOTH directions: false negatives (genuine overflow, flag stays 0) and false positives (no overflow, flag incorrectly asserts), confirmed via a 500,000-trial numerical classification at roughly symmetric rates (~19% false negative, ~20% false positive, ~61% correct, of the trials landing in the truncation-triggering region).

## 7.2 Root Cause

The original formula, accum_ovf = result_ext[48] ^ result_ext[47], is only valid if the 49-bit CSA-then-adder path carries the TRUE, un-truncated sum. But csa_3to2's own internal <<1 truncation (majority[WIDTH-1] dropped inside u_csa2) destroys exactly the bit that formula needs -- even though it never corrupts adder_result itself (that discrepancy is exactly 2^48, invisible mod 2^48, which is why the accumulated VALUE always stayed correct while the FLAG did not).

## 7.3 Confirming Evidence

FINDING1 sequence: MACLOAD 0x80000000 (acc = -2^47) followed by MACSUB of 1x1 (term = -1). True sum = -2^47-1, a genuine overflow. Original RTL: overflow=0 (false negative). Hand-verified bit-by-bit and confirmed the exact truncation condition (2 of 3 top bits aligned) is active in this case.

## 7.4 Fix Applied

Overflow is no longer derived from the truncation-corrupted intermediate. It is now derived using the standard two's-complement addition overflow rule -- overflow iff the two operands share a sign and the result's sign differs from theirs -- applied to acc and a freshly-computed pending = sum_reg + carry_reg (a plain 48-bit add, provably correct mod 2^48 by the same argument that already made adder_result safe):

wire [47:0] pending      = sum_reg + carry_reg;

wire        acc_sign     = acc[47];

wire        pending_sign = pending[47];

wire        result_sign  = adder_result[47];

wire        accum_ovf    = (acc_sign == pending_sign) && (result_sign != acc_sign);

This touches only the overflow-derivation logic -- the entire value-computation path (multiply, csa1, sum_reg/carry_reg, csa2, kogge_stone_49, adder_result, acc_next, acc) is unchanged from the original RTL. Validated offline: 0 mismatches over 500,000 trials against ground truth, including the same extreme/truncation-triggering value distribution that broke the original formula.

## 7.5 Severity / Priority

Severity: High -- any software or downstream hardware (including overflow_flag's sticky status register) reading this bit to detect genuine accumulator overflow could not trust it in either direction. Priority: P1 -- resolved prior to this signoff.

## 7.6 Verification of the Fix

Full regression re-run against the corrected RTL: 1084/1084 PASS, 0 fails, FINDING1 overflow false-negative count = 0, and the 7 COV_overflow1_* directed sequences (§4, category 16) confirm overflow=1 now correctly fires for every legitimately-reachable op kind.

# 8. Testbench Defects Found & Fixed

One testbench defect was found and corrected during this exercise, documented here as part of the verification record rather than silently absorbed into a clean final number.

| Defect | Root Cause & Fix | Status |
| --- | --- | --- |
| TB-001: registered-output sampling skew | mac_out (a real register) was sampled with the same convention used for dsu_stall's dsu_busy (purely combinational) -- reading it before the clock edge that actually commits the write, producing a systematic one-cycle-stale value on every sample. Root-caused from an exact one-cycle-lag match in the regression log; fixed by waiting for the commit edge (@(posedge clk)) before sampling mac_out, while continuing to sample the combinational overflow output immediately. | RESOLVED |

# 9. Results Summary

| Metric | Count | % of Total |
| --- | --- | --- |
| Total sequences (24 directed/closure categories + 25 CRV) | 49 | 100% |
| Total clock cycles checked | 1084 | - |
| PASS (scoreboard, vs. independent behavioral reference model) | 1084 | 100% |
| FAIL | 0 | 0% |
| Overflow false-negative count (FINDING1 sequence) | 0 | resolved |
| SVA property failures (4 properties) | 0 | - |
| Functional coverage (final) | - | 100.00% |

# 10. Signoff Status

- 100% pass on directed + closure + CRV regression: MET (1084/1084, 100%).

- All SVA properties pass: MET (4/4, zero fires).

- Functional coverage goals met: MET (100.00%, with 4 documented and justified ignore_bins).

- Bug burn-down: 1 RTL defect (MU-001, overflow flag) found, root-caused, fixed, and re-verified closed. 1 testbench defect (TB-001) found and fixed.

Recommendation: mac_unit.v (with the MU-001 fix applied) meets all Phase 1 unit-level signoff criteria. Approved to proceed to Phase 2 (Core Sanity TB) integration. Note for the integration owner: overflow_flag.v's sticky status register consumes this module's overflow output directly -- confirm the MU-001 fix is applied to the RTL used in any SoC-level or Core Sanity regression, not just this unit-level testbench's embedded copy.