GARUDA SoC — DSU (Custom-0 MAC cluster)

# Unit-Level Verification Report

Module Under Test: overflow_flag.v

| Document | Unit Verification Report — overflow_flag |
| --- | --- |
| Project | GARUDA RV32IM SoC — Team AeroSoC |
| Subsystem | DSU (Custom-0 MAC cluster) — sticky MAC-overflow status register, cleared by CSR write |
| Verification Phase | Phase 1 — Unit-Level Verification (per 1Tops RISC-V SoC Verification Strategy) |
| Simulator | Cadence Xcelium (xrun / xmsim) |
| Methodology | Exhaustive combinational sweep + directed sticky/race sequences + CRV + SVA + Functional Coverage |
| Overall Status | PASS (functional) — 526/526 cycles, 0 RTL defects; coverage fix applied, re-run pending |

# 1. Module Overview

overflow_flag is a small sticky status register: any MAC-lane overflow event (mac_overflow[2:0]) or saturation overflow (sat_overflow) sets dsu_overflow, which stays set across subsequent idle cycles until a software CSR write (csr_clear_overflow) or reset clears it. This is the module software actually reads to learn whether a MAC overflow occurred -- it is downstream of, and directly consumes, mac_unit's own overflow output (see §7).

- Priority order in the RTL: reset > csr_clear_overflow > any_ovf (new event) > hold. The implicit hold (no explicit else branch) is what gives the register its sticky behavior.

- This priority order means a genuine overflow event landing on the exact same cycle as a CSR clear is not latched -- clear unconditionally wins. See §7.

# 2. Signal Description

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| clk / rst_n | 1/1 bit | Input | Clock; asynchronous active-low reset. |
| mac_overflow | 3 bit | Input | One overflow bit per MAC lane in the cluster (per-lane overflow output of mac_unit -- see §7 for the cross-module implication). |
| sat_overflow | 1 bit | Input | Saturation-unit overflow event. |
| csr_clear_overflow | 1 bit | Input | Software-driven clear pulse (CSR write). |
| dsu_overflow | 1 bit | Output | Sticky status bit: set by any overflow event, cleared only by csr_clear_overflow or reset. Read by software to detect whether any MAC/saturation overflow occurred since the last clear. |

# 3. Verification Environment

Small, fully-enumerable state space (mac_overflow[2:0] x sat_overflow x csr_clear_overflow x prior dsu_overflow = 64 combinations), so this environment leans on a complete exhaustive sweep rather than sampling, plus directed sequences for the sticky/multi-cycle-hold behavior and the same-cycle clear-vs-set race the module's priority order implies.

- Every named sequence is isolated by a real async-reset pulse applied to the DUT before it runs, matching the convention established during dsu_stall and mac_unit verification.

- dsu_overflow is a REGISTERED output -- sampled after the commit edge from the start of this environment's design, applying the lesson learned (and fixed) during mac_unit verification rather than repeating it.

- 4 concurrent SVA properties bound directly to the interface, checking spec invariants independently of the scoreboard.

- Reference model computed independently: a fresh boolean priority chain (reset/clear/set/hold), not a copy of the RTL's own if/else-if structure.

# 4. Directed, Exhaustive & CRV Coverage

| Category | Coverage Intent | Result |
| --- | --- | --- |
| 1 | EXHAUSTIVE: every (mac_overflow[2:0], sat_overflow, csr_clear_overflow) combination -- 32 -- against both a dsu_overflow=0 and dsu_overflow=1 starting state (64 sequences total). Complete proof of the combinational/priority logic, not a sample. | 64 sequences / PASS |
| 2 | Sticky hold across 8 idle cycles once set -- confirms dsu_overflow never spontaneously self-clears | PASS |
| 3 | FINDING1 (central finding): same-cycle csr_clear_overflow AND a genuine overflow event -- see §7 | PASS (vs. spec) |
| 4 | Each of the 3 mac_overflow bits isolated individually (bit-wise OR correctness) | 3 sequences / PASS |
| 5 | sat_overflow alone | PASS |
| 6 | Async reset while dsu_overflow is set -- must clear immediately | PASS |
| 7 | Clear then re-set sequentially (not a race) -- confirms clear/set aren't order-confused across cycle boundaries | PASS |
| 8 | Constrained-random multi-cycle sequences, weighted toward the clear/set race scenario specifically | 15 sequences x 25 cycles / PASS |

# 5. SVA Assertions

| Assertion | Checks | Status |
| --- | --- | --- |
| a_no_x_propagation | Known inputs (post-reset) never produce an unknown dsu_overflow. | PASS |
| a_reset_clears | dsu_overflow reads 0 the cycle immediately after reset release. | PASS |
| a_sticky_holds | dsu_overflow never self-clears without csr_clear_overflow or reset. | PASS |
| a_clear_works | A clear with no concurrent overflow event actually clears dsu_overflow. | PASS |

# 6. Functional Coverage Model

- cp_mac_overflow (8 auto-bins), cp_sat_overflow, cp_clear, cp_dsu_overflow, cp_rst_n: single coverpoints.

- cross cp_clear, cp_mac_overflow: all 16 combinations reachable and hit -- both are independent inputs with no interdependency.

- cross cp_dsu_overflow, cp_clear: 1 of 4 combinations -- (dsu_overflow=1, csr_clear_overflow=1) -- is structurally impossible, not just rare. The RTL's priority chain means csr_clear_overflow ALWAYS forces dsu_overflow to 0 on the same cycle it is asserted, with no exception for a concurrent any_ovf. Since dsu_overflow is sampled after the commit edge (reflecting that cycle's own effect), this combination can never be observed regardless of stimulus.

Initial run measured 96.43% (27 of 28 reachable-by-construction bins hit) -- the shortfall traced exactly to the one structurally-impossible bin above, not a stimulus gap. Fixed via a documented ignore_bins excluding binsof(cp_dsu_overflow) intersect {1} && binsof(cp_clear) intersect {1}. Re-run to confirm the corrected model reaches genuine 100% is pending at the time of this report.

# 7. Finding 1 — Same-Cycle Clear-vs-Set Race (Architecture Question, Not a Defect)

This module's priority order is reset > clear > set > hold. A genuine overflow event (mac_overflow or sat_overflow asserted) landing on the exact same cycle as a csr_clear_overflow pulse is silently dropped -- never latched -- rather than being caught by the next software read. The FINDING1_clear_vs_set_same_cycle_race sequence demonstrates this directly: mac_overflow[2] and csr_clear_overflow asserted together, and the resulting dsu_overflow correctly matches overflow_flag.v's own written specification (clear wins) -- this is not a defect against the RTL as written.

This is flagged for the CSR/status-register contract owner to confirm as intended: some sticky-status designs deliberately choose set-wins-over-clear specifically so a same-cycle race can never hide a genuine event. Whether clear-wins (as implemented) or set-wins is the correct semantics for GARUDA's overflow-detection contract is an architecture decision, not something resolvable from this unit-level testbench alone.

# 8. Cross-Module Context — Relationship to mac_unit's Overflow Finding

overflow_flag.v's own logic is correct against its own specification (§10). However, as a sticky register downstream of mac_overflow[2:0], it directly inherits the reliability of whatever feeds those bits. mac_unit.v's own overflow output was independently found and fixed (Bug MU-001, see the mac_unit verification report) to be unreliable in both directions whenever csa2's internal carry-save truncation condition fired. Prior to that fix, overflow_flag would have faithfully latched that unreliability into a persistent, software-visible false status -- turning a per-cycle flag glitch into a status bit software could not trust until explicitly cleared. This module's own verification does not re-litigate that upstream finding; it is noted here so the connection is part of the permanent record.

# 9. Results Summary

| Metric | Count | % of Total |
| --- | --- | --- |
| Total sequences (7 directed categories + 15 CRV) | 87 | 100% |
| Total clock cycles checked | 526 | - |
| PASS (scoreboard, vs. independent reference model) | 526 | 100% |
| FAIL | 0 | 0% |
| SVA property failures (4 properties) | 0 | - |
| Functional coverage (pre-fix run, as executed) | - | 96.43% |
| Functional coverage (post-fix, pending re-run confirmation) | - | expected 100% |

# 10. Signoff Status

- 100% pass on directed + exhaustive + CRV regression: MET (526/526, 100%).

- All SVA properties pass: MET (4/4, zero fires).

- Functional coverage goals met: OPEN -- root cause identified and fixed (§6), re-run to confirm 100% pending.

- Bug burn-down: 0 RTL defects found in overflow_flag.v against its own specification. 1 architecture question raised (Finding 1, §7) for the CSR contract owner.

Recommendation: overflow_flag.v meets all functional and assertion signoff criteria against its own written specification. Re-run with the coverage fix to confirm 100% before final Phase 1 signoff. Route Finding 1 (§7) to the CSR/status-register contract owner for an explicit intended-semantics decision, and confirm with the integration owner that mac_unit's MU-001 fix (§8) is applied upstream of this module in any SoC-level or Core Sanity regression.